3GPP TSG-SA5 Meeting #159	S5-250846

Sophia-Antipolis, France, 17 - 21 February 2025 					(is revision of S5-250209, S5-250598)

# Source:	Nokia

# Title:	DP on TS 28.554 KPIs related to NetworkSlice and NetworkSliceSubnet KPI Object

# Document for:	Discussion

Agenda Item:	6.2.2

# 1	Decision/action requested

The group is requested to discuss and endorse the proposal in clause 4.

# 2	References

[1]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[3]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TS 23.501: " System Architecture for the 5G System; Stage 2".

# 3	Rationale

The following observations are made on the KPIs related to NetworkSlice and NetworkSliceSubnet KPI Object, defined in TS 28.554 [3]:

Observation #1:

Network slice is defined by SA2 as in clause 3.1 of TS 23.501 [5] as below:

"

Network Slice: A logical network that provides specific network capabilities and network characteristics.

"

The following terms are defined SA5 in clause 3.1 of TS 28.530 [4] as below. These describe the difference between “network slice” as defined in SA2, and “NetworkSlice instance” as defined in SA5.

"

network slice: a logical network that provides specific network capabilities and network characteristics, supporting various service properties for network slice customers.

NOTE 1: NetworkSlice Information Object Class (IOC)  (refer to TS 28.541 [6]) is used to model network slice.

NOTE 2: Represent network slice defined in TS 23.501 [3] with added service properties.

NetworkSlice instance: A Managed Object Instance (MOI) of NetworkSlice IOC.

NOTE 3: NetworkSlice instance represents service view of a network slice which exposes the root NetworkSliceSubnet instance .

network slice subnet: a representation of a set of network functions and the associated resources (e.g. compute, storage and networking resources) supporting network slice.

NOTE 4: NetworkSliceSubnet IOC (refer to TS 28.541 [6]) is used to model network slice subnet which may include core network functions and/or RAN network functions and/or other network slice subnets. The network slice instance defined in TS 23.501 [3] can be reflected via the NetworkSliceSubnet IOC and the allocated resources.

NetworkSliceSubnet instance: A Managed Object Instance (MOI) of NetworkSliceSubnet IOC.

"

Observation #1:

The NetworkSlice IOC is defined in clause 6.3.1 of TS 28.541 [1], with attribute serviceProfileList that holds a list of ServiceProfile (defined in clause 6.3.3 of TS 28.541 [1]) supported by the NetworkSlice MOI.

ServiceProfile is a data type with attribute pLMNInfoList (a list of data type PLMNInfo defined in clause 4.3.41 of TS 28.541 [1]) that defines which PLMN and S-NSSAI combinations that are assigned for the service to satisfy service requirements represented by the ServiceProfile in case of network slicing feature is supported.

A NetworkSlice MOI can be configured, in attribute serviceProfileList, with a list of ServiceProfile value, where the list can hold one or more than one element. Each element in the list attribute serviceProfileList can be configured with a list PLMN and S-NSSAI combinations in attribute pLMNInfoList.

Hence a NetworkSlice MOI can be configured with a list of PLMN and S-NSSAI combinations

Observation #2:

The KPIs in 28.554 [3] related to “network slice” (as defined in SA2) are reported on NetworkSlice KPI objects, while “network slice” in SA2 are identified using S-NSSAI.

The KPIs in 28.554 [3] that are reported on NetworkSlice KPI objects based on measurement (defined in TS 28.552 [2]) with SNSSAI filter. These KPIs are are not split based on per PLMN and per S-NSSAI.

Few examples of KPIs where this issue is noticed are as below. These are few examples and not the complete list of KPIs with the issue.

Example #1: Clause 6.2.1 of TS 28.554 [3] as below, where the issue is that which SNSSAI in the filtered “RM.RegisteredSubNbrMean.SNSSAI” measurement needs to be used is unclear since the NetworkSlice (and SubNetwork) instance can have more than one SNSSAI.

"

### 6.2.1	Mean registered subscribers of network and network slice through AMF

a)	AMFMeanRegNbr.

b)	This KPI describe the mean number of subscribers that are registered to a network slice instance. It is obtained by counting the subscribers in AMF that are registered to a network slice instance. It is an Integer. The KPI type is MEAN.

c)

d)	SubNetwork, NetworkSlice

"

Example #2: Clause 6.2.5 of TS 28.554 [3] as below, where the issue is that which SNSSAI in the filtered “SM.PduSessionCreationSucc.SNSSAI” measurement needs to be used is unclear since the NetworkSlice instance can have more than one SNSSAI.

"

### 6.2.5	PDU session Establishment success rate of one network slice (S-NSSAI)

a)	PDUSessionEstSR.

b)	This KPI describes the ratio of the number of successful PDU session establishment request to the number of PDU session establishment request attempts for all SMF which related to one network slice (S-NSSAI) and is used to evaluate accessibility provided by the end-to-end network slice and network performance. It is obtained by the number of successful PDU session requests divided by the number of attempted PDU session requests. It is a percentage. The KPI type is RATIO.

c)

d)	NetworkSlice

"

Example #3: Clause 6.2.6 of TS 28.554 [3] as below, where the issue is that which SNSSAI in the filtered “RM.RegisteredSubNbrMax.SNSSAI” measurement needs to be used is unclear since the NetworkSlice instance can have more than one SNSSAI.

"

### 6.2.6	Maximum registered subscribers of network slice through AMF

a)	AMFMaxRegNbr.

b)	This KPI describe the maximum number of subscribers that are registered to a network slice. It is obtained by counting the subscribers in AMF that are registered to a network slice. It is an Integer. The KPI type is CUM.

c)

d)	NetworkSlice

"

Observation #3:

The NetworkSliceSubnet IOC is defined in clause 6.3.2 of TS 28.541 [1], with attribute sliceProfileList that holds a list of SliceProfile (defined in clause 6.3.4 of TS 28.541 [1]) supported by the NetworkSliceSubnet MOI.

SliceProfile is a data type with attribute pLMNInfoList (a list of data type PLMNInfo defined in clause 4.3.41 of TS 28.541 [1]) that defines which PLMN and S-NSSAI combinations that are assigned for the service to satisfy service requirements represented by the ServiceProfile in case of network slicing feature is supported.

A NetworkSliceSubnet MOI can be configured, in attribute sliceProfileList, with a list of SliceProfile value, where the list can hold one or more than one element. Each element in the list attribute sliceProfileList can be configured with a list PLMN and S-NSSAI combinations in attribute pLMNInfoList.

Further, attribute networkSliceSubnetRef of NetworkSliceSubnet IOC is a list of DN of constituent NetworkSliceSubnet. Each consitituent NetworkSliceSubnet can further be configured with a list of PLMN and S-NSSAI combinations.

Hence a NetworkSliceSubnet MOI can be configured with a list of PLMN and S-NSSAI combinations.

Observation #4:

Some of the KPIs in 28.554 [3] that are reported on NetworkSliceSubnet KPI objects based on measurement (defined in TS 28.552 [2]) with SNSSAI filter. These KPIs are are not split based on per PLMN and per S-NSSAI.

For example, the issue can be noticed in the KPI defined in clause 6.2.1 of TS 28.554 [3] (as below). The issue is that which SNSSAI in the filtered “GTP.InDataOctNgUgNB.SNSSAI” measurement needs to be used is unclear since the NetworkSliceSubnet (and SubNetwork) instance can have more than one SNSSAI.

"

### 6.3.9	Downstream throughput for a Network Slice at gNB

a)	DlThroughputNSgNBNgU

b)	This KPI describes the downstream throughput of a network slice instance at gNB/CU-UP end on the NgU interface. It is obtained by measuring the total number of downstream octets provided over NgU interface from UPF to NG-RAN, related to the single network slice, divided by the granularity period (in milliseconds). The KPI unit is kbit/s and the KPI type is MEAN.

c)

d)	NetworkSliceSubnet, SubNetwork.

"

Based on the above observations, the following issues are identified:

Issue #1:

The NetworkSlice IOC can be configured with multiple PLMN and S-NSSAI combinations. The KPIs in TS 28.554 [3] with KPI object as NetworkSlice, are defined based on measurements with SNSSAI filter. It is unclear which value of SNSSAI configured in NetworkSlice MOI is used for filtering.

Issue #2:

Since the NetworkSliceSubnet IOC can be configured with multiple PLMN and S-NSSAI combinations. The KPIs in TS 28.554 [3] with KPI object as NetworkSliceSubnet, are defined based on measurements with SNSSAI filter. It is unclear which value of SNSSAI configured in NetworkSliceSubnet MOI in is used for filtering.

Issue #3:

The issues #1 and #2 are present in TS 28.554 from Rel-16 and onwards.

# 4	Detailed proposal

The group is requested to endorse the following proposals:

1) The KPIs defined with KPI object as NetworkSlice and NetworkSliceSubnet in TS 28.554 [3], needs to be investigated for the issues (Issue #1 and Issue #2) and corrected.

2) The correction CRs to fix the issues can be submitted for TS 28.554 [3] from Rel-19 onwards using TEI19 or PM_KPI_5G_Ph4 WI as applicable.

3) Any new KPI proposed with KPI object as NetworkSlice or NetworkSliceSubnet in TS 28.554 [3], should take the issues (Issue #1 and Issue #2) into consideration and should not introduce the issues described in Issue #1 and Issue #2.