3GPP TSG-SA5 Meeting #159	S5-250850

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Samsung

# Title:	Triggered CCL UC and Requirements.docx

# Document for:	Approval

Agenda Item:	6.19.4.1

# 1	Decision/action requested

Group is requested to discuss and agree on the “Detailed Proposal”.

# 2	References

[1]	None

# 3	Rationale

# 4	Detailed proposal

First Change

## 5.x			Triggered CCL Capability

### 5.x.1			Description

#### 5.x.2	Use Cases

5.x.2.1				Conditional trigger of a CCL

Considering the autonomous nature of CCL, it is beneficial to study possible improvements to CCL management including automated instantiation, update and deletion of a CCL based on information provided by the consumer that could be used by the system to trigger CCL management. The existing CCL mechanism places a burden on the consumer to monitor the network and decide whether to instantiate a CCL, update a CCL, or delete a CCL. A possible improvement may be to allow the consumer to define trigger conditions for automated instantiation, update and deletion of a CCL.

The MnS consumer may want to request for a CCL to be dynamically instantiated when certain conditions are met. For example, the MnS consumer may want that for a CCL of a stated type or that matches a set of stated characteristics (e.g. goal) to be instantiated under conditions A and another with variations in goals to be instantiated under other conditions. The MnS consumer should be enabled to define those conditions so that the CCL is instantiated when the stated conditions are met.

#### 5.x.2.2				Conditional execution of CCLs network changes

For the CCLs that have been instantiated, the MnS consumer may want to define conditions under which a CCL may execute actions onto the network, e.g. when the performance on a certain threshold is crossed, or when the confidence is the decision is above a stated threshold. The consumer does not need to be aware of all decisions, but by providing conditions under which decisions may be activated or not, it is able to have supervision over the CCL without having to continuously track the decisions. The MnS consumer should be enabled to define those conditions for executing the CCL actions. Otherwise, the consumer should be enabled to define alternative actions, e.g. to notify the consumer of the decision that is not executed.

By supporting this, the execution can be affected by producer based on consumer's conditions or requirements.

### 5.x.2					Requirements

Table 5.x.2-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-TRI-FUN-01 | The 3GPP management system should enable authorized consumers to provide information that can be used to trigger CCL instantiation | Conditional trigger of a CCL |
| REQ-TRI-FUN-02 | The 3GPP management system should enable authorized consumers to provide information that can be used to trigger CCL update. | Conditional trigger of a CCL |
| REQ-TRI-FUN-03 | The 3GPP management system should enable authorized consumers to provide information that can be used to trigger CCL deletion. | Conditional trigger of a CCL |
| REQ-TRI-FUN-03 | The 3GPP management system should enable authorized consumers to define conditions under which a CCL may execute actions onto the network. | Conditional execution of CCLs network changes |

End of Change