3GPP TSG-SA5 Meeting #159	S5-250857

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	TS28.567 CCL decision escalation

# Document for:	Approval

Agenda Item:	6.19.4.1

# 1	Decision/action requested

Group is requested to discuss and agree on the “Detailed Proposal”.

# 2	References

[1]	None

# 3	Rationale

# 4	Detailed proposal

Start of First change

# 5	Management Capabilities

## 5.C	 CCL decision escalation – CCLESC

### 5.C.1	Description

This use case related to the capability to escalate decision making to another entity e.g. another CCL

### 5.C.2	Use Cases

## 5.C.2.1	Triggering CCL decision escalation – ESCTRG

CCLs will make decisions in different contexts (states, status, conditions, etc.) of the network and not all decisions are equally effective. In some cases, the CCL may need to provide feedback indication the challenges faced on reaching a definite decision. The feedback may include a request to escalate its decision making to another entity.

A CCL can be fully or partially autonomous derivation or execution of its decisions. The degree to which the CCL independently executes decisions or escalates them, should be flexibly configurable by the MnS consumer as a confidence threshold. The confidence threshold could be configured based on the sensitivity of the operations under its control, the trust level in the decisions of the CCL and the necessity to consider a bigger picture at times. Then, based on how much confidence the CCL has in its decisions, the CCL can escalate a decision or situation to an escalation recipient which has this bigger picture (say has wider scope), can execute a different(larger) set of actions or has better capabilities, e.g. a larger and more capable ML model.

NOTE 1:	The computation of confidence within the CCL is up to implementation as it depends on the CCL's purpose and the scenario that the CCL is addressing.

The escalation recipient CCL enables the escalator CCL to request for escalation for a given network context or state with e.g. information about the escalator CCL preferences and observed constraints when driving decisions. Based on its evaluations, the escalation recipient CCL should provide to the escalator CCL a report that holds the outcomes that the CCL (acting as an escalation recipient) has derived for a given escalation request.

NOTE 2:	How this relates with existing Assurance closed loop execution supervision use case in TS 28.535 [2] is FFS.

Figure 5.C.1-1: End-to-end flow of CCL decision escalation

### 5.C.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-CCLESC-ESCTRG-01 | The CCL MnS producer should have a capability to enabling an authorized consumer to configure the degree of autonomy of the CCL as characterization of the conditions under which the ACCL should escalate a decision. | CCLESC-ESCTRG |
| REQ-CCLESC-ESCTRG-02 | The CCL MnS producer should have a capability to enabling an authorized consumer to configure the entity to which a decision should be escalated. | CCLESC-ESCTRG |
| REQ-CCLESC-ESCTRG-03 | The CCL MnS producer (acting as an escalation recipient CCL) should have a capability to enabling an authorized MnS consumer (e.g. an escalator CCL) to request escalation of a decision or escalation of decision-making for a given network context or state to the CCL associated with the CCL MnS producer. | CCLESC-ESCTRG |
| REQ-CCLESC-ESCTRG-04 | The 3GPP management system (or the CCL Mns producer) should have a capability enabling an MnS consumer to provide information related to its previous decisions, decision constraints, preferences, … as input to be used in resolving escalations sent towards the CCL associated with the CCL MnS producer. | CCLESC-ESCTRG |
| REQ-CCLESC-ESCTRG-05 | The CCL MnS producer (acting as an escalation recipient CCL) should have a capability to provide to an authorized MnS consumer (e.g. an escalator CCL) a report that holds the outcomes that the CCL (acting as an escalation recipient) has derived for a given escalation request. | CCLESC-ESCTRG |

End Change