3GPP TSG-SA5 Meeting #159	S5-250867

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:                         China Mobile, NTT DOCOMO

# Title:	pCR Add use case and solution for 3GPP management architecture evolution based on PaaS

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

For approval

# 2	References

- 3GPP TR 28.869 v1.2.0 Study on cloud aspects of management and orchestration.

# 3	Rationale

The contribution proposes to add a use case and solution for 3GPP management architecture evolution based on PaaS.

# 4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change

### 5.1.X	Use case #X: 3GPP management architecture evolution based on PaaS

#### 5.1.X.1	Description

A Telco Platform as a Service (PaaS) is a set of cloud services (related to compute, network, storage, and management) that facilitate the deployment and operation of Network Functions, as software applications, on an underlying generic and decoupled cloud infrastructure. See ETSI GR NFV 003 [11] for a definition of the term PaaS Service.

NOTE:	According to ETSI GS NFV-IFA 049 [2], a VNF generic OAM function is a PaaS Service of a specific type, i.e. a PaaS Service that provides and handles generic OAM functionality for VNFs.

There are many kinds of PaaS service types and PaaS capabilities in IT industry. A Telco PaaS can also be used to address fundamental problems like the scalability issue. For example, using entirely separate monitoring tools for storage resources, network resources, and NF application metrics creates scalability issues from the perspectives of both the 3GPP management system and the cloud infrastructure.

3GPP standardization work is currently missing modeling and interactions that can enable the use of PaaS capabilities; PaaS capabilities can be used to support advanced capabilities provided by the 3GPP management system, realize multi-network element sharing, reduce duplication of work, and improve management efficiency in a standardized way. For example, a Telco PaaS can also offer support for different cloud IaaS platforms. This can also enhance portability of Network Functions from one cloud environment to another one. Additional examples of Telco PaaS include services offering management of DNS, generic repository, authentication, monitoring, load balancing, firewall and connectivity, such as service meshes.

This use case considers the interaction of the 3GPP management system with a Telco PaaS management entity through a PaaS reference point

#### 5.1.X.2	Potential requirements

REQ _PaaS-1 The PaaS reference point should support the capability allowing the 3GPP management system to interact with external (non-3GPP) management entities to support the management of cloud native VNFs.

#### 5.X.1.3	Potential solutions

##### 5.X.1.3.1	High level architecture evolution

As shown in figure 5.X.1.3.1-1, this solution introduces a management entity providing Telco PaaS services that interacts with 3GPP management system for management of cloud-native VNFs via a new generic PaaS reference point.

Figure 5.X.1.3-1: High level architecture evolution solution for 3GPP management architecture evolution based on PaaS

Editor’s Note: to provide more details on the PaaS Reference point, the PaaS entity and possible updates needed in the 3GPP management system.

NOTE: the specification of the PaaS reference point is outside of the 3GPP management system. Existing interfaces specified by other SDOs and/or other de-facto solutions can be used to support communication and interactions over the PaaS reference point between the 3GPP management system and PaaS services.

End of Changes