3GPP TSG-SA5 Meeting #159	S5-250868

Sophia Antipolis, France, 17 - 21 February 2025

# Source:                         NTT DOCOMO, China Mobile

# Title:	pCR TR28.869 VNF configuration management

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

- 3GPP TR 28.869 v1.2.0 Study on cloud aspects of management and orchestration.

# 3	Rationale

The contribution proposes to add solution and evaluation for Cloud-native VNF configuration management.

# 4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change

### 5.1.1	Use case #1: Cloud-native VNF configuration management

#### 5.1.1.1	Description

Besides allocation and configuration of the necessary resources, to bring up a cloud native VNF into operation, i.e. full instance provisioning and configuration of the corresponding VNF instances needs to take place. Configuration actions can also be performed during the cloud native VNF instance operation.

An operator needs to be able to manage and orchestrate through the 3GPP management system the configuration of the cloud native VNF instances.

For data governance reasons, the operator needs to be able to control the location where configuration data is stored. Additional information is needed that can be used by the MnS producer to support operations like:

- backup configuration information,

- rollback configuration information (e.g., after reverting software to a previous stable version),

- remove unnecessary configuration files (e.g., very old versions),

#### 5.1.1.2	Potential requirements

REQ-CVNF_CM-1 The 3GPP management system should have the capability to perform configuration operations for cloud-native VNF instances.

REQ-CVNF_CM-2 The reference point between the 3GPP management system and external OAM entities should have the capability enabling the 3GPP management system to interact with external (non-3GPP) configuration management entities for the purpose of performing configuration operations for cloud-native VNF instances.

Editor's Note: The use of the term cloud native network function and cloud native VNFs in the present document is FFS.

2nd Change

#### 5.1.1.3	Potential solutions

##### 5.1.1.3.1	VNF Configuration Manager function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution proposes the use of the VNF Configuration Manager function defined in ETSI GS NFV-IFA 049 [2]. Some key functionalities supported by the VNF Configuration Manager function are the capability to convey configuration information to one or more cloud-native VNF/VNFC instances, the capability to perform pre-configuration actions (e.g. create configuration backup) and post-configuration actions (e.g. rollback running configuration) and the capability to query configuration information of cloud-native VNF/VNFC instances.

From 3GPP management system perspective, MnS provisioning handles configuration of managed NFs defined by 3GPP, which includes the necessary configuration related to the behavior and role of a NF (e.g., configuration of a gNB CU UP function). The 3GPP management system fully understands the semantics of this kind of configuration since it is needed to ensure that the 3GPP mobile network operates as intended by the network operator.

The VNF Configuration Manager does not understand the semantics of the configuration information that is conveyed to the cloud-native VNF/VNFC instances (both VNF application configuration parameters and VNF non-application configuration parameters).

Figure 5.1.1.3.1-1 depicts the interaction and reference point between 3GPP management system and the VNF Configuration Manager.

Figure 5.1.1.3.1-1: Interaction and reference point between 3GPP management system and VNF Configuration Manager

NOTE:	As specified in ETSI GS NFV-IFA 049 [2], the VNF Configuration Manager is a VNF generic OAM function, while close mapping between VNF generic OAM functions and Platform as a Service (PaaS) Services exists. According to ETSI GS NFV-IFA 049 [2], any VNF generic OAM function can be understood as a PaaS Service of a specific type, i.e. a PaaS Service that provides and handles generic OAM functionality for VNFs. See ETSI GR NFV 003 [11] for a definition of the term PaaS Service.

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-2.

This potential solution related to option 2 in clause 5.1.5.

Editor’s Note: update the solution description to better reflect the impact to the 3GPP management system.

##### 5.1.1.3.2	Network Configuration Manager function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution considers the use of the Network Configuration Manager function defined in ETSI GS NFV-IFA 049 [2]. The Network Configuration Manager function can be used to set network configuration information related to one or more VNF/VNFC instances. For example, the Network Configuration Manager can be used to configure the VNF Connection Points (CPs), VNF networking for VNF instances in a service mesh, etc.

Figure 5.1.1.3.2-1 depicts the interaction and reference point between 3GPP management system and the Network Configuration Manager.

Figure 5.1.1.3.2-1: Interaction and reference point between 3GPP management system and Network Configuration Manager

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-2.

This potential solution related to option 2 in clause 5.1.5.

##### 5.1.1.3.3	Configuration Server

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution proposes the use of a Configuration Server introduced in ETSI GR NFV-EVE 022 [44]. The Configuration Server can be understood as a logically centralized storage for configuration, without limiting the implementation of the storage. In addition, Configuration Server services can be used to store and fetch configuration data, convert configuration data formats, validate configuration data schema and apply version control for configuration data. The interface exposed and operations supported by the Configuration Server are defined in ETSI GS NFV-IFA 049 [2].

NOTE:	Implementation of the Configuration Server, e.g. structure of configuration data, format of storage is not in scope of the present solution.

Figure 5.1.1.3.3-1 depicts the interaction and reference point between 3GPP management system and the Configuration Server.

Figure 5.1.1.3.3-1: Interaction and reference point between 3GPP management system and Configuration Server

As depicted in Figure 5.1.1.3.3-1, a consumer of the Configuration Server interfaces could be other VNF generic OAM function such as the VNF Configuration Manager defined in ETSI GS NFV-IFA 049 [2] or the 3GPP management system.

The solution enables the 3GPP management system having the capability to handle configuration data, like distributing, fetching and transforming configuration data, for one or multiple cloud-native VNFs.

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-2.

This potential solution related to option 2 in clause 5.1.5

##### 5.1.1.3.4	Using the existing 3GPP provisioning management service and ETSI NFV MANO

In this solution a MnS producer offering the MnS provisioning service directly interacts with the VNF for VNF application specific parameters configuration purposes.

- The 3GPP management system configures VNF with VNF application specific parameters by utilizing already defined 3GPP provisioning MnS (as defined in clause 11.1 of TS 28.532 [10]).

- The 3GPP management system interacts with NFV-MANO to configure VNF non-application parameters (as defined in clause 5.1.18 of TS 28.531[7]).

This solution is not using the VNF Generic OAM functions described in ETSI ISG NFV-IFA049 [2].

This potential solution related to option 1 in clause 5.1.5.

2nd Change

##### 5.1.1.3.5	Proposed updates in the 3GPP management system

From a 3GPP management system point of view, additional information is needed by the MnS producer to support:

- backup of configuration data,

- rollback of configuration data,

- removal of unnecessary configuration files (e.g., very old versions)

Editor’s Note: it is TBD to determine possible options to support the operations above (e.g., introduce a new parameter configBackupInfo under vnfParametersList defined in TS 28.622 [X] etc.).

3rd Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[2]	ETSI GS NFV-IFA 049: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; VNF generic OAM functions and other Paas Services specification".

[3]	ETSI GR NFV-EVE 019: "Network Functions Virtualisation (NFV);Architectural Framework; Report on VNF generic OAM functions".

[4]	3GPP TR 28.834: "Technical Specification Group Services and System Aspects; Study on management of cloud-native Virtualized Network Functions (VNF)".

[5]	SP-230764 New WID on Management of cloud-native Virtualized Network Functions.

[6]	3GPP TS 28.526: "Technical Specification Group Services and System Aspects; Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[7]	3GPP TS 28.531: "Technical Specification Group Services and System Aspects; Management and orchestration; Provisioning".

[8]	ETSI GS NFV-IFA 013 (V4.5.1) (2023-09): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[9]	ETSI GS NFV-IFA 008 (V4.3.1) (2022-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[10]	3GPP TS 28.532: "Technical Specification Group Services and System Aspects; Management and orchestration; Generic management services".

[11]	ETSI GR NFV 003 (V1.8.6): "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[12]	3GPP TS 28.555: "Technical Specification Group Services and System Aspects; Management and orchestration; Network policy management for 5G mobile networks; Stage 1".

…

NOTE:	Available at https://github.com/cncf/toc/blob/main/DEFINITION.md.

[43]	"The twelve-factor app".

NOTE:	Available at https://12factor.net.

[44]	ETSI GR NFV-EVE 022: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; Report on VNF configuration".

[X]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

End of Changes