3GPP TSG-SA5 Meeting #159	S5-250874

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | 0148 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.533 Enhance SBMA definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is proposed to enhance SBMA definitions in TS 28.533 according to the WT-1 of Rel-19 SBMA_Ph3 WI objectives. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhancing the Management Services definition text and basic MnS concept figure. Other editorial enhancements are also proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are some ambiguities in the MnS definitions that are not conducive to the correct use of MnS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2.1, 4.2.3.1, 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 4	Service Based Management Architecture (SBMA)

## 4.1	Management Services (MnS)

The fundamental building block of the Service Based Management Architecture (SBMA) is the Management Service (MnS). An MnS is a set of offered capabilities for management and orchestration of network and services. The entity producing an MnS is called MnS producer. The entity consuming an MnS is called MnS consumer.

An MnS producer offers its MnS via a standardized service interface composed of individually specified MnS components. This service interface is represented in Figure 4.1.1. The MnS offering is depicted with a circle symbol attached by a solid line to the MnS producer. The MnS consumption is depicted with a socket symbol attached by a solid line to the MnS consumer.

MnS

Figure 4.1.1: MnS producer and MnS consumer

## 4.2	MnS components

### 4.2.1	Introduction

An MnS is specified using different independent components. A concrete MnS is composed of at least two of these components. Three different component types are defined, called MnS component type A, MnS component type B and MnS component type C. These components are defined in the following clauses 4.2.2 and 4.2.3.

Next change

### 4.2.3	Management information

#### 4.2.3.1	MnS component type B

MnS component type B refers to management information represented by information models representing the managed entities. A MnS component type B is also called Network Resource Model (NRM).

MnS component type B examples are:

1)	Network resource models as defined in TS 28.622 [32].

2)	 Network resource models as defined in TS 28.541 [4].

Next change

## 4.3	Combination of MnS components

An MnS is composed by an MnS component type A and

-	an MnS component type B, or

-	an MnS component type B and an MnS component type C.

The instances of management services carry information about specified management service components in the metadata attributes. Figure 4.3.1.illustrates examples of management service instances with various management service components of type A, type B and type C:

Figure 4.3.1: Examples of Management Service instances and their corresponding MnS component type A, B and C

End of change