3GPP TSG-SA5 Meeting #159	S5-250877

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | 0030 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.537 Add reference for solution description (NRM fragment) for each management capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The solution for improving the readability of 3GPP TS 28.537 to illustrate the reference for corresponding stage2 and stage 3 solutions for each management capability is documented in TR 28.871 and recommended for normative work. So, it proposes to add reference for corresponding stage2 and stage 3 solutions for each management capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference corresponding stage2 and stage 3 solutions for each management capability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.X (new), 5.2.X(new), 6.1.X (new), 7.2.X(new), 7.4.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[3]	3GPP TS 28.533: " Management and orchestration; Architecture framework".

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[6]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 32.404: "Telecommunication management; Performance Management (PM); Performance measurements; Definitions and template".

[8]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[X]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) "

[Y]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions "

2nd Change

4	Heartbeat

4.1	Overview

The communication between Management Service (MnS) producers and MnS consumers shall be monitored, and communication link breaks between them shall be discovered by MnS consumers as early as possible. The behaviour of the MnS consumers, after detection of communication failure, is outside the scope of the present document.

4.2	Specification level requirements

4.2.1	Use cases

4.2.1.1	Configuring heartbeat notification periodicity

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To configure the periodicity at which the management service producer shall emit heartbeat notifications to its authorized management service consumer. |  |
| Actors and Roles | An authorized consumer of the management service. |  |
| Telecom resources | The management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The periodicity requested by the management service consumer has a valid value. |  |
| Begins when | The management service consumer sends a request to the management service producer to set the periodicity at which it shall emit heartbeat notifications. |  |
| Step 1 | The management service producer receives the request and sets its internal countdown timer to a value (which can be zero) equal to the periodicity requested by the management service consumer. |  |
| Step 2 | The management service producer sends a heartbeat notification to all authorized management service consumer(s), provided they previously subscribed to heartbeat notifications. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The notification periodicity has been configured according to the management service consumer request. A heartbeat notification is sent out to all authorized management service consumer(s). |  |
| Traceability | REQ-HB-CTRL-2. |  |

4.2.1.2	Requesting immediate heartbeat notification

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To trigger the emission of an immediate heartbeat notification by the management service producer. |  |
| Actors and Roles | An authorized consumer of the management service. |  |
| Telecom resources | The management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The soliciting management service consumer sends a request to the management service producer to emit immediately a heartbeat notification. |  |
| Step 1 | The management service producer receives the request and sends immediately a heartbeat notification to all authorized management service consumer(s) who had previously subscribed to heartbeat notifications. The management service producer countdown timer is not impacted. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The immediate heartbeat notification has been emitted according to the soliciting management service consumer request. |  |
| Traceability | REQ-HB-CTRL-3, REQ-HB-NOTIF-2. |  |

4.2.1.3	Emitting periodic heartbeat notifications

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To send periodic heartbeat notifications at the periodicity requested by the management service consumer. |  |
| Actors and Roles | An authorized producer of the management service. |  |
| Telecom resources | The management service consumer. |  |
| Assumptions | The heartbeat notification periodicity has been configured according to the management service consumer request. | Configuring heartbeat notification periodicity |
| Pre-conditions | N/A |  |
| Begins when | The internal countdown timer managed by the management service producer has reached the value 0. |  |
| Step 1 | The management service producer sends a heartbeat notification to all authorized management service consumer(s), provided they previously subscribed to heartbeat notifications. |  |
| Step 2 | The management service producer resets its internal countdown timer to the value of the heartbeat notification periodicity. | Configuring heartbeat notification periodicity |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The periodic heartbeat notification has been emitted to all authorized management service consumer(s) at the requested periodicity. |  |
| Traceability | REQ-HB-NOTIF-1. |  |

4.2.2	Requirements

4.2.2.1	Requirements for controlling heartbeat

REQ-HB-CTRL-1: The management service provider shall have the capability to allow its authorized consumer to read the heartbeat period.

REQ-HB-CTRL-2: The management service provider shall have the capability to allow its authorized consumer to set the heartbeat period.

REQ-HB-CTRL-3: The management service provider shall have the capability to allow its authorized consumer to request the emission of an immediate heartbeat notification.

4.2.2.2	Requirements for notifying heartbeat

REQ-HB-NOTIF-1: The management service provider shall have the capability to send periodic heartbeat notifications to its authorized consumer at the frequency specified by the management service consumer.

REQ-HB-NOTIF-2: The management service provider shall have the capability to send immediate heartbeat notifications to its authorized consumer, upon request from the management service consumer.

4.3	Procedures for heartbeat

4.3.1	Procedure for configuring heartbeat notification periodicity

Figure 4.3.1-1 illustrates the procedure for configuring the periodicity of heartbeat notifications using operations and notifications of the provisioning MnS (see clause 11.1.1 of [2]).

Figure 4.3.1-1: Procedure for configuring heartbeat notification periodicity

4.3.2	Procedure for requesting immediate heartbeat notification

Figure 4.3.2-1 illustrates the procedure for requesting immediate heartbeat notification using operations and notifications of the provisioning MnS (see clause 11.1.1 of [2]).

Figure 4.3.2-1: Procedure for requesting immediate heartbeat notification

4.3.3	Procedure for notifying periodic heartbeat notifications

Figure 4.3.3-1 illustrates the procedure for notifying periodic heartbeat notifications using operations and notifications of the provisioning MnS (see clause 11.1.1 of [2]).

Figure 4.3.3-1: Procedure for notifying periodic heartbeat notifications

4.X	Solutions

4.X.1	Stage 2 definition

Following are the stage 2 definitions for controlling heartbeat:

- The operations and notifications of generic provisioning MnS defined in clause 11.1 in TS 28.532 [2].

- The Heartbeat notification control NRM fragment defined in clause 4.3.21 and clause 4.3.22 in TS 28.622 [X].

Following are the stage 2 definition for notifying heartbeat

- The Heartbeat notification defined in clause 11.4 in TS 28.532 [2].

4.X.2	Stage 3 definition

Following are the stage 3 definition for controlling heartbeat

- RESTful HTTP-based solution set

- RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in TS 28.532 [2]. Correponding OpenAPI document "TS28532_ProvMnS.yaml" in clause A.1 in TS 28.532 [2].

- OpenAPI document "TS28623_SubscriptionControlNrm.yaml" in clause 4.3 in TS 28.623 [Y].

- YANG/Netconf-based solution set

- YANG/Netconf-based solution set for generic provisioning management service is defined in clause 12.1.3 in TS 28.532 [2].

- YANG model "_3gpp-common-subscription-control.yang" in clause 4.4. in TS 28.623 [Y].

Following are the stage 3 definition for notifying heartbeat

-	RESTful HTTP-based solution set for heartbeat notification is defined in clause 12.1.1 in TS 28.532 [2].

-	OpenAPI document " TS28532_HeartbeatNtf.yaml " in clause A.5 in TS 28.532 [2].

2nd Change

5	Discovery of Management Services

5.1	Overview

To enable communication between MnS Consumers and MnS Producers, MnS Consumers need mechanisms to discover management service information available in the 3GPP management system, and their management capabilities.  To this end, MnS Producers and their management capabilities need to be exposed in the 3GPP management system.

From management service perspective the following information can be exposed:

-	Identifying data describing an MnS, e.g. name, version, type

-	Capabilities of an MnS, e.g. supported operations, supported notifications

From MnS Consumer perspective such information can be used for different purposes, including:

-	MnS Producer discovery:  allows MnS Consumer to discover identifying information about an MnS Producer instance.  In short, allows MnS Consumer to know which MnS Producers instances are exposed.

-	MnS Producer Capabilities retrieval:  allows MnS Consumer to retrieve capability information about an MnS Producer instance.  In short, allows MnS Consumer to know what an MnS Producer instance is capable of.

In case an exposed MnS Producer instance’s information changes the 3GPP management system needs to be updated.

MnS Consumers wishing to discover MnS Producer instances might have different questions.  For example, an MnS Consumer may wish to know which MnS Producers manage a certain geographical area or civic location. Or, after receiving an alarm notification specifying that a specific NF is alarmed, they may wish to know the MnS Producers from which they can request management data from that NF or to retrieve the configuration of that NF.

5.2	Specification level requirements

5.2.1	Use cases

5.2.1.1	Adding a new management service producer to MnS registry

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Add a MnS producer to a 3GPP management system. |  |
| Actors and Roles | MnS Producer, MnS Registry |  |
| Telecom resources | MnS producer. MnS discovery service producer. |  |
| Assumptions | MnS producer is ready to be added to MnS registry. |  |
| Pre-conditions | The MnS Producer is available. |  |
| Begins when | There is a need for a MnS producer to be exposed via MnS registry. |  |
| Step 1 (M) | The MnS producer is added to the MnS registry. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | MnS discovery service producer has stored the MnS information. |  |
| Traceability | REQ-DMS-1 |  |

5.2.1.2	Removing a management service producer from MnS registry

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Remove a MnS producerfrom MnS registry.. |  |
| Actors and Roles | Network operator. |  |
| Telecom resources | MnS producer. |  |
| Assumptions | - |  |
| Pre-conditions | The management service producer is no longer required in the MnS registry. |  |
| Begins when | The management service is ready to be removed from MnS Registry. |  |
| Step 1 (M) | The management service producer is removed from the MnS Registry. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | MnS discovery service producer has removed the MnS information related to the MnS Producer. |  |
| Traceability | REQ-DMS-1 |  |

5.2.1.3	MnS Consumer retrieves management service information from MnS registry

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | MnS consumer retrieves information from MnS registry. |  |
| Actors and Roles | MnS Consumer |  |
| Telecom resources | MnS registry |  |
| Assumptions | MnS consumer is authorized to obtain the MnS information for the available management service(s) from MnS discovery service producer. |  |
| Pre-conditions | MnS information exists in MnS registry. |  |
| Begins when | MnS Consumer needs to access a specific MnS Producer(s). |  |
| Step 1 (M) | MnS Consumer queries MnS Registry with filter criteria based on the management service(s) of interest. |  |
| Step 2 (M) | MnS Consumer receives response with MnS Info for the management service(s) which match the criteria. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | MnS Consumer has basic information about the management service(s). |  |
| Traceability | REQ-DMS-2, REQ-DMS-3, REQ-DMS-4 |  |

NOTE: MnS information refer to the information used by the consumer to discover the producers of specific Management Services and to derive the addresses of the Management Service.

5.2.1.4	Providing detailed capabilities about management service

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Management service detailed capabilities are exposed. |  |
| Actors and Roles |  |  |
| Telecom resources | - |  |
| Assumptions | Management service detailed capabilities are available. |  |
| Pre-conditions | Management service detailed capabilities are ready to be exposed. |  |
| Begins when | MnS Producer wants to expose its detailed capabilities. |  |
| Step 1 (M) | Management service detailed capabilities are exposed by MnS Producer. |  |
| Post-conditions | Management services detailed capabilities have been exposed. |  |
| Traceability |  |  |

5.2.1.5	MnS Consumer retrieves detailed capabilities about management service

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | MnS Consumer retrieves detailed capabilities for specific MnS Producer(s). |  |
| Actors and Roles | MnS Consumer |  |
| Telecom resources | - |  |
| Assumptions | Management service has ability to expose its detailed capabilities. |  |
| Pre-conditions | Authorized MnS Consumer knows location and method to retrieve detailed capabilities. |  |
| Begins when | MnS Consumer requires to retrieve detailed capabilities of specific MnS Producer(s). |  |
| Step 1 (M) | MnS Consumer reads detailed capabilities from MnS Producer of interest. |  |
| Post-conditions | MnS Consumer has retrieved detailed capabilities of specific MnS Producer(s). |  |
| Traceability |  |  |

5.2.2	Requirements

REQ-DMS-1: The 3GPP management system shall provide capabilities allowing MnS producers to register their management capabilities (including the endpoint address) at MnS discovery service producer for use by MnS consumers wishing to interact with these MnS producers.

REQ-DMS-2: The 3GPP management system shall provide capabilities allowing MnS consumers to retrieve the management capabilities registered at MnS discovery service producer by MnS producers.

REQ-DMS-3: The 3GPP management system shall provide capabilities allowing to discover MnS producers that are managing a specified managed entity.

REQ-DMS-4: The 3GPP management system shall provide capabilities allowing to discover the managed entities a MnS producer is responsible for.

5.2.X	Solutions

5.2.X.1	Stage 2 definition

Following are the stage 2 definition for Discovery of Management Services:

- The operations and notifications of generic provisioning MnS defined in clause 11.1 in TS 28.532 [2].

- The MnSRegistry NRM fragment defined in clause 4.3.41 and clause 4.3.42  in TS 28.622 [X].

5.2.X.2	Stage 3 definition

Following are the stage 3 definition for Discovery of Management Services:

- RESTful HTTP-based solution set

- RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in TS 28.532 [2]. OpenAPI document "TS28532_ProvMnS.yaml" in clause A.1 in TS 28.532 [2].

- OpenAPI document " TS28623_MnSRegistryNrm.yaml" in clause 4.3 in TS 28.623 [Y].

- YANG/Netconf-based solution set

- YANG/Netconf-based solution set for generic provisioning management service is defined in clause 12.1.3 in 3GPP TS 28.532 [2].

- YANG model " 3gpp-common-mnsregistry.yang" in clause 4.4 TS 28.623 [Y].

3rd Change

6.1	Producing and reporting management data

6.1.1	Description

Management data is referring to data produced by radio access network functions, core network functions or management functions and used for management purposes. Management data specified by 3GPP for 5G management is classified into 5G performance measurements as defined by TS 28.552 [4], 5G end to end key performance indicators as defined by TS 28.554 [5] and Trace metrics as defined by TS 32.423 [8]. The combined performance measurements and key performance indicators are also called performance metrics.

Management data is produced on request. Therefore, the 3GPP management system needs to enable a data consumer to request management data to be produced. The data requester needs to specify the type of data to be produced as well as the radio access network functions, core network functions and management functions where the data shall be produced. The target managed object instances can be identified in multiple ways:

-	The requester can specify the target managed object instances based on the managed object tree (as defined in the 3GPP Network Resource Models) representing the network and management functions. The simplest approach is to directly identify the managed object instances where data shall be produced. More sophisticated approaches allow to specify one or more subtrees where data shall be produced and may allow also to specify managed object classes to select only object instances of specific classes.

-	The requester can specify one or multiple of the following selection criteria. The system needs to translate this information into the target managed object instances. The selection criteria need to be deterministic in such a way that the target node(s) can be selected unambiguously.

-	Area of interest: In a big network, it makes sense to specify a limited area for which data shall be produced. The area of interest can be expressed for example with a geographical area, one or several cells or one or several tracking areas. The target managed object instances represent network functions serving that area of interest. Geographical areas can be expressed for example with multiple longitude-latitude pairs that define a convex polygon. In the radio domain the geographical area needs to be mapped to the coverage area of cells supported by RAN NE(s). The managed object instances (e.g. NRCellCU, GNBDUFunction) providing service to these cells can be identified as target managed object instances. Of course the coverage area of the target cell(s) will usually not exactly map to the described shape of the geographical area because, on the one side, the coverage area of cell has no sharp borders due to fast fading effects, on the other side, the coverage area of cells may vary slightly e.g. due to adaptation of the antenna downtilt angle or beamforming configurations. For the mapping between the geographical area and the corresponding managed object instances the cell coverage status at the time of the request shall be used. Later changes affecting the cell coverage shall not be reflected for the mapping.

-	Domain e.g RAN, CN: A consumer might only be interested in analysing and understanding the performance of a particular domain like RAN or Core e.g in case of recurrent issues, a consumer may want to have understanding of a particular domain only for further actions. In such a scenario, it should be possible to indicate the domain from where consumer wants measurements for its usage.

-	Traffic type e.g user plane or control plane: 5G brings clear separation (CUPS) of user plane and control plane in a network, a consumer may leverage it to identify target managed object instances to have measurements from. For example, the measurement report may be expected from user plane nodes only.

-	Slice type e.g eMBB, URLLC: Consumer may mention a particular slice type (eMBB, URLLC, mIoT, V2X, HMTC) as the selection criteria. It may help in narrowing down the target managed object instances, which are part of provided slice type(s).

This MnS to request management data in a simple way shall not be exposed at any network function.

The management data can be requested according to a certain time scheduler. The consumer can e.g. specify a start and stop time or can request for data at specific days of a week or specific hours of a day..

After production the data needs to be reported to the data consumers. Reporting can be based on multiple reporting methods such as file or streaming. Data reporting needs to be requested by the data consumer. The requestor needs to specify the control parameters for reporting such as the reporting method and the address the data shall be delivered to.

Depending on access rights and security settings, data consumers may be subject to restrictions regarding the data they can access.

Data is always produced in some context. The data describing this context is called context data. Context data contains information on all interrelated conditions in which the management data is produced. This includes for example the configuration of the measured network functions, information on the network entity where the network function is running such as vendor name or software version, but also alarms associated to the network function or load conditions.

Data consumers processing managemen data in an effort to accomplish some task typically benefit when taking context data into account. For that reason data consumers should be able to obtain the context data for the management data they obtain. However, access to certain management data does not automaticlly imply access to all context data. Access to management data and access to context data may be subject to different data security and data protection considerations.

6.1.2	Void

6.1.3	Requirements

REQ-MDM-PR-1: The 3GPP management system shall enable an authorized data consumer to request management data (specified by 3GPP) to be produced.

REQ-MDM-PR-2: The 3GPP management system shall enable an authorized data consumer to request management data specified by 3GPP to be produced by certain managed object instance(s) only. The selection criteria to determine the managed object instance(s) shall be deterministic in such a way that the target node(s) can be selected unambiguously. The managed object instances can be targeted based on:

-	Area of interest (e.g. list of cells, list of tracking areas or geographical area).

-	Domain (CN or RAN).

-	User plane or control plane.

-	Slice type (e.g. eMBB, URLLC, mIoT, V2X, HMTC).

The MnS to request management data specified by 3GPP in a simple way shall not be exposed at any network function.

The mapping of geographical area to corresponding managed object instances reflects the cell coverage status at the time of the request.

REQ-MDM-PR-3: The 3GPP management system shall enable an authorized data consumer to request management data specified by 3GPP to be produced according to a certain time scheduler.

REQ-MDM-PR-4: The 3GPP management system shall enable an authorized data consumer to request management data (specified by 3GPP) to be reported to the requesting or another authorized data consumer.

REQ-MDM-PR-5: The 3GPP management system shall enable an authorized data consumer to obtain context data for management data. Access to management data does not imply access to context data. Different data privacy considerations may apply.

NOTE: The term "management data specified by 3GPP" relates to

-	5G performance measurements as defined by TS 28.552 [4]

-	5G end to end key performance indicators as defined by TS 28.554 [5], and

-	Trace metrics as defined by TS 32.423 [8].

6.1.X	Solutions

6.1.X.1	Stage 2 definition

Following are the stage 2 definition for producing and reporting management data.

- The operations and notifications of generic provisioning MnS defined in clause 11.1 in TS 28.532 [2].

- The ManagementDataCollection control NRM fragment defined in clause 4.3.47 in TS 28.622 [X].

6.1.X.2	Stage 3 definition

Following are the stage 3 definition for MnS Registry and Discovery management capability.

- RESTful HTTP-based solution set

- RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in 3GPP TS 28.532 [2]. OpenAPI document "TS28532_ProvMnS.yaml" in clause A.1 in TS 28.532 [2].

- OpenAPI document " TS28623_ManagementDataCollectionNrm.yaml " in clause 4.3 in TS 28.623 [Y].

- YANG/Netconf-based solution set

- YANG/Netconf-based solution set for generic provisioning management service is defined in clause 12.1.3 in 3GPP TS 28.532 [2].

- YANG model "_3gpp-common-managementdatacollection.yang " in clause 4.4 in TS 28.623 [Y].

4th Change

7.2	File retrieval from a MnS producer by a MnS consumer

7.2.1	Description

File retrieval is when the MnS consumer retrieves (gets) a file from the MnS producer. Either the MnS consumer retrieves a file from the MnS producer because the MnS consumer receives a file ready notification from the MnS producer or the MnS consumer reads the list of available (ready) files on the MnS producer and decides to retrieve an available file.

7.2.2	Void

7.2.3	Requirements

REQ-FM-R-1: The MnS producer shall support the capability allowing a MnS consumer to retrieve (get) a file from the MnS producer.

REQ-FM-R-2: The MnS producer shall support the capability allowing a MnS consumer to retrieve the list of files available for transfer from the MnS producer.

REQ-FM-R-3: The MnS producer shall support the capability to inform a MnS consumer about files that are available for retrieval.

REQ-FM-R-4: The MnS producer shall support the capability to inform a MnS consumer about errors that occurred during the preparation of a file.

REQ-FM-R-5: The information transferred to a MnS consumer about an available file shall allow associating the file to the process on the MnS producer that generated the file, if any such process exists and has an identifier.

REQ-FM-R-6: The MnS producer shall support the capability allowing a MnS consumer to indicate to the MnS producer, that the MnS consumer does not need a file anymore, such that the MnS producer hides the file in responses to subsequent read requests or decide to delete it altogether.

7.2.X	Solutions

7.2.X.1	Stage 2 definition

Following are the stage 2 definition for File retrieval from a MnS producer by a MnS consumer.

- The operations and notifications of generic provisioning MnS defined in clause 11.1 in 3GPP TS 28.532 [2].

- The File retrieval NRM fragment defined inTS 28.622 [X].

7.2.X.1	Stage 3 definition

Following are the stage 3 definition for File retrieval from a MnS producer by a MnS consumer.

- RESTful HTTP-based solution set

- RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in 3GPP TS 28.532 [2]. Corresponding OpenAPI document "TS28532_ProvMnS.yaml" in clause A.1 in TS 28.532 [2].

- OpenAPI document " TS28623_FileManagementNrm.yaml " in clause 4.3 in TS 28.623 [Y].

- YANG/Netconf-based solution set

- YANG/Netconf-based solution set for generic provisioning management service is defined in clause 12.1.3 in 3GPP TS 28.532 [2].

- YANG model "_3gpp-common-files.yang " in clause 4.4 TS 28.623 [Y].

5th Change

7.4	File download from a MnS consumer to a MnS producer

7.4.1	Description

File download is when the MnS producer gets a file from the MnS consumer or a designated file server because the MnS consumer requests the MnS producer to download the file.

7.4.2	Void

7.4.3	Requirements

REQ-FM-D-1: The MnS producer shall support the capability to download a file from a MnS consumer or a designated file server when triggered by a MnS consumer.

REQ-FM-D-2: The MnS producer shall support the capability allowing a MnS consumer to trigger the MnS producer to download a file from the MnS consumer or a designated file server.

REQ-FM-D-3: The MnS producer shall support the capability to inform the MnS consumer that has triggered a file download, or any other consumer about the progress of that file download.

7.4.X	Solutions

7.4.X.1	Stage 2 definition

Following are the stage 2 definition for File download from a MnS consumer to a MnS producer.

- The operations and notifications of generic provisioning MnS defined in clause 11.1 in 3GPP TS 28.532 [2].

- The File download NRM fragment defined inTS 28.622 [X].

7.4.X.2	Stage 3 definition

Following are the stage 3 definition for File download from a MnS consumer to a MnS producer.

- RESTful HTTP-based solution set

- RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in 3GPP TS 28.532 [2].

- OpenAPI document "TS28532_ProvMnS.yaml" in clause A.1 in TS 28.532 [2].

- OpenAPI document " TS28623_FileManagementNrm.yaml " in clause 4.3 in TS 28.623 [Y].

- YANG/Netconf-based solution set

- YANG/Netconf-based solution set for generic provisioning management service is defined in clause 12.1.3 in 3GPP TS 28.532 [2].

- YANG model "_3gpp-common-filemanagement.yang" in TS 28.623 [Y].

End of Changes