3GPP TSG-SA5 Meeting #159	S5-250909

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.572 Add clause Fallback configurations and Managing fallback configurations

# Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## 6.7	Fallback configurations

### 6.7.1	Definition

Fallback is the process of returning to the configuration that was in place before the activation of a planned configuration or planned configuration group. Fallback configurations are generated by the system. Fallback may be applied when the activation of a planned configuration or a planned configuration group does not yield the desired results. Use cases include:

- A wrong planned configuration was activated. The operator wants to return to the old configuration.

- The new configuration does not perform as expected. The operator or the automation system wants to return to the old configuration.

- The new configuration was deployed on existing Network Functions in anticipation of some new Network Functions being rolled out. The roll out does not happen. The existing Network Functions are rolled back to the configuration state at the time the activation was triggered.

Fallback may be implemented by one of the following methods:

- "undo operations": This method only reverses the changes resulting from the activation of a planned configuration or planned configuration group. It has no effect on configuration data nodes that are not affected by the activation of the planned configuration or planned configuration group.

- "restore complete configuration": This method reverts all configuration changes done after the activation of a planned configuration or planned configuration group, i.e. those resulting from the activation of a planned configuration or planned configuration group and those done via some other means. This method restores the complete old configuration.

Editor's note: This clause needs further clean up to separate both fallback methods.

### 6.7.2	Fallback preparation

A fallback configuration is created by the system upon the start of an activation process when enable fallback is requested. The format of the fallback configuration is dependent on the fallback method.

In case of "undo operations" the fallback configuration shall be visible to the consumer as a read-only planned configuration. A fallback configuration is always a single configuration (not a plannned configuration group) and is atomic. For example, for a planned configuration that adds a new object to the current data node tree the fallback configuration simply removes this object.

In case of "restore configuration" the fallback configuration is implementation specific.

No fallback on fallback is possible.

Editor's notte: If no cancelling is possible is ffs.

### 6.7.3	Meta data for fallback configurations

Each fallback configuration is described by meta data. This includes the name, the version, and a human readable textual description.

Furthermore, an identifier of the activation process whose result is to be rolled back must be provided.

### A fallback is always atomic.

### 6.7.4	Fallback configuration descriptor

Editor's note: This descriptor is ffs.

End of modifications