3GPP TSG-SA5 Meeting #159	S5-250954

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0025 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.540 Add requirements for management of RedCap feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_OAM |  |  |  |  |  | Date: |  |  | 2025-01-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The use case and requirements for NG-RAN configuration to support RedCap features is described in clause 5.1 in TR 28.876 and recommended for normative work. Som it proposes to add concept and requirements for management of RedCap features in TS 28.540. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add concept and requirement for management of RedCap features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 4.X (new), 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[6]	3GPP TS 28.531: "Management and orchestration of 5G networks; Provisioning".

[7]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[8]	GSMA NG.116 – "Generic Network Slice Template" v1.0 (2019-05-23).

[9]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[X]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

2nd Change

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G Core Network: Defined in 3GPP TS 23.501 [2].

AMF Region: Defined in 3GPP TS 23.501 [2].

AMF Set: Defined in 3GPP TS 23.501 [2].

en-gNB: Defined in 3GPP TS 37.340 [5].

gNB: Defined in 3GPP TS 38.300 [3].

gNB Central Unit (gNB-CU): Defined in 3GPP TS 38.401 [4].

gNB-CU-Control Plane (gNB-CU-CP): Defined in 3GPP TS 38.401 [4].

gNB-CU-User Plane (gNB-CU-UP): Defined in 3GPP TS 38.401 [4].

gNB Distributed Unit (gNB-DU): Defined in 3GPP TS 38.401 [4].

ng-eNB: Defined in 3GPP TS 38.300 [3].

NG-RAN: Defined in 3GPP TS 23.501 [2].

RedCap UE: defined in 3GPP TS 38.331 [X].

eRedCap UE: defined in 3GPP TS 38.331 [X].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core network

5GS	5G System

AMF	Access and Mobility Management Function

EN-DC	E-UTRA-NR Dual Connectivity

EPS	Evolved Packet System

MR-DC	Multi-RAT Dual Connectivity

NG-RAN	NG Radio Access Network

NR	New Radio

PCF	Policy Control Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

3rd Change

4.X	Management of RedCap feature

RedCap is a lightweight network access solution aiming at scenarios with relatively low cost, low energy consumption, and low data rate requirement, mainly applying to scenarios such as industrial sensors, video surveillance, wearable scenarios, smart grid, etc.

NOTE:	The term RedCap UE used in the present document refers to both RedCap UE and eRedCap UE defined in TS 38.331 [X].

In some scenarios, operator may want to prohibit RedCap UEs to access specific NR cells for a specified time to improve the network performance for non-RedCap UEs. After the specified time, operator will allow RedCap UEs to access specific NR cells again. So, the 3GPP management system should have the capability to allow operators to configure the NR cell to allow or prohibit RedCap UEs to access.

An initial BWP can be configured for RedCap UEs' initial access instead of initial BWP that may exceed the (e)RedCap UE maximum bandwidth. So, the 3GPP management system should have the capability to configure initial BWP separately for RedCap UEs.

4th Change

5.X	Requirements for management of RedCap feature

REQ-RedCap_NRNRM-1: The NR NRM definition should support a capability to allow MnS consumer to configure Multi-Initial BWPs to support RedCap and Non-RedCap UEs co-existence scenario.

REQ-RedCap_NRNRM-2: The NR NRM definitions should support a capability to allow MnS consumer to configure the NR cell to allow or prohibit RedCap UEs to access.

REQ-RedCap_NRNRM-3: The NR NRM definitions should support a capability to allow MnS consumer to configure initial BWP separately for RedCap UEs.

End of Changes