3GPP TSG-SA5 Meeting #159	S5-250994

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0298 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Address the CR implmentation issue for infeasibilityReasons in CR252 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The enum values (including INVALID_INTENT_EXPRESSION, INTENT_CONFLICT) for infeasibilityReasons are proposed in CR252 in SA5#157 meeting. However, it is missing in TS 28.312 due to the implementation issue. Also these two enum values are already defined in TS28312_IntentNrm.yaml. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add enum values (including INVALID_INTENT_EXPRESSION, INTENT_CONFLICT) for infeasibilityReasons in TS 28.312 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mismatch of stage 2 and stage 3 definition for infeasibilityReasons |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1625 at commit 890f85f6499dddd9fa96e56d1a7caee4ab68d249 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.2.1.4	Attribute definition

Table 6.2.1.4-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentExpectations | It describes the expectations including requirements, goals and contexts (including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for expectation object(s). The intentExpectations are arranged in an ordered list such that the most important intentExpectations are on the top of the list. allowedValues: Not Applicable | type: IntentExpectation multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| intentFulfilmentInfo | It describes status of fulfilment of an intent and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationFulfilmentInfo | It describes status of fulfilment of an intentExpectation and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetFulfilmentInfo | It describes status of fulfilment of an expectationTarget and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fulfilmentStatus | It describes the current status of the fulfilment result for intent, intentExpectation or expectationTarget, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", "NOT_FULFILLED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "NOT_FULFILLED" isNullable: False |
| notFulfilledState | It describes the current state for not achieving fulfilment for the intent, intentExpectation or expectationTarget. It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: "ACKNOWLEDGED", "COMPLIANT", "DEGRADED", "SUSPENDED", "TERMINATED" "FULFILMENTFAILED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ACKNOWLEDGED" isNullable: False |
| notFulfilledReasons | It describes the reasons/observations related to the specific notFulfilledState allowedValues: Not Applicable | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentContexts | It describes the list of IntentContext(s) which represents the constraints and conditions that should apply for the entire intent even if there may be specific contexts defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationId | A unique identifier of the intentExpectation within the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationVerb | It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations. Examples of verbs and their related types of expectation are Deliver: DeliveryIntentExpectation, e.g. Deliver a RAN network, Service, Slice, function Ensure: AssuranceintentExpectation, e.g. Ensure the target performance value. allowedValues: DELIVER, ENSURE Vendor extensions are allowed | type: String multiplicity: 1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationObject | It describes the expectation objects to which the IntentExpectation should apply. allowedValues: Not Applicable | type: ExpectationObject multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectType | It describes the type of expectation object of the IntentExpectation that is required to be applied to. It can be class name of the managed object. allowedValues: see scenario specific IntentExpectation | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | It describes a specific object instance (e.g. instance of managed object) to which the intentExpectation should apply. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectContexts | It describes the list of ObjectContext(s) which represents the constraints and conditions to be used as filter information to identify the object(s) to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent, for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. The concrete ObjectContext depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ObjectContexts follow the common structure of ObjectContext. | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationTargets | It describes the list of ExpectationTarget(s) which represent specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. The concrete ExpectationTarget depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ExpectationTargets follow the common structure of ExpectationTarget. The expectionTargets are arranged in an ordered list such that the most important expectionTargets are on the top of the list. | type: ExpectationTarget multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| expectationContexts | It describes the list of context(s) which represents the constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Expectation Object in the IntentExpectation | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetName | It describes the name of the expectation target which represents specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. allowedValues: depends on ExpectationObject in the IntentExpectation | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| targetCondition | It expresses the limits within which the targetName is allowed/supposed to be. allowedValues: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE", "IS_ONE_OF", " IS_EQUAL_TO_OR_LESS_THAN”, "IS_EQUAL_TO_OR_GREATER_THAN", "IS_NOT_ONE_OF", "IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "IS_EQUAL_TO" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the targetCondition. allowedValues: depends on the targetCondition. The value will be a single value when the targetCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS EQUAL TO OR LESS THAN", "IS EQUAL TO OR GREATER THAN” The value will be a pair of values when the targetCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the targetCondition is "IS_ONE_OF", "IS_NOT_ONE_OF","IS_ALL_OF". See NOTE 1. | type: ValueRangeType multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| targetContexts | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: Not Applicable | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc) to which the expectation should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE, "IS_ONE_OF", "IS_EQUAL_TO_OR LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN", "IS_NOT_ONE_OF", "IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "IS_EQUAL_TO" isNullable: False |
| contextValueRange | It describes the range of values that applicable to the ContextAttribute and the ContextCondition. AllowedValue: depends on the contextCondition The value will be a single value when the contextCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_EQUAL_TO_OR_LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN". The value will be a pair of values when the contextCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the contextCondition is "IS_ONE_OF", "IS_NOT_ONE_OF","IS_ALL_OF". See NOTE 1. | type: ValueRangeType multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| intentPriority | It expresses the priority of the stated intent within an MnS consumer. AllowedValue: values in the range [1-100] where 1 indicates the highest priority and 100 indicates the lowest priority. NOTE: The handing of the priorities across MnS consumers is left to implementation | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 1 isNullable: False |
| geoArea | It describes a geographical area defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: GeoArea multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| pLMNId | It describes the information of a PLMN identification defined in 3GPP 28.658[10] AllowedValue: As defined by the data type | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dateTime | It describes the information of a date time defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| timeWindow | It describes the information of a time window (including startTime, endTime) defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| geoCoordinate | It describes the information of a geoCoordinate defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: GeoCoordinate multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| frequency | It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number) and/or the frequency operating band used for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD. AllowedValue: As defined by the data type | type: Frequency multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| arfcn | It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number). Allowed Value: For NR, see TS 38.104 [8] clause 5.4.2.1. For EUTRAN, see TS 36.104 [14] clause 5.7.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| freqband | It desribes the the frequency operating band. Allowed Value: For NR, see TS 38.104 [8] clause 5.4.2.3. For EUTRAN, see TS 36.104 [14] clause 5.7.3. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uEGroup | It describes the information of a UE Group (represented by specific 5QI, specific S-NSSAI, or a specific combination of S-NSSAI and 5QI). AllowedValue: As defined by the data type | type: UEGroup multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| fiveQI | It describes the information of a 5QI defined in 3GPP TS 28.541[5]. AllowedValue: 0 - 255 | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sNSSAI | It describes the information of a S-NSSAI defined in 3GPP TS 28.541[5]. AllowedValue: As defined by the data type | type: S-NSSAI multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intentAdminState | It describes the intent administrative state, which enables the MnS consumer to suspend an intent or cancel the suspension for a suspended intent. A suspended intent means this intent is not considered for fulfilment allowedValues: "ACTIVATED", "DEACTIVATED" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ACTIVATED" isNullable: False |
| intentReference | It indicates the associated intent instance allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentReportReference | It indicates the associated intent report instance(s) allowedValues: Not Applicable | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| observationPeriod | It represents the observation period of the fulfilmentInfo for corresponding ExpectationTargets, IntentExpectations and Intent. At the end of the observation period, the corresponding fulfilment info is updated in the intent report. The observation period can be assigned by MnS consumer through requesting the MnS producer to set attribute "observationPeriod". MnS producer also can assign the observation period if MnS consumer didn’t assign it. The observation time is expressed in seconds. allowedValues: Not Applicable | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentFulfilmentReport | It describes the fulfillment information which is reported for the associated intent instance. allowedValues: Not Applicable | type: IntentFulfilmentReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentConflictReports | It describes the conflict information which is reported for associated intent instance if needed. allowedValues: Not Applicable | type: IntentConflictReport multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conflictId | It is used to identify the detected conflict within an IntentReport instance. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictType | It describes the type of intent conflict. allowedValues: INTENT_CONFLICT, EXPECTATION_CONFLICT, TARGET_CONFLICT | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingIntent | It describes the DN of the conflicting intent allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingExpectation | It describes the expectationId of the conflicting IntentExpectation within an Intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingTarget | It describes the targetName of the conflicting ExpectationTarget within an IntentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedSolutions | It describes the action recommended by the MnS producer to be undertaken by the MnS consumer to resolve intent conflict. The recommended solution applies only for the specific intent whose intent report contains this attribute. allowedValues: "MODIFY", "DELETE" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationFulfilmentResults | It includes the expectationFulfilmentInfo and targetFulfilmentResults for each IntentExpectation. The expectationFulfilmentInfo describes status of fulfilment of an intentExpectation and the related reasons for infeasible status. allowedValues: Not Applicable | type: ExpectationFulfilmentResult multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetFulfilmentResults | It includes targetFulfilmentInfo and targetAchievedValue for each ExpectationTarget. The targetFulfilmentInfo describes status of fulfilment of an expectationTarget and the related reasons for infeasible status. The targetAchieveValue describes current performance value for the ExpectationTarget. allowedValues: Not Applicable | type: TargetFulfilmentResult multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetAchievedValue | It describes the value that has been achieved for the expectation target at the time at which the report is generated. allowedValues: Not Applicable | type: Number multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentFeasibilityCheckReport | It describes the intent feasibility check information which is reported if needed. allowedValues: Not Applicable | type: IntentFeasibilityCheckReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| feasibilityCheckResult | It describes the result of intent fulfilment feasibility check allowedValues: FEASIBLE, INFEASIBLE | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| infeasibilityReasons | It describes the reason (e.g. invalid intent expression, the intent conflict) of the result of intent fulfilment feasibility check is INFEASIBLE allowedValues: INVALID_INTENT_EXPRESSION, INTENT_CONFLICT | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentHandlingCapabilityList | It describes the list of expectation object information and expectation target information which can be supported by intent handling function. allowedValues: Not Applicable | type: IntentHandlingCapability multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentHandlingCapabilityId | A unique identifier of property of intent handling capability should be supported by the intent handling function of MnS producer. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedExpectationObjectType | It describes the expectation object type which can be supported by a specific intent handling function of MnS producer. allowedValues: objectType defined in clause 6.2.1.3.2. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedExpectationTargetNames | It describes the supported expectation targets for the supported expectation object type. allowedValues: targetName defined in clause 6.2.1.3.3 | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| lastUpdatedTime | It describes the time for the latest update of the IntentReport Instance. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ContextSelectivity | It expresses the may in which all or a subset of the expectationTargets may be applied. AllowedValue: "ALL_OF", "ONE_OF", "ANY_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ALL_OF" isNullable: False |
| intentPreemptionCapability | It describes the pre-emption capability. The attribute is used by MnS producer to decide the target of intent deletion or intent modification allowedValue: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |
| NOTE: For "IS_ALL_OF", the value shall be a match of the entire list. |  |  |

2nd Change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1625 at commit 890f85f6499dddd9fa96e56d1a7caee4ab68d249

*** START OF CHANGE 1 ***

*** OpenAPI/TS28312_IntentNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Intent NRM

version: 19.0.0

description: >-

OAS 3.0.1 definition of the Intent NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.312; Intent driven management services for mobile networks

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.312/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-IntentNrm:

type: object

properties:

IntentHandlingFunction:

$ref: '#/components/schemas/IntentHandlingFunction-Multiple'

#-------Definition of generic IOCs ----------#

Intent-Single:

description: >-

This IOC represents the properties of an Intent driven management information between MnS consumer and MnS producer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

userLabel:

type: string

intentExpectations:

type: array

uniqueItems: true

minItems: 1

items:

type: object

oneOf:

- $ref: "#/components/schemas/IntentExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/RadioNetworkExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/EdgeServiceSupportExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/5GCNetworkExpectation"

- $ref: "TS28312_IntentExpectations.yaml#/components/schemas/RadioServiceExpectation"

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

intentContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

description: >-

It describes the list of Context(s) which represents the constraints and conditions that should apply

for the entire intent even if there may be specific contexts defined for specific parts of the intent

intentAdminState:

type: string

enum:

- ACTIVATED

- DEACTIVATED

description: >-

It describes the intent administrative state.

This attribute is used when MnS consumer-suspension mechanism is supported

intentPriority:

type: integer

minimum: 1

maximum: 100

description: It expresses the priority of the stated intent within a MnS consumer.

intentPreemptionCapability:

type: boolean

observationPeriod:

type: integer

description: >-

It represents the observation period of the fulfilmentInfo for corresponding

ExpectationTargets, IntentExpectations and Intent.

intentReportReference:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

IntentReport-Single:

description: It represents intent report information from MnS producer to MnS consumer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

intentFulfilmentReport:

$ref: '#/components/schemas/IntentFulfilmentReport'

intentConflictReports:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/IntentConflictReport'

intentFeasibilityCheckReport:

$ref: '#/components/schemas/IntentFeasibilityCheckReport'

lastUpdatedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

intentReference:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

IntentHandlingFunction-Single:

description: >-

It represents the intent handling capabilities can be supported by a specific intent

handling function of MnS producer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

intentHandlingCapabilityList:

type: array

uniqueItems: true

minItems: 1

items:

$ref: '#/components/schemas/IntentHandlingCapability'

Intent:

$ref: '#/components/schemas/Intent-Multiple'

IntentReport:

$ref: '#/components/schemas/IntentReport-Multiple'

#-------Definition of generic IOCs ----------#

#-------Definition of the generic IntentExpectation dataType ----------#

IntentExpectation:

description: >-

This data type is the "IntentExpectation" data type without specialisations

It represents MnS consumer's requirements, goals and contexts given to a 3GPP system

type: object

properties:

expectationId:

type: string

description: A unique identifier of the intentExpectation within the intent.

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObject:

$ref: "#/components/schemas/ExpectationObject"

expectationTargets:

type: array

uniqueItems: true

minItems: 1

items:

$ref: '#/components/schemas/ExpectationTarget'

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

expectationContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

required:

- expectationId

#-------Definition of the generic IntentExpectation dataType ----------#

#-------Definition of the generic ExpectationObject dataType ----------#

ExpectationObject:

description: >-

It represents the Object to which the IntentExpectation should apply.

This data type is the "ExpectationObject" data type without specialisations

type: object

properties:

objectType:

type: string

enum:

- RAN_SUBNETWORK  #value for Radio Network Expectation--#

- EDGE_SERVICE_SUPPORT  #value for Edge Service Support Expectation--#

- 5GC_SUBNETWORK  #value for 5GC Network Expectation--#

- Radio_Service  #value for Radio Service Expectation--#

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

objectContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

description: >-

It describes the list of Context(s) which represents the constraints and conditions to be

used as filter information to identify the object(s) to which a given intentExpectation should apply.

#-------Definition of the generic ExpectationObject dataType ----------#

#-------Definition of the generic dataType --------------#

Condition:

type: string

enum:

- IS_EQUAL_TO

- IS_LESS_THAN

- IS_GREATER_THAN

- IS_WITHIN_RANGE

- IS_OUTSIDE_RANGE

- IS_ONE_OF

- IS_NOT_ONE_OF

- IS_EQUAL_TO_OR_LESS_THAN

- IS_EQUAL_TO_OR_GREATER_THAN

- IS_ALL_OF

Selectivity:

type: string

enum:

- ALL_OF

- ONE_OF

- ANY_OF

FulfilmentStatus:

type: string

readOnly: true

enum:

- FULFILLED

- NOT_FULFILLED

default: NOT_FULFILLED

description: It describes the current status of the intent fulfilment result.

NotFulfilledState:

type: string

readOnly: true

enum:

- ACKNOWLEDGED

- COMPLIANT

- DEGRADED

- SUSPENDED

- TERMINATED

- FULFILMENTFAILED

default: ACKNOWLEDGED

description: It describes the current progress of or the reason for not achieving fulfilment

for the intent, intentExpectation or expectationTarget.

An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

FulfilmentInfo:

description: >-

This dataType represents the properties of a specific fulfilment information for an aspect of

the intent (i.e. either an expectation, a target or the whole intent).

type: object

properties:

fulfilmentStatus:

$ref: '#/components/schemas/FulfilmentStatus'

notFullfilledState:

$ref: "#/components/schemas/NotFulfilledState"

notFulfilledReasons:

type: array

uniqueItems: true

items:

type: string

readOnly: true

description: An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

ExpectationVerb:

type: string

enum:

- DELIVER

- ENSURE

description: It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations. Vendor extensions are allowed

Frequency:

description: >-

It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number)

and/or the frequency operating band used for a given direction (UL or DL) in FDD or

for both UL and DL directions in TDD.

type: object

properties:

arfcn:

type: integer

description: >-

This attribute shall be supported, when the frequency information represent RF reference frequency.

The allowed values for NR see TS 38.104 subclause 5.4.2.1; The allowed values for EUTRAN see TS 36.104 [X] subclause 5.7.3;

freqband:

type: string

description: >-

This attribute shall be supported, when the frequency information represent frequency operating band.

The allowed values for NR see TS 38.104 subclause 5.4.2.3; The allowed value for EUTRAN see TS 36.104 subclause 5.7.3

ValueRangeType:

oneOf:

- type: number

- type: string

- type: boolean

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoCoordinate'

- $ref: '#/components/schemas/UEGroup'

- $ref: '#/components/schemas/Frequency'

UEGroup:

description: >-

It describes the UE Group, which is

represented by specific 5QI, specific S-NSSAI, or a specific combination

of S-NSSAI and 5QI

type: object

properties:

fiveQI:

type: integer

minimum: 0

maximum: 255

sNssai:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/Snssai'

#-------Definition of the generic dataType --------------#

#-------Definition of the generic ExpectationTarget dataType----------#

ExpectationTarget:

description: >-

This data type represents the target of the IntentExpectation that are required to be achieved.

This data type is the "ExpectationTarget" data type without specialisations

type: object

properties:

targetName:

type: string

targetCondition:

$ref: '#/components/schemas/Condition'

targetValueRange:

oneOf:

- type: array

uniqueItems: true

minItems: 1

items:

$ref: "#/components/schemas/ValueRangeType"

- $ref: "#/components/schemas/ValueRangeType"

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

targetContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

description: It describes the list of constraints and conditions that should apply for a specific expectationTarget.

#-------Definition of the generic ExpectationTarget  dataType----------#

#-------Definition of the generic Context dataType----------------#

Context:

description: >-

This data type is the "Context" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: '#/components/schemas/Condition'

contextValueRange:

oneOf:

- type: array

uniqueItems: true

minItems: 1

items:

$ref: "#/components/schemas/ValueRangeType"

- $ref: "#/components/schemas/ValueRangeType"

#-------Definition of the generic Context dataType----------------#

#-------Definition of the generic IntentFulfilmentReport dataType----------------#

IntentFulfilmentReport:

description: >-

It includes the intentFulfilmentInfo and expectationFulfilmetResult.

This attribute shall be supported when intent fulfilment information is supported by IntentReport

type: object

properties:

intentFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

expectationFulfilmentResult:

type: array

uniqueItems: true

minItems: 1

items:

$ref: '#/components/schemas/ExpectationFulfilmentResult'

#-------Definition of the concrete IntentFulfilmentReport dataType----------------#

#-------Definition of the generic ExpectationFulfilmentResult dataType----------------#

ExpectationFulfilmentResult:

description: >-

It includes the expectationFulfilmentInfo and targetFulfilmentResults for each IntentExpectation.

type: object

properties:

expectaitonId:

type: string

readOnly: true

expectationFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

targetFulfilmentResults:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/TargetFulfilmentResult'

#-------Definition of the concrete ExpectationFulfilmentResult dataType----------------#

#-------Definition of the generic TargetFulfilmentResult dataType----------------#

TargetFulfilmentResult:

description: >-

This data type includes targetFulfilmentInfo and targetAchievedValue for each ExpectationTarget.

type: object

properties:

targetName:

type: string

readOnly: true

targetFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

targetAchievedValue:

type: number

description: >-

It describes the value that has been achieved for the expectation target at the time at which

the report is generated.

readOnly: true

#-------Definition of the concrete TargetFulfilmentResult dataType----------------#

#-------Definition of the generic IntentConflictReport dataType----------------#

IntentConflictReport:

description: >-

It represents the conflict information for the detected conflict

This attribute shall be supported when intent conflict information is supported by IntentReport

type: object

properties:

conflictId:

type: string

readOnly: true

conflictType:

type: string

readOnly: true

enum:

- INTENT_CONFLICT

- EXPECTATION_CONFLICT

- TARGET_CONFLICT

conflictingIntent:

description: >-

This will be present if the value of conflictType is INTENT_CONFLICT. It describes the DN of the conflicting intent

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

conflictingExpectation:

description: >-

This will be present if the value of conflictType is EXPECTATION_CONFLICT. It describes the expectationId of the conflicting IntentExpectation with an Intent

type: string

readOnly: true

conflictingTarget:

description: >-

This will be present if the value of conflictType is TARGET_CONFLICT. It describes the targetName of the conflicting ExpectationTarget with an IntentExpectation

type: string

readOnly: true

recommendedSolutions:

type: string

readOnly: true

enum:

- MODIFY

- DELETE

#-------Definition of the concrete IntentConflictReport dataType----------------#

#-------Definition of the generic IntentFeasibilityCheckReport dataType----------------#

IntentFeasibilityCheckReport:

description: >-

It represents the intent feasibility check information

This attribute shall be supported when intent feasibility check information information is supported by IntentReport

type: object

properties:

feasibilityCheckResult:

type: string

readOnly: true

enum:

- FEASIBLE

- INFEASIBLE

infeasibilityReasons:

description: An attribute which is used when feasibilityCheckResult is INFEASIBLE

type: array

items:

type: string

readOnly: true

enum:

- INVALID_INTENT_EXPRESSION

- INTENT_CONFLICT

#-------Definition of the concrete IntentFeasibilityCheckReport dataType----------------#

#-------Definition of the generic IntentHandlingCapability dataType----------------#

IntentHandlingCapability:

description: >-

It represents expectation object information and expectation target information

which can be supported by a specific intent handling function of MnS producer.

type: object

properties:

intentHandlingCapabilityId:

type: string

readOnly: true

supportedExpectationObjectType:

type: string

enum:

- RAN_SUBNETWORK

- EDGE_SERVICE_SUPPORT

- 5GC_SUBNETWORK

- Radio_Service

readOnly: true

description: It describes the expectation object type which can be supported by a specific intent handling function of MnS producer.

supportedExpectationTargetNames:

type: array

uniqueItems: true

minItems: 1

items:

type: string

readOnly: true

description: It describes the supported expectation targets for the supported expectation object type.

#-------Definition of the concrete IntentHandlingCapability dataType----------------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

Intent-Multiple:

type: array

items:

$ref: '#/components/schemas/Intent-Single'

IntentReport-Multiple:

type: array

items:

$ref: '#/components/schemas/IntentReport-Single'

IntentHandlingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/IntentHandlingFunction-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

resources-intentNrm:

oneOf:

- $ref: '#/components/schemas/IntentHandlingFunction-Single'

- $ref: '#/components/schemas/Intent-Single'

- $ref: '#/components/schemas/IntentReport-Single'

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

<CODE ENDS>

*** END OF CHANGE 1 ***

End of Changes