# 3GPP TSG-SA5 Meeting #159	S5-250995

# 17 - 21 February 2025, Sophia Antipolis, France                          revision of S5-250105

# Source:	Ericsson Hungary

# Title:		Discussion paper on: How to use notifyPotentialFaultyAlarmList

# Document for:	   Endorsement

# Agenda Item:	6.4.5

# 1	Decision/action requested.

The group is asked to endorse the proposal.

# 2	References

[1] 	3GPP TS 28.111: "Management and orchestration; Fault Management (FM)".

# 3	Rationale

Observation 1: Unknown, infinite length of the potential-faulty AlarmList state

If an MnS producer sends the notification notifyPotentialFaultyAlarmList the consumer knows that the AlarmList is in a state where part or the whole of the AlarmRecords attribute is unreliable. However, there is no mandatory indication about when the this unreliable state ends:

- The notification notifyAlarmListRebuilt is optional to support and use.

- The attribute AlarmList.unreliableAlarmScope is optional to support.

Due to the lack of a reliably available indication of the end of the faulty state the consumer does not know whether to handle the error situation and how. If the client explicitly reads/syncs the AlarmList the faulty state may still persist, so the synchronisation may have been to early. The client could wait with the synchronization, but has no indication how long to wait.

So from the consumer point of view, once notifyPotentialFaultyAlarmList is received the AlarmRecords (or the part of the AlarmRecords indicated by the objectInstance) is unreliable once and for all.

An interoperable multi-vendor consumer needs to mostly ignore notifyPotentialFaultyAlarmList (except maybe logging it) as there is no reliable method to detect the end of the AlarmList’s faulty period.

# 4	Detailed proposal

The group is requested to endorse the following recommendations.

Recommendation 1: If the producer is able to detect a faulty or potentially faulty AlarmList condition, it should support and send the notifyPotentialFaultyAlarmList notification.

Recommendation 2: If the notification notifyPotentialFaultyAlarmList is supported/used by a producer it shall also support/send the notifyAlarmListRebuilt notification when the (potentially) faulty situation ends. (Make it mandatory if notifyPotentialFaultyAlarmList is supported)