3GPP TSG-SA5 (Telecom Management)	S5-08xyzw

Meeting SA5#60bis, 10-12 September 2008, Budapest, HUNGARY	revision of S5-080xyz

- Source:	Orange SA

- Title:	Proposition of functionalities definitions meeting Orange requirements

- Document for:	Approval, Information, Discussion

Agenda Item:

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

(Reference - in list form - should be made to previous SA5/3GPP/etc. documents.)

# 3	Rationale

This contribution tries to define functionalities Orange needs to deliver the AoC service. These functionalities meet Orange requirements and try to keep the vision of CDP/CGP in 3GPP TS 29.458.

First definition:

- The "Functionality 1" determines which tariff/add-on charge should be applied, inserts the charging information to the appropriate messages, and defines rules to be used by " Functionality 2" when the last receives a remote charge/tariff. The local "Functionality 1" could be localized in the OCS (e.g. the rating function) or in a local database (e.g SIP AS). The remote "Functionality 1" may be implemented in a SIP AS located in the Home Network, Visited Network or in another network, including a CS-based network where the MGCF acts as a remote " Functionality 1" vis-à-vis the AoC function.

Second definition:

- The " Functionality 2" computes costs with the local tariff received from the " Functionality 1" and can take into account (i.e. filter, decide to aggregate, forward or not) a remote tariff/charge received from a remote " Functionality 1". It supports protocols described in 3GPP TS 32.299 and 3GPP TS 29.458, and may convert tariff information from one to the other. It includes the resulted cost in an AoC information message and sends it to the UE according to the 3GPP TS 24.447. The " Functionality 2" may be an originating SIP AS at the home IMS network for AoC purposes).

# 4	Detailed proposal