3GPP TSG-SA5 (Telecom Management)	S5-094263

Meeting SA5#68, 9 - 13 Nov 2009, Shanghai, China	revision of S5-093752

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0079 |  rev | - |  Current version: | 9.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Modification on SDP handling in IMS Charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH9 |  |  |  |  |  |  | Date:  |  | 12/11/2009 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | In IMS Charging the SDP offer can include several different media, which might be used for the IMS service, but only the SDP answer correctly describes the final negotiated media, i.e. the SDP media exchanged in the SDP answer are used for the IMS service which is currently not recorded. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | An alternative trigger for the ACR start/interim is introduced to provide SDP data, which are actually used by the IMS users, to the CDF. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Subscriber and Inter-operator Billing errors for NGN Charging while using differend Multimedia components. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 5.2.1	Basic Principles

The offline charging functionality is based on the IMS network nodes reporting accounting information upon reception of various SIP methods or ISUP messages, as most of the accounting relevant information is contained in these messages. This reporting is achieved by sending Diameter Accounting Requests (ACR) [start, interim, stop and event] from the IMS network elements to the CDF.

The Diameter client uses ACR start, interim and stop in procedures related to successful SIP sessions. It uses ACR events for unsuccessful SIP sessions and for session unrelated procedures. Further details are specified in the tables below and in clause 5.2.2.

It is operator configurable in the nodes for which SIP method or ISUP messages an Accounting Request is sent. Table 5.2.1.1 describes all possible ACRs that might be sent from a P-CSCF, I-CSCF, S-CSCF, MGCF or BGCF. A list of node specific ACRs, along with the AVPs to be included are detailed in TS 32.299 [50].

The ACRs to be sent from a MRFC are described in table 5.2.1.2.

It is configurable for the operators to enable or disable the generation of an ACR message by the IMS node in response to a particular "triggering SIP method /ISUP message". However, for those table entries marked with *, the operator can enable or disable the ACR message based on whether or not the SIP (Re) Invite message that is replied to by the "triggering SIP method /ISUP message" carried piggybacked user data.

Table 5.2.1.1: Accounting Request messages triggered by SIP Methods or ISUP Messages
for all IMS nodes except for MRFC and AS

| Diameter message | Triggering SIP Method /ISUP message |
| --- | --- |
| ACR [Start] | SIP 200 OK acknowledging an initial SIP INVITE (see note 2 and note 4) (not applicable for BGCF) |
|  | SIP ACK acknowledging an initial SIP INVITE (including a SDP answer, see note 4) |
|  | ISUP:ANM (applicable for the MGCF) |
| ACR [Interim] | SIP 200 OK acknowledging a SIP RE-INVITE or SIP UPDATE [e.g. change in media components] (see note 2) |
|  | SIP ACK acknowledging an initial SIP INVITE or a SIP RE-INVITE (including a SDP answer, see note 4) |
|  | Expiration of AVP [Acct-Interim-Interval] (see note 2) |
|  | SIP Response (4xx, 5xx or 6xx), indicating an unsuccessful SIP RE-INVITE or SIP UPDATE |
| ACR [Stop] | SIP BYE message (both normal and abnormal session termination cases) (see note 2) (not applicable for BGCF) |
|  | ISUP:REL (applicable for the MGCF) |
| ACR [Event] | SIP 200 OK acknowledging non-session related SIP messages, which are: |
|  | SIP NOTIFY (see note 1 and note 2) |
|  | SIP MESSAGE |
|  | SIP REGISTER (see note 1) |
|  | SIP SUBSCRIBE (see note 3) |
|  | SIP PUBLISH |
|  | SIP 200 OK acknowledging an initial SIP INVITE (only for BGCF) |
|  | SIP 202 Accepted acknowledging a SIP REFER or any other method |
|  | SIP Final Response 2xx (except SIP 200 OK) |
|  | SIP Final/Redirection Response 3xx |
|  | SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful SIP session set-up |
|  | SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful session-unrelated procedure |
|  | SIP CANCEL, indicating abortion of a SIP session set-up (see note 2) |
|  | I-CSCF completing a Cx Query that was issued in response to a SIP INVITE |
| NOTE 1: SIP REGISTER with its "Expires" header field or "Expires" parameter equal to 0 or local deregistration due to expiry means Deregistration (see 3GPP TS 24.229 [204]). NOTE 2: (only for I-CSCF): This trigger may only occur if I-CSCF is acting in THIG mode. NOTE 3: SIP SUBSCRIBE with the field "Expires" set to 0 means unsubscribe. NOTE 4: If at the time when the SIP 200 OK is received only the SDP offer is available, the CTF may trigger the ACR message immediately (subsequent ACK containing the SDP answer triggers ACR(interim)),or may trigger the ACR message once the SIP ACK has been received. The precise behaviour shall depend on operator policy. |  |

- Table 5.2.1.2: Accounting Request messages triggered by SIP Methods for the MRFC

| Diameter message | Triggering SIP Method |
| --- | --- |
| ACR [Start] | SIP 200 OK acknowledging an SIP INVITE for initiating a multimedia ad hoc conferencing session |
| ACR [Interim] | SIP ACK acknowledging a SIP INVITE to connect an UE to the conferencing session |
|  | SIP REINVITE (see Note 1) |
|  | SIP BYE (see Note 2) |
|  | Expiration of AVP [Acct-Interim-Interval] |
| ACR [Stop] | SIP BYE message (see Note 3) |
|  | SIP CANCEL (see Note 3) |
|  | SIP Final Response with error codes 4xx, 5xx or 6xx indicating termination of an ongoing session (see Note 3) |
| ACR [Event] | SIP Final/Redirection Response 3xx |
|  | SIP Final Response with error codes 4xx, 5xx or 6xx indicating termination of an ongoing session |
|  | SIP CANCEL, indicating abortion of a SIP session set-up |
|  | SIP REFER |
|  | SIP SUBSCRIBE |
| NOTE 1: This trigger only applies to a user joining an ongoing conferencing session NOTE 2: This trigger only applies to a user leaving an ongoing conferencing session NOTE 3: This trigger only applies if this causes the ongoing conferencing session to terminate |  |

End of modifications