3GPP TSG-SA5 (Telecom Management)	S5-094320

Meeting SA5#68, 09 -13 Nov 2009, Shanghai, PRC	revision of S5-094241

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.299 | CR | 0296 |  rev | - |  Current version: | 9.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add CSG parameters for CSG based online and offline charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH9 |  |  |  |  |  |  | Date:  |  | 13/11/2009 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | Charging requirements related to "Enhanced Home NodeB / eNodeB” have been introduced for EPC. The associated “User CSG information” are not considered for CSG based charging. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Define User-CSG-Information AVP, and add this AVP in PS-information Add “User Information Change” in Change-Condition for offline charging Add “User Information Change” in Trigger-type for online charging |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Charging for network services via a CSG cell is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 3.3, 7.2, 7.2.25A, 7.2.xx, 7.2.xx, 7.2.124, 7.2.176, 7.2.xx |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging Architecture and Principles".

[2]-[49]	Void.

[50]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”

[51]-[101]	Void.

[102]	3GPP TS 22.115: "Service aspects; Charging and billing".

[103]-[199]	Void.

[200]	3GPP TS 23.207: "End to end quality of service concept and architecture".

[201]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[202]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP; Stage 3."

[203]	3GPP TS 29.207: "Policy control over Go interface".

[204]	3GPP TS 29.229: "Cx and Dx Interfaces based on the Diameter protocol; Protocol Details".

[205]	Void.

[206]	3GPP TS 29.230: "3GPP specific codes and identifiers".

[207]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[208]	3GPP TS 23.140: " Multimedia Messaging Service (MMS); Functional description; Stage 2".

[209]	OMA "Multimedia Messaging Service; Encapsulation Protocol".

[210]	OMNA WSP Content Type Numbers 
http://www.openmobilealliance.org/tech/omna/omna-wsp-content-type.aspx

[211]	OMA-CP-POC: "OMA PoC Control Plane"

[212]	3GPP 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[213]	3GPP TS 29.140: "MM10 interface based on Diameter protocol; Stage 3".

[214]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point; Stage 3".

[215]	3GPP TS 29.212: "Policy and Charging Control over Gx reference point".

[216]	3GPP TS 23.040: "Technical realization of Short Message Service (SMS)".

[217]	3GPP TS 22.142: "Value Added Services (VAS) for Short Message Service (SMS) requirements".

[218]	3GPP TS 23.203: "Policy and Charging control architecture".

[219]	3GPP TS 29.272: " Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol”.

[220]	3GPP TS 24.605: "Conference (CONF) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[221]	3GPP TS 29.329: "Sh Interface based on the Diameter protocol;Protocol details".

[222]	3GPP TS 29.658: "SIP Transfer of IP Multimedia Service Tariff Information".

[223] 	OMA-DDS-Charging_Data: "Charging Data"

[xx] 	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[xx] 	3GPP TS 29.274: "Evolved GPRS Tunnelling Protocol for Control Plane (GTPv2-C); Stage 3".

[226] - [400]	Void.

[401]	IETF RFC 3588: "Diameter Base Protocol".

[402]	IETF RFC 4006: "Diameter Credit Control Application"

[403]	IETF Draft, "Carrying Location Objects in RADIUS", draft-ietf-geopriv-radius-lo-04.txt, work in progress

[404]	IETF RFC 3455 , "Private Extensions to the Session Initiation Protocol (SIP) for the 3rd Generation Partnership Projects (3GPP)".

[405]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[406]	IETF Internet-Draft, "SDP: Session Description Protocol".
http://www.ietf.org/internet-drafts/draft-ietf-mmusic-sdp-new-24.txt

[407]	IETF RFC 4005: "Diameter Network Access Server Application".

[408]	IETF RFC 3264: An Offer/Answer Model with the Session Description Protocol (SDP).

NOTE:	The above reference will need to be updated to reference the assigned RFC number, once the draft achieves RFC status within the IETF.

Next Modified Section

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ACA	ACcounting Answer

ACR	ACcounting Request

AoC	Advice of Charge

AS	Application Server

ASA	Abort Session Answer

ASR	Abort Session Request

AVP	Attribute Value Pair

CCA	Credit Control Answer

CCR	Credit Control Request

CDF	Charging Data Function

CDR	Charging Data Record

CEA	Capabilities Exchange Answer

CER	Capabilities Exchange Request

CI	Cost-Information

CSG	Closed Subscriber Group

CSG ID	Closed Subscriber Group Identity

DBPA	Diameter Base Protocol Accounting

DCD	Dynamic Content Delivery

DPA	Disconnect Peer Answer

DPR	Disconnect Peer Request

DWA	Device Watchdog Answer

DWR	Device Watchdog Request

ECUR	Event Charging with Unit Reservation

FUI	Final-Unit-Indication

HSGW	HRPD Serving GateWay

GSU	Granted-Service-Unit

IEC	Immediate Event Charging

IM	Instant Messaging

IMS	IP Multimedia Subsystem

MSCC	Multiple Services Credit Control

OCS	Online Charging System

RAA	Re-Auth Answer

RAR	Re-Auth Request

SDP	Session Description Protocol

Next Modified Section

## 7.2	3GPP specific AVPs

For the purpose of offline charging additional AVPs are used in ACR / ACA and for online charging additional AVPs are used in CCR / CCA. All 3GPP specific AVPs mentioned are relevant for both offline and online charging unless specifically excluded. The information is summarized in the following table along with the AVP flag rules.

The 3GPP Charging Application uses the value 10415 (3GPP) as Vendor-Id.

Detailed descriptions of AVPs that are used specifically for 3GPP charging are provided in the subclauses below the table. However, for AVPs that are just borrowed from other applications only the reference (e.g. TS 29.229 [204]), is provided in the following table and the detailed description is not repeated.

Where 3GPP RADIUS VSAs are re-used, they shall be translated to Diameter AVPs as described in IETF RFC 4005 [407] with the exception that the 'M' flag shall be set and the ''P' flag may be set.

Table 7.2: 3GPP specific AVPs

| AVP Name | AVP Code | Used in |  |  |  | Value Type | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ACR | ACA | CCR | CCA |  | Must | May | Should not | Must not | May Encr. |
| 3GPP-Charging-Characteristics | 13 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Charging-Id | 2 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-GGSN- MCC-MNC | 9 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-IMSI | 1 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-IMSI-MCC-MNC | 8 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-MS-TimeZone | 23 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-NSAPI | 10 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-PDP-Type | 3 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-RAT-Type | 21 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Selection-Mode | 12 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Session-Stop-Indicator | 11 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-SGSN-MCC-MNC | 18 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-User-Location-Info | 22 | X | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP2-BSID | 5535 | X | - | X | - | refer [215] |  |  |  |  |  |
| Access-Network-Charging-Identifier-Value | 503 | X | - | X | - | refer [214] |  |  |  |  |  |
| Access-Network-Information | 1263 | X | - | X | - | OctetString | V,M | P |  |  | N |
| Accumulated-Cost | 2052 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Adaptations | 1217 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Additional-Content-Information | 1207 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Additional-Type-Information | 1205 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Address-Data | 897 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Address-Domain | 898 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Addressee-Type | 1208 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Address-Type | 899 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| AF-Charging-Identifier | 505 | - | - | X | - | refer [214] |  |  |  |  |  |
| AF-Correlation-Information | 1276 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Allocation-Retention-Priority | 1034 | X | - | X | - | refer [215] |  |  |  |  |  |
| Alternate-Charged-Party-Address | 1280 | X | - | - | - | UTF8string | V.M | P |  |  | N |
| AoC-Cost-Information | 2053 | - | - | X | X | Grouped | V,M | P |  |  | N |
| AoC-Information | 2054 | - | - | - | X | Grouped | V.M | P |  |  | N |
| AoC-Request-Type | 2055 | - | - | X | - | Enumerated | V.M | P |  |  | N |
| Application-provided-called-party-address | 837 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Application-Server | 836 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Application-Server-ID | 2101 | X | - | X | - | refer[223] |  |  |  |  |  |
| Application-Server-Information | 850 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Application-Service-Type | 2102 | X | - | X | - | refer[223] |  |  |  |  |  |
| Application-Session-ID | 2103 | X | - | X | - | refer[223] |  |  |  |  |  |
| Applic-ID | 1218 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Associated-Party-Address | 2035 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Associated-URI | 856 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Authorized-QoS | 849 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Aux-Applic-Info | 1219 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Base-Time-Interval | 1265 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Bearer-Service | 854 | X | - | - | - | OctetString | V,M | P |  |  | N |
| Called-Asserted-Identity | 1250 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Called-Party-Address | 832 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Calling-Party-Address | 831 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Carrier-Select-Routing-Information | 2023 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Cause-Code | 861 | X | - | X | - | Integer32 | V,M | P |  |  | N |
| CG-Address | 846 | X | - | X | - | Address | V,M | P |  |  | Y |
| Change-Condition | 2037 | X | - | - | - | Integer32 | V,M | P |  |  | N |
| Change-Time | 2038 | X | - | - | - | Time | V,M | P |  |  | N |
| Charged-Party | 857 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Charging-Rule-Base-Name | 1004 | X | - | X | - | refer [215] |  |  |  |  |  |
| Class-Identifier | 1214 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Client-Address | 2018 | - | - | X | - | Address | V,M | P |  |  | N |
| CN-IP-Multicast-Distribution | 921 | X | - | - | - | refer [207] |  |  |  |  |  |
| Content-Class | 1220 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Content-Disposition | 828 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Content-ID | 2116 | X | - | X | - | refer[223] |  |  |  |  |  |
| Content-Provider-ID | 2117 | X | - | X | - | refer[223] |  |  |  |  |  |
| Content-Length | 827 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Content-Size | 1206 | - | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Content-Type | 826 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| CSG-Access-Mode | 2317 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| CSG-Id | 1437 | X | - | X | - | refer[219] |  |  |  |  |  |
| CSG-Membership-Indication | 2318 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Current-Tariff | 2056 | - | - | X | X | Grouped | V,M | P |  |  | N |
| CUG-Information | 2304 | X | - | - | - | OctetString | V,M | P |  |  | N |
| Data-Coding-Scheme | 2001 | - | - | X | - | Integer32 | V,M | P |  |  | N |
| DCD-Information | 2115 | X | - | X | - | refer[223] |  |  |  |  |  |
| Deferred-Location-Event-Type | 1230 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Delivery-Report-Requested | 1216 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Delivery-Status | 2104 | X | - | X | - | refer[223] |  |  |  |  |  |
| Destination-Interface | 2002 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Diagnostics | 2039 | X | - | - | - | Integer32 | V,M | P |  |  | N |
| Domain-Name | 1200 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| DRM-Content | 1221 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Dynamic-Address-Flag | 2051 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| Early-Media-Description | 1272 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Envelope | 1266 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Envelope-End-Time | 1267 | - | - | X | - | Time | V,M | P |  |  | N |
| Envelope-Reporting | 1268 | - | - | - | X | Enumerated | V,M | P |  |  | N |
| Envelope-Start-Time | 1269 | - | - | X | - | Time | V,M | P |  |  | N |
| Event | 825 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Event-Charging-TimeStamp | 1258 | - | - | X | - | Time | V,M | P |  |  | N |
| Event-Type | 823 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Expires | 888 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| File-Repair-Supported | 1224 | X | - | X | - | Enumerated | V,M | P |  |  | Y |
| Flows | 510 | - | - | X | - | refer [214] |  |  |  |  |  |
| GGSN-Address | 847 | X | - | X | - | Address | V,M | P |  |  | N |
| Guaranteed-Bitrate-UL | 1026 | X | - | X | - | refer[215] |  |  |  |  |  |
| IM-Information | 2110 | X | - | X | - | refer[223] |  |  |  |  |  |
| IMS-Charging-Identifier | 841 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| IMS-Communication-Service-Identifier | 1281 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| IMS-Information | 876 | X | - | X | - | Grouped | V,M | P |  |  | N |
| IMSI-Unauthenticated-Flag | 2308 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Incoming-Trunk-Group-Id | 852 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Incremental-Cost | 2062 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Interface-Id | 2003 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Interface-Port | 2004 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Interface-Text | 2005 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Interface-Type | 2006 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Inter-Operator-Identifier | 838 | X | - | - | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Dialed-By-MS | 1233 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-External-ID | 1234 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-Id | 1232 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Name | 1231 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-Name | 1235 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Type | 1241 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| LCS-Data-Coding-Scheme | 1236 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Format-Indicator | 1237 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| LCS-Information | 878 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Name-String | 1238 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Requestor-Id | 1239 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Requestor-Id-String | 1240 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Local-Sequence-Number | 2063 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Location-Estimate | 1242 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Location-Estimate-Type | 1243 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Location-Type | 1244 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Low-Balance-Indication | 2020 | - | - | - | X | Enumerated | V,M | P |  |  | N |
| Mandatory-Capability | 604 | X | - | - | - | refer [204] |  |  |  |  |  |
| Max-Requested-Bandwidth-DL | 515 | X | - | X | - | refer [214] |  |  |  |  |  |
| Max-Requested-Bandwidth-UL | 516 | X | - | X | - | refer [214] |  |  |  |  |  |
| MBMS-2G-3G-Indicator | 907 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS GW-Address | 2307 | X | - | - | - | Address | V,M | P |  |  | N |
| MBMS-Information | 880 | X | - | X | - | Grouped | V,M | P |  |  | N |
| MBMS-Service-Area | 903 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-Service-Type | 906 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-Session-Identity | 908 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-User-Service-Type | 1225 | X | - | X | - | Enumerated | V,M | P |  |  | Y |
| Media-Initiator-Flag | 882 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Media-Initiator-Party | 1288 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Message-Body | 889 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Message-Class | 1213 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Message-ID | 1210 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Message-Size | 1212 | - | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Message-Type | 1211 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| MMBox-Storage-Requested | 1248 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| MM-Content-Type | 1203 | - | - | X | - | Grouped | V,M | P |  |  | N |
| MMS-Information | 877 | - | - | X | - | Grouped | V,M | P |  |  | N |
| MMTel-Information | 2030 | X | - | - | - | Grouped | V,M | P |  |  | N |
| MSISDN | 701 | - | - | X | - | refer [221] |  |  |  |  |  |
| Next-Tariff | 2057 |  |  | X | X | Grouped | V/M | P |  |  | N |
| Node-Functionality | 862 | X | -. | X | - | Enumerated | V,M | P |  |  | N |
| Node-Id | 2064 | X | - | - | X | UTF8String | V,M | P |  |  | N |
| Number-Of-Diversions | 2034 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Messages-Sent | 2019 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Messages-Successfully-Exploded | 2111 | X | - | X | - | refer[223] |  |  |  |  |  |
| Number-Of-Messages-Successfully-Sent | 2112 | X | - | X | - | refer[223] |  |  |  |  |  |
| Number-Of-Participants | 885 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Received-Talk-Bursts | 1282 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Number-Of-Talk-Bursts | 1283 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Number-Portability-Routing-Information | 2024 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Offline-Charging | 1278 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Online-Charging-Flag | 2303 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| Optional-Capability | 605 | X | - | - | - | refer [204] |  |  |  |  |  |
| Originating-IOI | 839 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Originator-SCCP-Address | 2008 | - | - | X | - | Address | V,M | P |  |  | N |
| Originator | 864 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Originator-Address | 886 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Originator-Received-Address | 2027 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Originator-Interface | 2009 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Outgoing-Trunk-Group-Id | 853 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Participant-Access-Priority | 1259 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Participant-Action-Type | 2049 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| Participant-Group | 1260 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Participants-Involved | 887 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PDG-Address | 895 | X | - | X | - | Address | V,M | P |  |  | N |
| PDG-Charging-Id | 896 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| PDN-Connection-ID | 2050 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| PDP-Address | 1227 | X | - | X | - | Address | V,M | P |  |  | Y |
| PDP-Context-Type | 1247 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Change-Condition | 1261 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| PoC-Change-Time | 1262 | X | - | - | - | Time | V,M | P |  |  | N |
| PoC-Controlling-Address | 858 | X | - | X |  | UTF8String | V,M | P |  |  | N |
| PoC-Event-Type | 2025 | X | - | X |  | Enumerated | V,M | P |  |  | N |
| PoC-Group-Name | 859 | X | - | X |  | UTF8String | V,M | P |  |  | N |
| PoC-Information | 879 | X | - | X | - | Grouped | V,M | P |  |  | N |
| PoC-Server-Role | 883 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Session-Id | 1229 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PoC-Session-Initiation-type | 1277 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Session-Type | 884 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-User-Role | 1252 | X | - | X | - | Grouped | V,M | P |  |  | N |
| PoC-User-Role-IDs | 1253 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PoC-User-Role-info-Units | 1254 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Positioning-Data | 1245 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Priority | 1209 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Priority-Level | 1046 | X | - | X | - | refer [215] |  |  |  |  |  |
| PS-Append-Free-Format-Data | 867 | X | - | - | X | Enumerated | V,M | P |  |  | N |
| PS-Free-Format-Data | 866 | X | - | - | X | OctetString | V,M | P |  |  | N |
| PS-Furnish-Charging-Information | 865 | X | - | - | X | Grouped | V,M | P |  |  | N |
| PS-Information | 874 | X | - | X | X | Grouped | V,M | P |  |  | N |
| QoS-Information | 1016 | X | - | - | - | refer [215] |  |  |  |  |  |
| QoS-Class-Identifier | 1028 | X | - | X | - | refer [215] |  |  |  |  |  |
| Quota-Consumption-Time | 881 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Quota-Holding-Time | 871 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| RAI | 909 | X | - | X | - | refer [207] |  |  |  |  |  |
| Rate-Element | 2058 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Read-Reply-Report-Requested | 1222 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Real-Time-Tariff-Information | 2305 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Received-Talk-Burst-Time | 1284 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Received-Talk-Burst-Volume | 1285 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Recipient-Address | 1201 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Recipient-Info | 2026 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Recipient-Received-Address | 2028 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Recipient-SCCP-Address | 2010 | - | - | X | - | Address | V,M | P |  |  | N |
| Refund-Information | 2022 | - | - | X | X | OctetString | V,M | P |  |  | N |
| Remaining-Balance | 2021 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Reply-Applic-ID | 1223 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Reply-Path-Requested | 2011 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Reporting-Reason | 872 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Requested-Party-Address | 1251 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Required-MBMS-Bearer-Capabilities | 901 | X | - | X | - | refer [207] |  |  |  |  |  |
| Role-of-Node | 829 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Scale-Factor | 2059 | - | - | X | X | Grouped | V,M | P |  |  | N |
| SDP-Answer-Timestamp | 1275 | X | - | - | - | Time | V,M | P |  |  | N |
| SDP-Media-Component | 843 | X | - | X | - | Grouped | V,M | P |  |  | N |
| SDP-Media-Description | 845 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-Media-Name | 844 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-Offer-Timestamp | 1274 | X | - | - | - | Time | V,M | P |  |  | N |
| SDP-Session-Description | 842 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-TimeStamps | 1273 | X | - | - | - | Grouped | V,M | P |  |  | N |
| SDP-Type | 2036 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Served-Party-IP-Address | 848 | X | - | - | - | Address | V,M | P |  |  | N |
| Server-Capabilities | 603 | X | - | - | - | refer [204] |  |  |  |  |  |
| Server-Name | 602 | X | - | - | - | refer [204] |  |  |  |  |  |
| Service-Data-Container | 2040 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Service-Generic-Information | 1256 | X | - | X | - | Refer[223] |  |  |  |  |  |
| Service-Id | 855 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Service-Information | 873 | X | - | X | X | Grouped | V,M | P |  |  | N |
| Service-Mode | 2032 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Service-Specific-Data | 863 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Service-Specific-Info | 1249 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Service-Specific-Type | 1257 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Serving-Node-Type | 2047 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Service-Type | 2031 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| SGSN-Address | 1228 | X | - | X | - | Address | V,M | P |  |  | N |
| SGW-Change | 2065 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| SIP-Method | 824 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SIP-Request-Timestamp-Fraction | 2301 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| SIP-Request-Timestamp | 834 | X | - | X | - | Time | V,M | P |  |  | N |
| SIP-Response-Timestamp-Fraction | 2302 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| SIP-Response-Timestamp | 835 | X | - | X | - | Time | V,M | P |  |  | N |
| SM-Discharge-Time | 2012 | - | - | X | - | Time | V,M | P |  |  | N |
| SM-Message-Type | 2007 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| SM-Protocol-ID | 2013 | - | - | X | - | OctetString | V,M | P |  |  | N |
| SMSC-Address | 2017 | - | - | X | - | Address | V,M | P |  |  | N |
| SMS-Information | 2000 | - | - | X | - | Grouped | V,M | P |  |  | N |
| SMS-Node | 2016 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| SM-Service-Type | 2029 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| SM-Status | 2014 | - | - | X | - | OctetString | V,M | P |  |  | N |
| SM-User-Data-Header | 2015 | - | - | X | - | OctetString | V,M | P |  |  | N |
| Start-Time | 2041 | X | - | - | - | Time | V,M | P |  |  | N |
| Stop-Time | 2042 | X | - | - | - | Time | V,M | P |  |  | N |
| Submission-Time | 1202 | - | - | X | - | Time | V,M | P |  |  | N |
| Subscriber-Role | 2033 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| Supplementary-Service | 2048 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Talk-Burst-Exchange | 1255 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Talk-Burst-Time | 1286 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Talk-Burst-Volume | 1287 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Tariff-Information | 2060 | X | - | X | X | Grouped | V,M | P |  |  | N |
| Tariff-XML | 2306 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Terminal-Information | 1401 | X | - | X | - | refer [219] |  |  |  |  |  |
| Terminating-IOI | 840 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Time-First-Usage | 2043 | X | - | - | - | Time | V,M | P |  |  | N |
| Time-Last-Usage | 2044 | X | - | - | - | Time | V,M | P |  |  | N |
| Time-Quota-Mechanism | 1270 | - | - | - | X | Grouped | V,M | P |  |  | N |
| Time-Quota-Threshold | 868 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Time-Quota-Type | 1271 | - | - | - | X | Enumerated | V,M | P |  |  | N |
| Time-Stamps | 833 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Time-Usage | 2045 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| TMGI | 900 | X | - | X | - | refer [207] |  |  |  |  |  |
| Token-Text | 1215 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Total-Number-Of-Messages-Exploded | 2113 | X | - | X | - | refer[223] |  |  |  |  |  |
| Total-Number-Of-Messages-Sent | 2114 | X | - | X | - | refer[223] |  |  |  |  |  |
| Traffic-Data-Volumes | 2046 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Trigger | 1264 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Trigger-Type | 870 | - | - | X | X | Enumerated | V,M | P |  |  | N |
| Trunk-Group-Id | 851 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Type-Number | 1204 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Unit-Cost | 2061 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Unit-Quota-Threshold | 1226 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| User-CSG-Information | 2319 | X | - | X | X | Grouped | V,M | P |  |  | N |
| User-Data | 606 | X | - | - | - | refer [204] | V,M | P |  |  | N |
| User-Participating-Type | 1279 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| User–Session-Id | 830 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| VAS-Id | 1102 | - | - | X | - | refer [213] |  |  |  |  |  |
| VASP-Id | 1101 | - | - | X | - | refer [213] |  |  |  |  |  |
| Volume-Quota-Threshold | 869 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| WAG-Address | 890 | X | - | X | - | Address | V,M | P |  |  | N |
| WAG-PLMN-Id | 891 | X | - | X | - | OctetString | V,M | P |  |  | N |
| WLAN-Information | 875 | X | - | X | - | Grouped | V,M | P |  |  | N |
| WLAN-Radio-Container | 892 | X | - | X | - | Grouped | V,M | P |  |  | N |
| WLAN-Session-Id | 1246 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| WLAN-Technology | 893 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| WLAN-UE-Local-IPAddress | 894 | X | - | X | - | Address | V,M | P |  |  | N |

Editor’s Note : Fields setting  for  MMTel charging for CCR is ffs

Next Modified Section

### 7.2.25A	Change-Condition AVP

The Change-Condition AVP (AVP code 2037) is of type Integer32,  and indicates the change in charging condition: (Qos change, tariff time change …) which causes:

-	sending of  Accounting-request from PCN node

-	volume counts container closing  for an IP-CAN bearer.

-	service data container closing.

The following  values are defined :

"Normal Release"									0

The "Normal Release" value is used to indicate IP-CAN session termination , IP-CAN bearer release or Service Data Flow Termination

" Abnormal Release "								1

" Qos Change "										2

“Volume Limit”										3

“Time Limit											4

“Serving Node Change”								5

“Serving Node PLMN Change” 					6

“User Location Change” 							7

“RAT Change”										8

“UE TimeZone Change”							9

“Tariff Time Change” 							      10

“Service Idled Out”									11

“serviceSpecificUnitLimit”							12

“Max Number of Changes in Charging conditions” 13

“Management Intervention”							20

“User CSG Information Change”						22

Next Modified Section

### 7.2.xx		CSG-Access-Mode AVP

The CSG-Access-Mode AVP (AVP code 2317) is of type Enumerated and holds the mode in which the CSG cell User is accessing to, operates.

It has the following values:

0	Closed mode

1	Hybrid Mode

### 7.2.xx		CSG-Membership-Indication AVP

The CSG-Membership-Indication AVP (AVP code 2318) is of type Enumerated, and indicates the UE is a member of the accessing CSG cell, if the access mode is Hybrid, as described in TS 29.060 [xx], and in TS 29.274 [xx]. If this indication is not present, this means the UE is a Not member of the CSG cell for hybrid access mode.

The following values are defined:

0	Not CSG member

1	CSG Member

Next Modified Section

### 7.2.124	PS-Information AVP

The PS-Information AVP (AVP code 874) is of type Grouped. Its purpose is to allow the transmission of additional PS service specific information elements.

It has the following ABNF grammar:

PS-Information :: = 	< AVP Header: 874>

[ 3GPP-Charging-Id ]

[ PDN-Connection-ID ]
   [Node-Id]
   [ 3GPP-PDP-Type ]
   [ PDP-Address ]
   [ Dynamic-Address-Flag ]
   [QoS-Information]
   [ SGSN-Address ]
   [ GGSN-Address ]
   [ CG-Address ]
   [ Serving-Node-Type ]
   [ SGW-Change ]
   [ 3GPP-IMSI-MCC-MNC ]
   [ IMSI-Unauthenticated-Flag ]
   [ 3GPP-GGSN- MCC-MNC ]
   [ 3GPP-NSAPI ]
   [ Called-Station-Id ]
   [ 3GPP-Session-Stop-Indicator ]
   [ 3GPP-Selection-Mode ]
   [ 3GPP-Charging-Characteristics ]
   [ 3GPP-SGSN-MCC-MNC ]
   [ 3GPP-MS-TimeZone ]

* [ Charging-Rule-Base-Name ] 
   [ 3GPP-User-Location-Info ]

[ User-CSG-Information ]

[ 3GPP2-BSID ]
   [ 3GPP-RAT-Type ]
   [ PS-Furnish-Charging-Information ]
   [ PDP-Context-Type ] 
   [ Offline-Charging ]

* [ Traffic-Data-Volumes ]

* [ Service-Data-Container ]

[ User-Equipment-Info ]

[ Terminal-Information ]

[ Start-Time ]

[ Stop-Time ]

[ Change-Condition ]

[ Diagnostics ]

Editor’s Note: PS-information field content is to be further refined.

Next Modified Section

### 7.2.176	Trigger-Type AVP

The Trigger-Type AVP (AVP code 870) is of type Enumerated and indicates a single re-authorisation event type.

When included in the Credit Control Answer command, the Trigger-Type AVP indicates the events that shall cause the credit control client to re-authorise the associated quota. The client shall not re-authorise the quota when events which are not included in the Trigger AVP occur.

When included in the Credit Control Request command indicates the specific event which caused the re-authorisation request of the Reporting-Reason with value RATING_CONDITION_CHANGE associated.

It has the following values:

CHANGE_IN_SGSN_IP_ADDRESS (1)

- This value is used to indicate that a change in the SGSN IP address shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGE_IN_QOS (2)

- This value is used to indicate that a change in the end user  negotiated QoS shall cause the credit control client to ask for a re-authorisation of the associated quota.

NOTE 1:	This should not be used in conjunction with enumerated values 10 to 23.

CHANGE_IN_LOCATION (3)

- This value is used to indicate that a change in the end user location shall cause the credit control client to ask for a re-authorisation of the associated quota.

NOTE 2:	This should not be used in conjunction with enumerated values 30 to 34.

CHANGE_IN_RAT (4)

- This value is used to indicate that a change in the radio access technology shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_TRAFFIC_CLASS (10)

- This value is used to indicate that a change in the end user negotiated traffic class shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_RELIABILITY_CLASS (11)

- This value is used to indicate that a change in the end user negotiated reliability class shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_DELAY_CLASS (12)

- This value is used to indicate that a change in the end user negotiated delay class shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_PEAK_THROUGHPUT (13)

- This value is used to indicate that a change in the end user negotiated peak throughput shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_PRECEDENCE_CLASS (14)

- This value is used to indicate that a change in the end user negotiated precedence class shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_MEAN_THROUGHPUT (15)

- This value is used to indicate that a change in the end user negotiated mean throughput shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_MAXIMUM_BIT_RATE_FOR_UPLINK (16)

- This value is used to indicate that a change in the end user negotiated uplink maximum bit rate shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_MAXIMUM_BIT_RATE_FOR_DOWNLINK (17)

- This value is used to indicate that a change in the end user negotiated downlink maximum bit rate shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_RESIDUAL_BER (18)

- This value is used to indicate that a change in the end user negotiated residual BER shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_SDU_ERROR_RATIO (19)

- This value is used to indicate that a change in the end user negotiated SDU error ratio shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_TRANSFER_DELAY (20)

- This value is used to indicate that a change in the end user negotiated transfer delay shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_TRAFFIC_HANDLING_PRIORITY (21)

- This value is used to indicate that a change in the end user negotiated traffic handling priority shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_GUARANTEED_BIT_RATE_FOR_UPLINK (22)

- This value is used to indicate that a change in the end user negotiated uplink guaranteed bit rate shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINQOS_GUARANTEED_BIT_RATE_FOR_DOWNLINK (23)

- This value is used to indicate that a change in the end user negotiated downlink guaranteed bit rate shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINLOCATION_MCC (30)

- This value is used to indicate that a change in the MCC of the serving network shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINLOCATION_MNC (31)

- This value is used to indicate that a change in the MNC of the serving network shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINLOCATION_RAC (32)

- This value is used to indicate that a change in the RAC where the end user is located shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINLOCATION_LAC (33)

- This value is used to indicate that a change in the LAC where the end user is located shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGEINLOCATION_CellId (34)

- This value is used to indicate that a change in the Cell Identity where the end user is located shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGE_IN_MEDIA_COMPOSITION (40)

- This value is used to indicate that a change in the media composition (as identified within SDP) for an existing SIP session shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGE_IN_PARTICIPANTS_NMB (50)

- This value is used specifically for multi participating session to indicate that a change in the number of active participants within a session shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGE_IN_ THRSHLD_OF_PARTICIPANTS_NMB(51)

- This value is used specifically to indicate that a change in the threshold of participants number within a session shall cause the credit control client to ask for a re-authorisation of the associated quota.

NOTE 3:	The threshold and the granularity of threshold are operator configurable. This should not be used in conjunction with value 50.

CHANGE_IN_USER_PARTICIPATING_TYPE(52)

- This value is used specifically to indicate that a change in the user participating type within a session shall cause the credit control client to ask for a re-authorisation of the associated quota.

CHANGE_IN_SERVICE_CONDITION(60)

- This value is used to indicate that a change in rating conditions associated with a service occurs. 
The description of the conditions causing a change are service specific and may be documented in middle-tier specifications or may be configurable.

- CHANGE_IN_SERVING_NODE

- This value is used to indicate that a change in serving node shall cause the credit control client to ask for a re-authorisation of the associated quota.

- CHANGE_IN_USER_CSG_INFORMATION (70)

- This value is used to indicate that a change in “User CSG Information” (CSG membership in hybrid access mode has changed, or user entering/leaving a CSG cell) for the served user  shall cause the credit control client to ask for a re-authorisation of the associated quota.

Editor’s note: potentially add new values depending on introduction of QCI specifics.

Next Modified Section

### 7.2.xx	User-CSG-Information AVP

The User-CSG-Information AVP (AVP code 2319) is of type Grouped and holds the user “Closed Subscriber Group” Information associated to CSG cell access: it comprises CSG ID within the PLMN, Access mode and indication on CSG membership for the user when hybrid access applies, as defined in TS 29.060 [xx] for GPRS case, and in TS 29.274 [xx] for EPC case.

It has the following ABNF grammar:

User-CSG-Information :: = 		< AVP Header: 2319>

{ CSG-Id }

{ CSG-Access-mode }

[ CSG-Membership-Indication ]

End of modifications