3GPP TSG-SA5 (Telecom Management)	S5-094307

Meeting SA5#68, 09 -13 Nov 2009, Shanghai, PRC	revision of S5-094178

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.275 | CR | 0025 |  rev | - |  Current version: | 9.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add Messages description for MMTel Online Charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | eMMTel-SS-CH |  |  |  |  |  |  | Date:  |  | 13/11/2009 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | Messages description for MMTel Online Charging is missing |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add Messages description for MMTel Online Charging |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Not possible to apply online charging for MMTel service |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.3, 5.3.1, 6.1.1, 6.2, 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 5.3	MMTel online charging scenarios

Next Modified Section

### 5.3.1	Basic principles

Online charging of MMTel services is done according to the general principles of Diameter Credit-Control Applications (DCCA) as specified in TS 32.299 [50].

The CTFs implementing the MMTel online charging functionality may apply:

- Immediate Event Charging (IEC) with CCR [event] generated, or

- Event Charging with Unit Reservation (ECUR), or Session Charging with Unit Reservation (SCUR), both with CCR [Initial or Termination] generated.

The circumstances on which IEC, ECUR or SCUR are applied, depend on the MMTel supplementary service and/or operator's policy. Further details are specified in clause 5.3.2.

Next Modified Section

### 6.1.1	Rf message contents

For offline charging, the different service CTFs supporting MMtel specific service charging generate accounting information transferred to the CDF using the Diameter accounting application, as described in the TS 32.299 [50].

The Charging Data Request and Charging Data Response are used for this MMTel service offline charging, as described in TS 32.260 [20].

Next Modified Section

## 6.2	Data description for MMTel online charging

Next Modified Section

6.2.1	Ro message contents

For online charging, the different service CTFs supporting MMTel specific service charging, generate debit and reserve units information transferred to the OCF. For this purpose, MMTel online charging utilises the Debit Units and Reserve Units procedure specified in the Debit / Reserve Units operation in TS 32.299 [50].

The Debit / Reserve Units Request and Debit / Reserve Units Response messages are used for this MMTel service online charging, as described in TS 32.260 [20].

#### 6.2.1.1 	Debit and Reserve Units Request Message

The generic Debit / Reserve Units Request message described in TS 32.260[20], used for MMTel online charging is enhanced with specific MMTel service information.

The following table illustrates the overall Debit / Reserve Units Request message as used for MMTel online charging.

Table 6.2.1.1: Debit and Reserve units Request Message Contents

| Field | Category | Description |
| --- | --- | --- |
| NOTE 1 |  |  |
| Multiple Unit Operation | OC | This field contains all parameters for the quota management. |
| Service Identifier | OC | This field contains identity of MMTel service. It holds the “Service Type” parameter (described in table 6.3.1.2 ) value. |
| NOTE 1 |  |  |
| Service Information | OM | This field holds the MMTel specific parameter and is described in clause 6.3. |

NOTE 1:  	See Debit / Reserve Units Request message fields described in TS 32.260 [20].

NOTE 2:	Detailed descriptions of the fields are provided in TS 32.299 [50].

#### 6.2.1.2	Debit and Reserve Units Response Message

The generic Debit / Reserve Units Response message described in TS 32.260[20] is used for MMTel online charging.

End of modifications