3GPP TS 32.712 V10.0.0 (2010-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Configuration Management (CM);

Transport Network (TN) interface Network Resource Model (NRM) Integration Reference Point (IRP);

Information Service (IS)

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

©2009, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Information Object Classes	7

4.1	Imported information entities and local labels	7

4.2	Class diagram	7

4.2.1	Attributes and relationships	7

4.2.2	Inheritance	9

4.3	Information object class definitions	9

4.3.1	TransportNetworkInterface	9

4.3.1.1	Definition	9

4.3.1.2	Attributes	9

4.3.2	ATMChannelTerminationPoint	10

4.3.2.1	Definition	10

4.3.2.2	Attributes	10

4.3.2.3	Attribute constraints	11

4.3.3	ATMPathTerminationPoint	12

4.3.3.1	Definition	12

4.3.3.2	Attributes	12

4.3.3.3	Attribute constraints	12

4.4	Information relationship definitions	13

4.4.1	AssociatedWith (M)	13

4.4.1.1	Definition	13

4.4.1.2	Roles	13

4.4.1.3	Constraints	13

4.5	Information attributes definition	14

4.5.1	Definition and legal values	14

4.5.2	Constraints	15

4.6	Particular information configurations	15

Annex A (informative):	 Example Configuration of ATM Transport Network in UTRAN	16

Annex B (informative):	Change history	17

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.711:	Configuration Management (CM); Transport Network (TN) interface Network Resource Model (NRM) Integration Reference Point (IRP); Requirements

32.712:	Configuration Management (CM); Transport Network (TN) interface Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

32.713:	Configuration Management (CM); Transport Network (TN) interface Network Resource Model (NRM) Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)

32.715	Configuration Management (CM); Transport Network (TN) interface Network Resource Model (NRM) Integration Reference Point (IRP); eXtensible Markup Language (XML) file format definition

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

CM actions may be requested as part of an implementation programme (e.g. additions and deletions), as part of an optimisation programme (e.g. modifications), and to maintain the overall Quality of Service (QoS). The CM actions are initiated either as single actions on single NEs of the 3G network, or as part of a complex procedure involving actions on many resources/objects in one or several NEs.

# 1	Scope

The present document is part of an Integration Reference Point (IRP) named "Transport Network (TN) Interface Network Resource Model (NRM) IRP ", through which an "IRPAgent" (typically an Element Manager or Network Element) can communicate Configuration Management information to one or several "IRPManagers" (typically Network Managers) concerning Transport resources. The "Transport Network (TN) Interface Network Resource Model (NRM) IRP" comprises a set of specifications defining Requirements, a protocol neutral Network Resource Model (NRM) and corresponding Solution Set(s).

The present document:

- specifies the protocol neutral Transport Network Interface Resources IRP: Network Resource Model. It reuses relevant parts of the generic NRM in TS 32.622 [6], either by direct reuse or sub-classing, and in addition to that defines Transport specific Managed Object Classes.

The Configuration Management (CM) area is very large. The intention is to split the specification of the related interfaces in several IRPs - as described in the Introduction clause above. An important aspect of such a split is that the Network Resource Models (NRMs) defined in different IRPs containing NRMs are consistent, and that NRMs supported by an IRPAgent implementation can be accessed as one coherent model through one IRP Information Service.

In order to access the information defined by this NRM, an IRP Information Service (IS) is needed, such as the Basic CM IRP: IS (TS 32.602 [7]) or the Bulk CM IRP: IS (TS 32.612 [8]). However, which Information Service that is applicable is outside the scope of this document.

Finally, regarding the support of the State Management IRP: IS (TS 32.672 [3]), all NRM's of one release shall support the same State Management IRP version.

This specification is related to 3GPP TS 32.672.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication Management, Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.672: "Telecommunication management; Configuration Management (CM); State Management Integration Reference Point (IRP): Information Service (IS)".

[4]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[5]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[6]	3GPP TS 32.622: " Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[7]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP) Information Service (IS)".

[8]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Information Service (IS)".

[9]	3GPP TS 25.430: "UTRAN Iub interface:general aspects and principles".

[10]	3GPP TS 25.431:  "UTRAN Iub interface Layer 1".

[11]	3GPP TS 25.411: "UTRAN Iu interface Layer 1".

[12]	ITU-T Recommendation I.361 (11/95):"B-ISDN ATM Layer Specification".

# 3	Definitions and abbreviations

## 3.1	Definitions

- For the purposes of the present document, the terms and definitions given in 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.150 [3] and the following apply:

Association: in general it is used to model relationships between Managed Objects. Associations can be implemented in several ways, such as:

1.	name bindings;

2.	reference attributes; and

3.	association objects.

This IRP stipulates that containment associations shall be expressed through name bindings, but it does not stipulate the implementation for other types of associations as a general rule. These are specified as separate entities in the object models (UML diagrams)

Managed Element (ME): an instance of the Managed Object Class ManagedElement defined in 3GPP TS 32.622 [6].

Managed Object (MO): in the context of the present document, a Managed Object (MO) is a software object that encapsulates the manageable characteristics and behaviour of a particular Network Resource. The MO is instance of a MO class defined in a MIM/NRM. This class, called Information Object Class (IOC) has attributes that provide information used to characterize the objects that belong to the class (the term "attribute" is taken from TMN and corresponds to a "property" according to CIM). Furthermore, the IOC can have operations that represent the behaviour relevant for that class (the term "operation" is taken from TMN and corresponds to a "method" according to CIM). The IOC may support the emission of notifications that provide information about an event occurrence within a network resource

Management Information Model (MIM): also referred to as NRM - see the definition below

Network Resource Model (NRM): A model representing the actual managed telecommunications network resources that a System is providing through the subject IRP. An NRM identifies and describes the IOCs, their associations, attributes and operations. The NRM is also referred to as "MIM" (see above), which originates from the ITU-T TMN

Node B: A logical node responsible for radio transmission/reception in one or more cells to/from the User Equipment. It terminates the Iub interface towards the RNC

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.150 [3] and the following apply:

CIM	Common Information Model

DN	Distinguished Name (see 3GPP TS 32.300 [4])

EM	Element Manager

FM	Fault Management

IOC	Information Object Class

IRP	Integration Reference Point

ITU-T	International Telecommunication Union, Telecommunication Sector

Iub	Interface between RNC and Node B

ME	Managed Element

MIM	Management Information Model

MO	Managed Object

NE	Network Element

NM	Network Manager

NR	Network Resource

NRM	Network Resource Model

PM	Performance Management

RDN	Relative Distinguished Name (see 3GPP TS 32.300 [4])

RNC	Radio Network Controller

TMN	Telecommunications Management Network

UML	Unified Modelling Language

UMTS	Universal Mobile Telecommunications System

# 4	Information Object Classes

## 4.1	Imported information entities and local labels

IOCs ManagedElement,  IubLink,  and vsDataComtainer are imported.

## 4.2	Class diagram

### 4.2.1	Attributes and relationships

This sub clause depicts the set of IOCs that encapsulate information relevant for this service. This sub clause provides the overview of all information object classes in UML. Subsequent subclasses provide more detailed specification of various aspects of these information object classes.

Figure 4.2.1.1 shows the name-containment relation and other types of relations of the Transport Network NRM.

NOTE:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios.

Figure 4.2.1.1: Transport Network NRM Containment/Naming and Association diagram

Each IOC is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [4] that expresses its containment hierarchy. As an example, the DN of a IOC representing a ATMPathTerminationPoint could have a format like:

SubNetwork=Sweden,meContext=MEC-Gbg-1,ManagedElement=RNC-Gbg-1, TransportNetworkInterface=ATM-1, ATMPathTerminationPoint=Gbg-1.

NOTE 1:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios.

NOTE 2:	Each instance of the vsDataContainer shall only be contained under one IOC. The vsDataContainer can be contained under IOCs defined in other NRMs.

Figure 4.2.1.2: vsDataContainer in Transport Network Containment/Naming and Association diagram

The vsDataContainer is only used for the Bulk CM IRP.

### 4.2.2	Inheritance

This sub-clause depicts the inheritance relationships that exist between IOCs.

Figure 4.2.2.1 shows the inheritance hierarchy for the Transport Network NRM.

Figure 4.2.2.1: Transport Network NRM Inheritance Hierarchy

## 4.3	Information object class definitions

### 4.3.1	TransportNetworkInterface

#### 4.3.1.1	Definition

This IOC represents the Transport Network Interface technology (e.g. ATM, IP).

#### 4.3.1.2	Attributes

Table 4.3.1: Attributes of TransportNetworkInterface

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| transportNetworkInterfaceId | + | M | M | - |
| userLabel | + | M | M | M |
| transportNetworkType | + | M | M | - |

### 4.3.2	ATMChannelTerminationPoint

#### 4.3.2.1	Definition

This IOC represents a bi-directional ATM Virtual Channel Connection Termination Point.

#### 4.3.2.2	Attributes

Table 4.3.2: Attributes of ATMChannelTerminationPoint

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| aTMChannelTerminationPointid | + | M | M | - |
| usageChannel | + | M | M | - |
| virtualPathId | + | M | M | O |
| virtualChannelId | + | M | M | O |
| physicalPortId | + | M | M | O |
| physicalInterfaceType | + | M | M | O |
| serviceCategoryIn | + | M | M | O |
| ServiceCategoryEg | + | M | M | O |
| usedAAL | + | M | M | O |
| peakCellRateIn | + | M | M | O |
| peakCellRateEg | + | M | M | O |
| sustainableCellRateIn | + | O | M | O |
| sustainableCellRateEg | + | O | M | O |
| maximumBurstSizeIn | + | M | M | O |
| maximumBurstSizeEg | + | M | M | O |
| minimumDesiredCellRateIn | + | O | M | O |
| minimumDesiredCellRateEg | + | O | M | O |
| minimumCellRateIn | + | O | M | O |
| minimumCellRateEg | + | O | M | O |
| aTMChannelTerminationPoint- ATMPathTerminationPoint | + | M | M | - |
| aTMChannelTerminationPoint- IubLink | + | M | M | - |

Table 4.3.2: Attributes of ATMChannelTerminationPoint

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| aTMChannelTerminationPointid | + | M | M | - |
| usageChannel | + | M | M | - |
| virtualPathId | + | M | M | O |
| virtualChannelId | + | M | M | O |
| physicalPortId | + | M | M | O |
| physicalInterfaceType | + | M | M | O |
| serviceCategoryIn | + | M | M | O |
| serviceCategoryEg | + | M | M | O |
| usedAAL | + | M | M | O |
| peakCellRateIn | + | M | M | O |
| peakCellRateEg | + | M | M | O |
| sustainableCellRateIn | + | O | M | O |
| sustainableCellRateEg | + | O | M | O |
| maximumBurstSizeIn | + | M | M | O |
| maximumBurstSizeEg | + | M | M | O |
| minimumDesiredCellRateIn | + | O | M | O |
| minimumDesiredCellRateEg | + | O | M | O |
| minimumCellRateIn | + | O | M | O |
| minimumCellRateEg | + | O | M | O |
| aTMChannelTerminationPoint-ATMPathTerminationPoint | + | M | M | - |
| aTMChannelTerminationPoint-IubLink | + | M | M | - |

#### 4.3.2.3	Attribute constraints

The Write Qualifier for attributes virtualPathId, virtualChannelId, physicalPortId, physicalInterfaceType, serviceCategoryIn/Eg, usedAAL, peakCellRateIn/Eg, sustainableCellRateIn/Eg, and maximumBurstSizeIn/Eg shall be Mandatory if these attributes can be set over Itf-N.

The attributes sustainableCellRateIn/Eg and maximumBurstSizeIn/Eg are only applicable for ServiceCategory values RT-VBR, NRT-VBR.

The attributes minimumDesiredCellRateIn/Eg is only applicable for Service Category UBR.

The attributes minimumCellRateIn/Eg is only applicable for Service Category values ABR, GFR.

### 4.3.3	ATMPathTerminationPoint

#### 4.3.3.1	Definition

This IOC represents a bi-directional ATM Virtual Path Connection Termination Point.

#### 4.3.3.2	Attributes

Table 4.3.3: Attributes of ATMPathTerminationPoint

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| aTMPathTerminationPointid | + | M | M | - |
| virtualPathId | + | M | M | O |
| physicalPortIdList | + | M | M | O |
| peakCellRateIn | + | M | M | O |
| peakCellRateEg | + | M | M | O |
| aTMPathTerminationPoint-ATMChannelTerminationPoint | + | M | M | - |

NOTE:	The attribute peakCellRateIn/Eg of ATM Path is the maximum Peak Cell Rate of its channels.

#### 4.3.3.3	Attribute constraints

The Write Qualifier for attributes virtualPathId, physicalPortIdList, peakCellRateIn/Eg shall be Mandatory if these attributes can be set over Itf-N.

## 4.4	Information relationship definitions

### 4.4.1	AssociatedWith (M)

#### 4.4.1.1	Definition

This represents a bi-directional relation between the ATMPathTerminationPoint and ATMChannelTerminationPoint.  The roles of the relation shall be mapped to a reference attribute of the IOCs. The name of the reference attribute shall be the role name.

#### 4.4.1.2	Roles

Table 4.4.1: Roles of the relation AssociatedWith

| Name | Definition |
| --- | --- |
| aTMPathTerminationPoint-ATMChannelTerminationPoint | This role (when present) represents aTMPathTerminationPoint capability to identify the set of related ATMChannelTerminationPoint. ATMPathTerminationPoint-ATMChannelTerminationPoint shall carry the set of ATMChannelTerminationPoint DN(s). |
| aTMChannelTerminationPoint-ATMPathTerminationPoint | This role (when present) represents ATMChannelTerminationPoint capability to identify one related ATMPathTerminationPoint. When the role is absent, the ATMChannelTerminationPoint- ATMPathTerminationPoint shall contain no information. When it is present, it shall contain one ATMPathTerminationPoint DN. |

#### 4.4.1.3	Constraints

None.

## 4.5	Information attributes definition

### 4.5.1	Definition and legal values

Table 4.5.1 defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

Table 4.5.1: Attributes

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| transportNetworkInterfaceId | An attribute whose "name+value" can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance |  |
| transportNetworkType | The type of underlying transport network, i.e. ATM, IP | Type: Enumerated Range: ATM, IP |
| aTMChannelTerminationPointId | An attribute whose "name+value" can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance |  |
| aTMPathTerminationPointId | An attribute whose "name+value" can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance |  |
| UsageChannel | The logical channel using the transport network connection. Ref. 3GPP TS 25.430 [9] | Type: String e.g. Iub-NBAP, Iub-ALCAP |
| virtualPathId | The ATM Virtual Path Identifier (VPI). Ref. ITU-T Recommendation I.361[12] | Type: Integral numeric value |
| virtualChannelId | The ATM Virtual Channel Identifier (VCI). Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value |
| physicalPortidList | The list of identifiers of the ATM physical port containing termination points | Type: String |
| physicalPortid | The identifier of the ATM physical port containing termination points | Type: String |
| physicalInterfaceType | The ATM physical interface type. Ref. 3GPP TS 25.431[10], 3GPP TS 25.411[11] | Type: String e.g. E1, STM1 |
| serviceCategoryIn | The ATM Service Category used for the virtual connection Ingress (incoming) traffic. Ref. ITU-T Recommendation I.361[12] | Type: Enumerated Range: CBR, RT-VBR, NRT-VBR, ABR, UBR, GFR |
| serviceCategoryEg | The ATM Service Category used for the virtual connection Egress (outgoing) traffic. Ref. ITU-T Recommendation I.361[12] | Type: Enumerated Range: CBR, RT-VBR, NRT-VBR, ABR, UBR, GFR |
| usedAAL | The ATM Adaptation Layer (AAL) used for the virtual connection. Ref. ITU-T Recommendation I.361[12] | Type: Enumerated Range: Null, AAL1,..... AAL5 |
| peakCellRateIn | Peak Cell Rate (PCR) in kbits/sec for Ingress traffic. Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value |
| peakCellRateEg | Peak Cell Rate (PCR) in kbits/sec for Egress traffic. Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value |
| sustainableCellRateIn | Sustainable Cell Rate (SCR) in kbits/sec for Ingress traffic. Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value Range: 1 .....n, Null |
| sustainableCellRateEg | Sustainable Cell Rate (SCR) in kbits/sec for Egress traffic. Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value Range: 1 .....n, Null |
| maximumBurstSizeIn | Maximum Burst Size (MBS) for VBR Service Categories for Ingress traffic. Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value Range: 1 .....n, Null |
| maximumBurstSizeEg | Maximum Burst Size (MBS) for VBR Service Categories for Egress traffic. Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value Range: 1 .....n, Null |
| minimumCellRateIn | Minimum Cell Rate (MCR) in kbits/sec for ABR, GFR Service Categories for Ingress traffic. Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value Range: 1 .....n, Null |
| minimumCellRateEg | Minimum Cell Rate (MCR) in kbits/sec for ABR, GFR Service Categories for Egress traffic. Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value Range: 1 .....n, Null |
| minimumDesiredCellRateIn | Minimum Desired Cell Rate (MDCR) in kbits/sec for UBR Service Category for Ingress traffic. Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value Range: 1 .....n, Null |
| minimumDesiredCellRateEg | Minimum Desired Cell Rate (MDCR) in kbits/sec for UBR Service Category for Egress traffic. Ref. ITU-T Recommendation I.361 [12] | Type: Integral numeric value Range: 1 .....n, Null |
| userLabel | A user-friendly (and user assigned) name of the associated object |  |

### 4.5.2	Constraints

None.

## 4.6	Particular information configurations

Not applicable.

Annex A (informative): 
Example Configuration of ATM Transport Network in UTRAN

Figure A.1:  Virtual connection of a logical Iub interface channel over ATM network

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| Sep 2003 | SA_21 | SP-030429 | -- | -- | Submitted to TSG SA#21 for Information | 1.0.0 |  |
| Sep 2004 | SA_25 | SP-040597 | -- | -- | Submitted to TSG SA#25 for Approval | 2.0.0 | 6.0.0 |
| Jun 2007 | SA_36 | -- | -- | -- | Automatic upgrade to Rel-7 (no CR) at freeze of Rel-7. Deleted reference to CMIP SS, discontinued from R7 onwards. | 6.0.0 | 7.0.0 |
| Sep 2007 | SA_37 | SP-070612 | 0001 | -- | Correct IS Attribute Tables | 7.0.0 | 7.1.0 |
| Dec 2008 | SA_42 | -- | -- | -- | Upgrade to Release 8 | 7.1.0 | 8.0.0 |
| Dec 2009 | - | - | - | - | Update to Rel-9 version | 8.0.0 | 9.0.0 |
| Dec 2010 | SA_50 | SP-100759 | 0002 | -- | Correcting the association between ATMChannelTerminationPoint and IubLink | 9.0.0 | 10.0.0 |
| Dec 2010 | SA_50 | SP-100759 | 0003 | -- | Correcting the name of AssociatedWith1 - Align with 32.642 | 9.0.0 | 10.0.0 |