3GPP TSG-SA5 (Telecom Management)	S5-110428

SA5#75, 24 - 28 Jan 2011, Sorrento, ITALY	revision of S5-100348

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0159 |  rev | 2 |  Current version: | 9.4.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correcting the condition to deliver SGW-CDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Orange, Ericsson, Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH8 |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | Wrong statement introducing Table 6.1.2 on SGW-CDR charging data table. Whether FBC is activated or not is not a condition for the S-GW to deliver IP-CAN bearer charging data in SGW-CDR. The statement is probably inherited from G-CDR description in Rel-7. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Remove the condition on FBC disabled to make available charging data in SGW-CDR |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Wrong condition to deliver SGW-CDR |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

### 6.1.2	IP CAN bearer charging data in S-GW (SGW-CDR)

If the collection of CDR data is enabled then the S-GW data specified in the following table shall be available for each IP CAN bearer.

Table 6.1.2: S-GW IP CAN bearer data (SGW-CDR)

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | S-GW IP CAN bearer record. |
| Served IMSI | C | IMSI of the served party, if available. |
| IMSI Unauthenticated Flag | OC | This field indicates the provided served IMSI is not authenticated (emergency bearer service situation). |
| Served IMEISV | OC | IMEISV of the ME, if available. It is used for identifying the user in case Served IMSI is not present during emergency bearer service. |
| S-GW Address used | M | The control plane IP address of the S-GW used. |
| Charging ID | M | IP CAN bearer identifier used to identify this IP CAN bearer in different records created by PCNs |
| PDN Connection Id | OM | This field holds the PDN connection (IP-CAN session) identifier to identify different records belonging to same PDN connection. |
| Serving Node Address | M | List of serving node control plane IP addresses (e.g. SGSN, MME, …) used during this record. |
| Serving node Type | M | List of serving node types in control plane. The serving node types listed here map to the serving node addresses listed in the field “Serving node Address” in sequence. |
| S-GW Change |  | Present if this is first record after S-GW change. |
| PGW PLMN Identifier |  | PLMN identifier (MCC MNC) of the PGW used. |
| Access Point Name Network Identifier | OM | The logical name of the connected access point to the external packet data network (network identifier part of APN). |
| PDP/PDN Type | OM | This field indicates PDN type (i.e IPv4, IPv6 or IPv4v6). |
| Served PDP/PDN Address | OC | IP address allocated for the PDP context / PDN connection, if available, i.e. IPv4 when PDN Type is IPv4 or IPv6 when PDN Type is IPv6 or IPv4v6. |
| Served PDP/PDN Address extension | OC | This field holds IPv4 address of the served IMSI, if available, when PDN type is IPv4v6. |
| Dynamic Address Flag | OC | Indicates whether served PDP/PDN address is dynamic, which is allocated during IP CAN bearer activation, initial attach (E-UTRAN or over S2x) and UE requested PDN connectivity. This field is missing if address is static. |
| List of Traffic Data Volumes | OM | A list of changes in charging conditions for this QCI/ARP pair, each change is time stamped. Charging conditions are used to categorize traffic volumes, such as per tariff period. Initial and subsequently changed QoS and corresponding data values are also listed. |
| Record Opening Time | M | Time stamp when IP CAN bearer is activated in this S-GW or record opening time on subsequent partial records. |
| MS Time Zone |  | This field contains the MS Time Zone the MS is currently located as defined in TS 29.060 [203], if available. |
| Duration | M | Duration of this record in the S-GW. |
| Cause for Record Closing | M | The reason for the release of record from this S-GW. |
| Diagnostics | OM | A more detailed reason for the release of the connection. |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |
| Node ID | OM | Name of the recording entity. |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| Local Record Sequence Number | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |
| APN Selection Mode | OM | An index indicating how the APN was selected. |
| Served MSISDN | OM | The primary MSISDN of the subscriber. |
| User Location Information | OC | This field contains the User Location Information of the MS as defined in TS 29.060 [203] for GPRS case, and in TS 29.274 [210] for EPC case, if available. |
| User CSG information | OC | This field contains the User CSG information of the UE, if available, including CSG ID, access mode and CSG membership indication. |
| Charging Characteristics | M | The Charging Characteristics applied to the IP CAN bearer. |
| Charging Characteristics Selection Mode | OM | Holds information about how Charging Characteristics were selected. |
| IMS Signalling Context | OC | Included if the IM-CN Subsystem Signalling Flag is set, see [201] IP CAN bearer is used for IMS signalling. |
| P-GW Address used. | OC | This field is the P-GW IP Address for the Control Plane |
| Serving Node PLMN Identifier | OC | Serving node PLMN Identifier (MCC and MNC) used during this record, if available. |
| RAT Type | OC | This field indicates the Radio Access Technology (RAT) type currently used by the Mobile Station, when available. This RAT type is defined in TS 29.060 [204] for GTP case, in TS 29.274 [210] for eGTP case and in TS 29.275 [211] for PMIP case. |
| Start Time |  | This field holds the time when User IP-CAN session starts, available in the CDR for the first bearer in an IP-CAN session. |
| Stop Time |  | This field holds the time when User IP-CAN session is terminated, available in the CDR for the last bearer in an IP-CAN session. |

End of changes