3GPP TSG-SA5 (Telecom Management)	S5-111432

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-110178

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.409 | CR | 0024 |  rev | 1 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Enhance session establishment measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM-PM-KPI_IMS |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Current session establishment related measurements mix the originating and terminating together, which can not support IMS KPI Session Establishment Success Rate. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Enhance Session Establishment measurements by adding split measurements for differentiatiated originating session and terminating sessions. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | IMS KPI Session Establishment Success Rate can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.1.2.x(new), 4.1.2.x+1(new), 4.1.2.x+2(new), 4.1.2.x+3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 4.1.2.x	Attempted originating session establishments

- This measurement provides the number of attempted originating session establishments at P-CSCF.

- CC.

- On receipt by the P-CSCF of an initial SIP_INVITE message from UE (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttSessionOrig.

- PCSCFFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.2.x+1	Successful originating session establishments

- This measurement provides the number of successful originating session establishments at the P-CSCF. The measurement is split into subcounters per message type (e.g. SIP_180) indicating the successful session establishments.

- CC.

- On transmission by the P-CSCF of the first response message (e.g. SIP_180) indicating a successful session establishment corresponding to the initial SIP_INVITE message (see 3GPP TS 23.228 [7]) to UE, each successful session establishment is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful session establishments. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.SuccSessionOrig.type
where type identifies the message type.

The following subcounters with specific message types are the samples of this measurement:

-	SC.SuccSessionOrig.180	Message type: SIP_180;

-	SC.SuccSessionOrig.200	Message type: SIP_200_OK only if the SIP_180 has not been sent for this 	session.

- PCSCFFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.2.x+2	Attempted terminating session establishments

- This measurement provides the number of attempted terminating session establishments at P-CSCF and I-CSCF.

- CC.

- On receipt by the terminating I-CSCF of an initial SIP_INVITE message from originating S-CSCF and receipt by the P-CSCF of an initial SIP_INVITE message from S-CSCF (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttSessionTerm.

- PCSCFFunction

- ICSCFFunction

- Valid for packet switched traffic.

- IMS.

#### 4.1.2.x+3	Successful terminating session establishments

- This measurement provides the number of successful terminating session establishments at the P-CSCF and I-CSCF. The measurement is split into subcounters per message type (e.g. SIP_180) indicating the successful session establishments.

- CC.

- On transmission by the P-CSCF of the first response message (e.g. SIP_180) indicating a successful terminating session establishment corresponding to the initial SIP_INVITE message (see 3GPP TS 23.228 [7]) to S-CSCF or on transmission by the I-CSCF of the first response message (e.g. SIP_180) indicating a successful terminating session establishment corresponding to the initial SIP_INVITE message (see 3GPP TS 23.228 [7]) to originating S-CSCF, each successful session establishment is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful session establishments. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.SuccSessionTerm.type
where type identifies the message type.

The following subcounters with specific message types are the samples of this measurement:

-	SC.SuccSessionTerm.180	Message type: SIP_180;

-	SC.SuccSessionTerm.200	Message type: SIP_200_OK only if the SIP_180 has not been sent for this 	session.

- PCSCFFunction

- ICSCFFunction

- Valid for packet switched traffic.

- IMS.

End of modifications