3GPP TSG-SA5 (Telecom Management)	S5-110385

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-110065

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.442 | CR | 0004 |  rev | - |  Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Adding Minimization of Drive Tests (MDT) to Trace IRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM-PM-UE |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | TraceIRP defines the interface for managing trace over the Itf-N. To complete the management support for MDT the TraceIRP is also enhanced to cover the MDT specific parameters. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | MDT specific parameters are added to the TraceIRP |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.3.1.1, 5.3.1.2, 5.3.1.3, 5.3.4.1, 5.5.1, 6.3.1.2, 6.3.1.3, 6.3.1.7, 6.3.2.2, 6.3.3.3, 6.3.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.446 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

## 5.3	Information object class definitions

### 5.3.1	TraceJob

#### 5.3.1.1	Definition

It represents a task that controls the Trace Sessions and collects the trace data (i.e. collects the TraceRecord of multiple ManagedEntity instances). The TraceReference is a unique ID, which identifies the Trace Session that has been created by the TraceJob and activated to one or multiple ManagedEntity instance(s).

It represents also the task that controls the UE based network performance measurements.

When a TraceJob is created the following attributes cannot be modified via the Itf-N:

- TraceReference

- ListOfInterfaces

- ListofNeTypes

- TraceDepth

- TraceTarget

- TriggeringEvent

- JobType

- ListOfMeasurements

- ReportingTrigger

- ReportInterval

- ReportAmount

- EventThreshold

- LoggingInterval

- LoggingDuration

- IPAddressOfTCE

If for any reason the TraceIRP determines that a Trace Session has been activated in its ManagedEntity(ies) the TraceIRP shall emit the "noitfyTraceSessionLocalActivation"  notification to the subscribed IRPManagers to inform the active Trace Sessions. The IRPManagers can decide whether they deactivate the Trace Session or keep the Trace Session active. (E.g. if the TraceReference is colliding with an existing TraceJob’s TraceReference, the IRPManager may decide to immediately deactivate the Trace Session in that ManagedEntity.)

The TraceJob shall use its information to activate and configure Trace Session(s) in the requested ManagedEntity instance(s). When the TraceIRP determines that there are available TraceRecord files, it shall emit a notification to all subscribed IRPManagers informing the availability of the files. The method and the notification of the available files is described in the File Transfer IRP (3GPP TS 32.342 [7]).

If a TraceJob receives an indication from one of its ManagedEntity that starting a Trace Recording Session is failed for any reason, the "notifyTraceRecordingSessionFailure" notification may be emitted to inform all subscribed IRPManagers that there was a Trace Recording Session that was not started in the ManagedEntity.

#### 5.3.1.2	Attributes

| Attribute name | Support Qualifier |  |  |
| --- | --- | --- | --- |
| traceReference | M |  |  |
| listOfInterfaces | O |  |  |
| listOfNeTypes | CM |  |  |
| traceDepth | CM |  |  |
| traceTarget | M |  |  |
| triggeringEvent | CM |  |  |
| traceCollectionEntityAddress | M |  |  |
| jobType | M |  |  |
| listOfMeasurements | CM |  |  |
| reportingTrigger | CM |  |  |
| reportInterval | CM |  |  |
| reportAmount | CM |  |  |
| eventThreshold | CM |  |  |
| loggingInterval | CM |  |  |
| loggingDuration | CM |  |  |

#### 5.3.1.3	Attribute constraints

The listOfNeTypes attributes shall be present only for Signalling Based Activation.

The traceTarget shall be public ID in case of a Management Based Activation is done to an ScscfFunction. The TraceTarget shall be cell only in case of the UTRAN cell traffic trace function. 
The TraceTarget shall be E-UtranCell only in case of E-UTRAN cell traffic trace function.The traceTarget shall be either IMSI or IMEI(SV) if the Trace Session is activated to any of the following ManagedEntity(ies):

- HssFunction

- MscServerFunction

- SgsnFunction

- GgsnFunction

- BmscFunction

- RncFunction

- MmeFunction

The traceTarget shall be IMSI if the Trace Session is activated to a ManagedEntity playing a role of ServinGWFunction.

In case of MDT, the TraceTarget attribute shall be able to carry (IMSI or IMEI(SV))and a list of (cell or EUtranCell or TA/LA/RA).

- traceTarget: This attribute shall be present only if Trace is supported.

- triggeringEvent: This attribute shall be present only if Trace is supported.

- listOfMeasurements: This attribute shall be present only if MDT is supported and the JobType attribute is set to ImmediateMDT.

- reportingTrigger: This attribute shall be present only if MDT is supported and the JobType attribute is set to ImmediateMDT and the ListOfMeasurements attribute is configured for M1.

- reportInterval: This attribute shall be present only if MDT is supported and the JobType attribute is set to ImmediateMDT and the ReportingTrigger is configured for PeriodicMeasurements

- reportAmount: This attribute shall be present only if MDT is supported and the JobType attribute is set to ImmediateMDT and the ReportingTrigger attribute is configured for PeriodicMeasurements.

- eventThreshold: This attribute shall be present only if MDT is supported and the JobType attribute is set to ImmediateMDT and the ReportTrigger attribute is configured for A2EventReporting.

- loggingInterval: This attribute shall be present only if MDT is supported and the JobType attribute is set to LoggedMDT.

- loggingDuration: This attribute shall be present only if MDT is supported and the JobType attribute is set to LoggedMDT.

- 5.3.2	TraceRecord

### 5.3.2.1	Definition

TraceRecord is the representation of the files containing the logged information from the Trace Recording Sessions.

#### 5.3.2.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| fileName | + | M | M | - |

### 5.3.3	TraceIRP

#### 5.3.3.1	Definition

TraceIRPis the representation of the trace management capabilities specified by the present document. This IOC inherits from ManagedGenericIRP IOC specified in 3GPP TS 32.312 [8].

### 5.3.4	ManagedEntity

#### 5.3.4.1	Definition

In case of Signalling Based Activation the ManagedEntity represents the role that can be played by an instance of one of the following IOCs:

- HssFunction

- MscServerFunction

- SgsnFunction

- MmeFunction

In case of Management Based Activation the ManagedEntity represents the role that can be played by an instance of the following IOCs:

- HssFunction

- MscServerFunction

- SgsnFunction

- GgsnFunction

- BmscFunction

- RncFunction

- CscfFunction

- MmeFunction

- ServingGWFunction

In case of Cell Traffic Trace the ManagedEntity represents the role that can be played by an instance of the of the following IOCs:

- UtranCell

- E-UtranCell

## 5.4	Information relationship definitions

### 5.4.1	relation-traceIRP-traceJob (M)

#### 5.4.1.1	Definition

This represents the relationship between TraceIRP and the  TraceJob.

#### 5.4.1.2	Roles

| Name | Definition |
| --- | --- |
| theTraceIRP | It represents the TraceIRP |
| theTraceJobList | It represents the TraceJobList |

### 5.4.2	relation-traceJob-managedEntity (M)

#### 5.4.2.1	Definition

This represents the relationship between TraceJob and the ManagedEntity.

#### 5.4.2.2	Roles

| Name | Definition |
| --- | --- |
| theManagedEntity | The ManagedEntity, when playing this role, represents the actual network resource instance, where a Trace Session is activated. |
| theTraceJob | It represents the TraceJob |

### 5.4.3	relation-traceJob-traceRecord (M)

#### 5.4.3.1	Definition

This represents the relationship between TraceJob and the  TraceRecord.

#### 5.4.3.2	Roles

| Name | Definition |
| --- | --- |
| theTraceJob | It represents the TraceJob |
| theTraceRecord | It represnts the TraceRecord. |

## 5.5	Information attribute definitions

### 5.5.1	Definition and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| listOfInterfaces | It specifies the interfaces that needs to be traced in the given ManagedEntityFunction.The attribute is applicable only for Trace. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9] |
| listOfNETypes | It specifies in which type of ManagedFunction the trace should be activated. The attribute is applicable only for Trace with Signalling Based Trace activation. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9] |
| traceCollectionEntityAddress | It specifies the address of the Trace Collection Entity within an IRPManager. The attribute is applicable for for both Trace and MDT | See 3GPP TS 32.422 [9] |
| traceDepth | It specifies the trace depth of the ManagedEntityFunction instances. The attribute is applicable only for Trace. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9] |
| traceReference | A globally unique identifier, which uniquely identifies the Trace Session that is created by the TraceJob. The attribute is applicable for both Trace and MDT. | Any positive integer value |
| traceTarget | It specifies the target object of the Trace MDT. The attribute is applicable for both Trace and MDT. | (IMSI or IMEI or IMEISV or Public ID or Private IDand/or a list of Cell(identified by its DN id) and/or TA/LA/RA). IMSI, IMEI, IMEISV and Public ID are mutually exclusive. Cell, TA, LA, RA are mutually exclusive. |
| triggeringEvent | It specifies the triggering event parameter of the trace session. The attribute is applicable only for Trace. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9] |
| jobType | It specifies the MDT mode and it specifies also whether the TraceJob represents only MDT, Trace or a combined Trace and MDT job. The attribute is applicable for both Trace and MDT. | See 3GPP TS 32.422 [9] |
| listOfMeasurements | It specifies the UE measurements that shall be collected in an Immediate MDT job. The attribute is applicable only for Immediate MDT. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9] |
| reportingTrigger | It specifies whether periodic or event based measurements should be collected.The attribute is applicable only for Immediate MDT and when the listOfMeasurements is configured for M1. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9] |
| reportInterval | It specifies the interval between the periodical measurements that shall be taken when the UE is in connected mode. The attribute is applicable only for Immediate MDT and when reportingTrigger is configured for periodical measurements. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9] |
| reportAmount | It specifies the number of measurement reports that shall be taken for periodic reporting while the UE is in connected. The attribute is applicable only for Immediate MDT and when reportingTrigger is configured for periodical measurements. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9] |
| eventThreshold | It specifies the threshold which should trigger the reporting in case A2 event reporting. The attribute is applicable only for Immediate MDT and when reportingTrigger is configured for A2 event. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9] and 3GPP TS 36.331 [y] |
| loggingInterval | It specifies the periodicty for logging MDT. The attribute is applicable only for Logged MDT. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9], 3GPP TS 25.331 [] 3GPP TS 36.331 [y] |
| loggingDuration | It specifies how long the MDT configuration is valid at the UE in case of Logged MDT. The attribute is applicable only for Logged MDT. In case this attribute is not used, it carries a nulls semantic. | See 3GPP TS 32.422 [9] |

# 6	Interface Definition

## 6.1	Class diagram representing interfaces

Figure 6.1: Class Diagram

## 6.2	Generic rules

-	Rule 1: each operation with at least one input parameter supports a pre-condition valid_input_parameter which indicates that all input parameters shall be valid with regards to their information type. Additionally, each such operation supports an exception operation_failed_invalid_input_parameter which is raised when pre-condition valid_input_parameter is false. The exception has the same entry and exit state.

-	Rule 2: each operation with at least one optional input parameter supports a set of pre-conditions supported_optional_input_parameter_xxx where "xxx" is the name of the optional input parameter and the pre-condition indicates that the operation supports the named optional input parameter. Additionally, each such operation supports an exception operation_failed_unsupported_optional_input_parameter_xxx which is raised when (a) the pre-condition supported_optional_input_parameter_xxx is false and (b) the named optional input parameter is carrying information. The exception has the same entry and exit state.

-	Rule 3: each operation shall support a generic exception operation_failed_internal_problem which is raised when an internal problem occurs and that the operation cannot be completed. The exception has the same entry and exit state.

NOTE:	These rules are mapped at the solution set level. Pre-conditions and exceptions, generated by these rules, need not appear explicitly in the present document.

## 6.3	TraceIRPManagement (M)

### 6.3.1	Operation activateTraceJob (M)

#### 6.3.1.1	Definition

This operation support IPRManager’s request to create a TraceJob through Itf-N.

Once the TraceJob has been created, the attributes of the TraceJob will not be modified during the lifetime of the TraceJob.

One TraceJob can manage Trace Sessions in one or more ManagedEntity.

#### 6.3.1.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| iOCInstance | M | ManagedEntity.objectInstance | It specifies the DN of ManagedEntity instance where Trace Session is to be activated. |
| listOfInterfaces | O | TraceJob.listOfInterfaces |  |
| listOfNeTypes | CM | TraceJob.listOfNeTypes | It specifies the type of ManagedFunctions. |
| traceDepth | CM | TraceJob.traceDepth | It shows the traceDepth set to the Trace Session. |
| traceReference | M | TraceJob.traceReference | It identifies the TraceSession. |
| traceTarget | M | TraceJob.traceTarget | (IMSI or IMEI or IMEISV or Public ID or Private IDand/or a list of Cell(identified by its DN id) and/or a list of TA/LA/RA. IMSI, IMEI, IMEISV and Public ID are mutually exclusive. Cell, TA, LA, RA is mutually exclusive. |
| triggeringEvent | CO | TraceJob.triggeringEvent |  |
| traceCollectionEntityAddress | CM | TraceJob.traceCollectionEntityAddress | It specifies the address to the Trace Collection Entity that is associated to the TraceJob. See 3GPP TS 32.422 [9]. |
| jobType | M | TraceJob.jobType | It specifies the type of the TraceJob |
| listOfMeasurements | CM | TraceJob.listOfMeasurements | It specifies the measurements to be collected from the UE |
| reportingTrigger | CM | TraceJob.reportingTrigger | It specifies the reporting trigger (event based reporting or periodic reporting) in the UE. |
| reportInterval | CM | TraceJob.reportInterval | It specifies the interval between the periodical measurements to be taken by the UE. |
| reportAmount | CM | TraceJob.reportAmount | It specifies the nuber of measurement reports to be taken in periodical reporting in the UE |
| eventThreshold | CM | TraceJob.eventThreshold | It specifies the threshold triggering the reporting in case of A2 event reporting. |
| loggingInterval | CM | TraceJob.loggingInterval | It specifies the periodicity of Logged MDT. |
| loggingDuration | CM | TraceJob.loggingDuration | It specifies the duration of the Logged MDT at the UE. |

#### 6.3.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Success, Failure, PartialSuccess) |  |
| unsupportedList | M | List of <ManagedEntity, TraceDepth, ListOfInterfaces, TraceTarget,listOfMeasurements,reportingTrigger, reportInterval,reportAmount,eventThreshold, loggingInterval, loggingDuration, reason) | It specifies what attributes are not supported when a Trace Session is activated. The list can contain one or all of the elements and relevant only for error cases. |

#### 6.3.1.4	Pre-condition

validTraceReference AND validTraceDepth AND validTraceTarget

| Assertion Name | Definition |
| --- | --- |
| validTraceDepth | The traceDepth input parameter is valid. |
| validTraceReference | The traceReference given is not matching to any existing traceReference value in the activated TraceJobs. |
| validTraceTarget | The traceTarget input parameter is valid. |

#### 6.3.1.5	Post-condition

traceSessionActivated

| Assertion Name | Definition |
| --- | --- |
| traceSessionActivated | The Trace Session identified by the traceReference is activated in the given ManagedEntity instances. |

#### 6.3.1.6	Exceptions

| Exception Name | Definition |
| --- | --- |
| invalidTraceDepth | Condition: (validTraceDepth) is false. Returned Information: output parameter status is set to "Failure". Exit state: Entry State. |
| invalidTraceTarget | Condition: (validTraceTarget) is false. Returned Information: output parameter status is set to "Failure". Exit state: Entry State. |
| notuniqueTraceReference | Condition: (validTraceReference) is false. Returned Information: output parameter status is set to "Failure". Exit state: Entry State. |

#### 6.3.1.7	Constraints

| Name | Definition |
| --- | --- |
| listOfNeTypes | It is a Signalling Based Activated trace that is requested. |
| traceCollectionEntityAddress CM qualifier | Tracing is performed in EPS |
| listOfMeasurements | MDT is supported |
| reportingTrigger | MDT is supported |
| reportInterval | MDT is supported |
| reportAmount | MDT is supported |
| eventThreshold | MDT is supported |
| loggingInterval | MDT is supported |
| loggingDuration | MDT is supported |
| listOfMeasurements | MDT is supported |
| traceDepth | Trace is supported |
| triggeringEvent | Trace is supported |

### 6.3.2	Operation deactivateTraceJob (M)

#### 6.3.2.1	Definition

This operation supports IPRManager’s request to stop a TraceJob through Itf-N. When this operation is received in the TraceIRP the TraceJob shall deactivate the requested Trace Session in the requested ManagedEntity instances.

#### 6.3.2.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| traceReference | M | TraceJob.traceReference | This is a unique ID of the TraceJob |
| traceTarget | M | TraceJob.traceTarget | (IMSI or IMEI or IMEISV or Public ID or Private IDand/or a list of Cell(identified by its DN id) and/or a list of TA/LA/RA. IMSI, IMEI, IMEISV and Public ID are mutually exclusive. Cell, TA, LA, RA is mutually exclusive. |

#### 6.3.2.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Success, Failure) | The operation may fail because of a specified or an unspecified reason. |
| traceRecordingSessionReference | CM |  | This would indicate if a Trace Recording Session is ongoing when the deactivation command has been given. |

#### 6.3.2.4	Pre-condition

validTraceReference

| Assertion Name | Definition |
| --- | --- |
| validTraceReference | The TraceReference input parameter is valid, which means that the TraceIRP is aware of such TraceJob, which has this traceReference value and is aware of the ManagedEntity holding such Trace Session. |

#### 6.3.2.4	Post-condition

TraceSessionisdeactivated

| Assertion Name | Definition |
| --- | --- |
| TraceSessionisdeactivate | The Trace Session identified by the traceReference is deactivated in the requested ManagedEntity instance and the TraceJob is stopped. |

#### 6.3.2.6	Exceptions

| Exception Name | Definition |
| --- | --- |
| notuniqueTraceReference | Condition: (validTraceReference) is false. Returned Information: output parameter status is set to "Failure". Exit state: Entry State. |

### 6.3.3	Operation listTraceJob (M)

#### 6.3.3.1	Definition

This operation support IPRManager’s request to list the parameters of a specific TraceJob through Itf-N.

#### 6.3.3.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| traceReference | M | TraceJob.traceReference | It specifies the Trace Session that is requested for interrogation. |

#### 6.3.3.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| iOCInstance | M | ManagedElement.objectInstance | It specifies the DN of ManagedElement instance where a Trace Session is activated. |
| listOfInterfaces | O | TraceJob.listOfInterfaces | It specifies the list of interfaces trace control and configuration parameter that is associated with the TraceJob. See 3GPP TS 32.422 [9] |
| Status | M | ENUM (Success, Failure) | The operation may fail because of a specified or an unspecified reason. |
| traceDepth | CM | TraceJob.traceDepth | It shows the traceDepth trace control and configuration parameter that is associated to the TraceJob. |
| traceRecordingSessionReference | CM |  | This would indicate if a Trace Recording Session is ongoing when the deactivation command has been given. |
| traceTarget | M | TraceJob.traceTarget | (IMSI or IMEI or IMEISV or Public ID or Private IDand/or A list of Cell(identified by its DN id) and/or A list of TA/LA/RA. IMSI, IMEI, IMEISV and Public ID are mutually exclusive. Cell, TA, LA, RA is mutually exclusive. |
| triggeringEvent | CO | TraceJob.triggeringEvent | It specifies the triggering event trace control and configuration parameter that is associated to the TraceJob. See 3GPP TS 32.422 [9]. |
| traceCollectionEntityAddress | CM | TraceJob.traceCollectionEntityAddress | It specifies the address to the Trace Collection Entity that is associated to the TraceJob. See 3GPP TS 32.422 [9]. |
| jobType | M | TraceJob.jobType | It specifies the type of the TraceJob. It can be one of the following: Trace, MDT data collection, Trace and MDT data collection. |
| listOfMeasurements | CM | TraceJob.listofMeasurements | It specifies the measurements to be collected from the UE |
| reportingTrigger | CM | TraceJob.reportingTrigger | It specifies the reporting trigger (event based reporting or periodic reporting) in the UE. |
| reportInterval | CM | TraceJob.reportInterval | It specifies the interval between the periodical measurements to be taken by the UE. |
| reportAmount | CM | TraceJob.reportAmount | It specifies the nuber of measurement reports to be taken in periodical reporting in the UE |
| eventThreshold | CM | TraceJob.eventThreshold | It specifies the threshold triggering the reporting in case of A2 event reporting. |
| loggingInterval | CM | TraceJob.loggingInterval | It specifies the periodicity of the logging for dowlink pilot strength measurement in Logged MDT. |
| loggingDuration | CM | TraceJob.loggingDuration | It specifies the duration of the Logged MDT at the UE. |

#### 6.3.3.4	Pre-condition

validTraceReference

| Assertion Name | Definition |
| --- | --- |
| validTraceReference | The traceReference input parameter is valid, which means that the TraceIRP is aware of such TraceJob, which has this traceReference value and is aware of the ManagedEntity holding such Trace Session. |

#### 6.3.3.5	Post-condition

TraceSessionFound

| Assertion Name | Definition |
| --- | --- |
| TraceSessionFound | The TraceIRP has found the requested TraceJob with the traceReference and can read the configured parameters. |

#### 6.3.3.6	Exceptions

| Exception Name | Definition |
| --- | --- |
| notuniqueTraceReference | Condition: (validTraceReference) is false. Returned Information: output parameter status is set to "Failure". Exit state: Entry State. |

#### 6.3.3.7	Constraints

| Name | Definition |
| --- | --- |
| traceCollectionEntityAddress CM qualifier | Tracing is performed in EPS |
| listOfMeasurements | MDT is supported |
| reportingTrigger | MDT is supported |
| reportInterval | MDT is supported |
| reportAmount | MDT is supported |
| eventThreshold | MDT is supported |
| loggingInterval | MDT is supported |
| loggingDuration | MDT is supported |
| traceDepth | Trace is supported |
| triggeringEvent | Trace is supported |

### 6.3.4	Operation listActivatedTraceJobs (M)

#### 6.3.4.1	Definition

This operation support IPRManager’s request to list all the activated TraceJobs through Itf-N.

#### 6.3.4.2	Input parameters

No input parameters for this operation.

#### 6.3.4.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| traceReferenceList | M | List of < TraceJob.traceReference.objectinstance > | The TraceReferenceList provides the identification of each activated Trace Session. If no TraceReference can be found, then this list is empty and status is "Success" |
| status | M | ENUM (Success, Failure) | The operation may fail because of a specified or an unspecified reason. |

### 6.3.5	Notification notifyTraceRecordingSessionFailure (O)

#### 6.3.5.1		Definition

The TraceIRP notifies all subscribed IRPManagers and the Trace Collection Entity (if its address is provided) if a Trace Recording Session in a ManagedEntity has not been started due to any problem.

#### 6.3.5.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M,Y | TraceIRP.objectClass | Notification header |
| objectInstance | M,Y | TraceIRP.objectInstance | Notification header |
| eventTime | M,Y | -- | Notification header |
| notificationType | M,Y | "notifyTraceRecordingSessionFailure" | Notification header |
| systemDN | M,Y | -- | Notification header |
| notificationID | O,Y | -- | Notification header |
| traceRecordingSessionReference | O,N | -- | The Trace Recording Session Reference may be visible only in signalling based activation. |
| traceReference | M,Y | TraceJob.traceReference |  |
| reason | O,N | -- |  |

#### 6.3.5.3	Triggering event

##### 6.3.5.3.1	From state

internalProblemInManagedEntity

| Assertion Name | Definition |
| --- | --- |
| internalProblemInManagedEntity | Because of an internal problem the ManagedEntity cannot start a Trace Recording Session. |

##### 6.3.5.3.2	To state

newNotificationReported

| Assertion Name | Definition |
| --- | --- |
| newNotificationReported | The " notifyTraceRecordingSessionFailure " notification is emitted to the subscribed IRPManager(s). |

### 6.3.6	Notification notifyTraceSessionLocalActivation (M)

#### 6.3.6.1	Definition

The TraceIRP notifies all subscribed IRPManagers if a Trace Session is configured by the Element Manager.

#### 6.3.6.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M,Y | TraceIRP.objectClass | Notification header |
| objectInstance | M,Y | TraceIRP.objectInstance | Notification header |
| eventTime | M,Y | -- | Notification header |
| notificationType | M,Y | "notifyTraceSessionLocalActivation" | Notification header |
| systemDN | M,Y | -- | Notification header |
| notificationID | O,Y | -- | Notification header |
| traceReference | M,Y | TraceJob.traceReference |  |
| traceTarget | M,Y | TraceJob.traceTarget |  |
| iOCInstance | M,Y | ManagedEntity.objectInstance |  |

#### 6.3.6.3	Triggering event

##### 6.3.6.3.1	From state

unknownTraceReference

| Assertion Name | Definition |
| --- | --- |
| unknownTraceReference | The TraceIRP has detected a traceReference associated to a Trace Session in a ManagedEntity that is not initiated via the Itf-N. |

##### 6.3.6.3.2	To state

newNotificationReported

| Assertion Name | Definition |
| --- | --- |
| newNotificationReported | The " notifyTraceSessionLocalActivation " notification is emitted to the subscribed IRPManager(s). |

### 6.3.7	Notification notifyTraceSessionIdentities (CM)

#### 6.3.7.1	Definition

The TraceIRP or the MmeFunction notifies the Trace Collection Entity about the identities of the subscriber and Equipment in case of tracing a session in E-UTRAN only.

#### 6.3.7.2	Input parameters

| Parameter Name | Qualifiers | Matching Information | Comment |
| --- | --- | --- | --- |
| objectClass | M,Y | TraceIRP.objectClass, or MmeFunction.objectClass | Notification header |
| objectInstance | M,Y | TraceIRP.objectInstance, or MmeFunction.objectClass | Notification header |
| eventTime | M,Y | -- | Notification header |
| notificationType | M,Y | "notifyTraceSessionIds" | Notification header |
| systemDN | M,Y | -- | Notification header |
| notificationID | O,Y | -- | Notification header |
| traceReference | M,Y | TraceJob.traceReference |  |
| traceRecordingSessionReference | M,Y | TraceJob.traceRecordingSessionReference |  |
| traceTarget | M,Y | TraceJob.traceTarget |  |

#### 6.3.7.3	Triggering event

##### 6.3.7.3.1	From state

FFS

| Assertion Name | Definition |
| --- | --- |
| FFS | FFS |

##### 6.3.7.3.2	To state

FFS

| Assertion Name | Definition |
| --- | --- |
| FFS | FFS |

#### 6.3.7.4 Constraint

| Name | Definition |
| --- | --- |
| notifyTraceSessionIdentities Notification CM qualifier | Tracing is performed only in E-UTRAN |

End of modifications