3GPP TSG-SA5 (Telecom Management)	S5-110527

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-110375

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0098 |  rev | - |  Current version: | 10.2.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Removing UE capability from Minimization of Drive Tests (MDT) parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM-PM-UE |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | UE category is a statis parameter and in the current MDT functionality UE category as selection criteria does not change the functionality. See also the LS from RAN2 (S5-110366) |  |  |  |  |  |  |  |  |
| Summary of change:  |  | UE category has been removed from the MDT parameters, some editorial cleanup. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.1.1.6a, 4.1.2.12.1, 4.1.2.12.2, 4.1.2.12.3, 4.1.2.14, removing 5.8, and 5.10.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

#### First change

#### 4.1.1.6a	E-UTRAN activation mechanisms for area based UE measurement collections without IMSI/IMEI(SV) selection

For area based MDT data collection with no IMSI/IMEI(SV) criteria, the UE selection can  be done in the radio network at eNB/RNC based on the input information received from OAM. This mechanism works for the following input parameters:

- Area information only

The following figure summarizes the flow how the MDT configuration is done utilising the cell traffic trace functionality for this scenario:

- The EM sends a Trace Session activation request to the eNB. This request includes the parameters for configuring UE measurements:

  - Job type

  - Area selection condition where the UE measurements should be collected: list of E-UTRAN cells. Tracking Area should be converted to E-UTRAN cell.

  - List of measurements

  - Reporting Trigger

  - Report Interval

  - Report Amount

  - Event Threshold

  - Logging Interval

  - Logging Duration

  - Trace Reference

  - IP address of TCE

Note that at the same time not all the parameters can be present. The criteria for which parameters are present are described in section 5.10 of this document..

- When eNB receives the Trace Session activation request from its EM, it shall start a Trace Session and should save the parameters associated to the Trace Session.

- eNB shall activate the MDT functionality to the selected UEs. eNB shall assign Trace Recording Session Reference corresponding to the selected UE. The eNB shall send at least the following configuration information to the UE in case of Logged MDT:

  - Trace Reference

  - Trace Recording Session Reference

  - IP address of TCE

  - Logging Interval

  - Logging Duration

  - Absolut time reference

Note: For UEs currently being in idle mode and camping in the cell the logged MDT configuration cannot be sent. These UEs may be configured when they initiate some activity (e.g., Service Request or Tracking Area Update) at next time.

In case of Immediate MDT the following parameters shall be sent to the UE:

- List of measurements

- Reporting trigger

- Report Interval

- Report Amount

- Event Threshold

Note that at the same time not all the parameters can be present. Conditions of the parameters are described in section 5.10 of this document.

6.	When UE receives the MDT activation it shall start the MDT functionality based on the received configuration parameters. The MDT related measurements are reported to the eNB via RRC signalling. In case of Logged MDT the MDT reporting is done when the network requests the log  First the UE indicates the MDT log availability as a one bit indicator in the RRCConnectionSetupComplete message during connection establishment as specified in 3GPP TS 37.320 [30]. The UE should report the Trace reference and IP address of TCE together with the MDT reports to the network in case of Logged MDT.

- eNB shall contact the MME so MME can send the subscriber identities (IMSI, IMEI(SV) to the TCE for the given UE, which reported the MDT reports to the eNB.

- For Immediate MDT when the eNB receives the MDT report from the UE in the RRC message the eNB shall capture it and put it to the trace record. A UE configured to perform Logged MDT measurements in IDLE indicates the availability of MDT measurements, by means of a one bit indicator, in RRCConnectionSetupComplete message during connection establishment as specified in [2]. The eNB can decide to retrieve the logged measurements based on this indication by sending the UEInformationRequest message to the UE. The UE can answer with the collected MDT logs in UEInformationResponse message.

- The Trace Records shall be forwarded to the Trace Collection Entity indicated in the MDT report received from the UE in case of Logged MDT.

The Immediate MDT measurement configuration is deleted in the UE together with the RRC context when entering idle mode.

The Logged MDT trace session is preserved in the UE until the duration time of the trace session expires, including also multiple idle periods interrupted by idle-connected-idle state transitions.

The Logged MDT trace session context of the UE is stored in the network as long as the trace session is active, including also the periods when the UE is in connected state.

#### Next change

#### 4.1.2.12	EPC Activation mechanism for UE based network performance measurements

##### 4.1.2.12.1	General

UE measurements activation extends the EPC trace activation procedure, as described in 4.1.2.10. When a Trace Session is activated, configuration parameters of UE based measurements are added into the message.

For IMSI/IMEI(SV) based UE selection, or IMSI/IMEI(SV) combined with geographical area based UE selection, UE performance measurements activation request is propagated to UE finally.

This mechanism works for the following input parameters:

- IMSI only or

- IMSI and area information or

- or

- IMEI(SV) only or

- IMEI(SV) and area information or

After the IMSI or IMEISV user attached to the network, the MME will forward the MDT configurations to the corresponding eNB which serves the IMSI or IMEISV user.

- MDT criteria checking on eNB:

- For immediate MDT, after eNB got the MDT configuration, the eNB can detect the area informationand decide whether the selected IMSI/IMEISV can fit into the criteria for initiating MDT data collection.

- For logged MDT, the eNB will forward the MDT configuration criteria to the selected IMSI/IMEISV. The area criteria checking will be done at UE side after UE received the MDT configuration criteria.

- MDT criteria checking on UE:

- For immediate MDT, there is no need to do MDT criteria checking on UE.

- For logged MDT, The area criteria checking will be done at UE side after UE received the MDT configuration criteria.

After UE receives from eNodeB the configuration parameters via the message RRC Connection Reconfiguration, it detects whether it stays within the specified area. If yes the UE will execute measurement job. Otherwise UE will do nothing but waiting.

In case of Immediate MDT trace (e.g., IMSI/IMEI based selection), the Immediate MDT trace session context of the UE shall be preserved in the network when the UE enters idle mode.

The Logged MDT trace session is preserved in the UE until the duration time of the trace session expires, including also multiple idle periods interrupted by idle-connected-idle state transitions.

The Logged MDT trace session context of the UE is stored in the network as long as the trace session is active, including also the periods when the UE is in connected state.

Two scenarios shall be considered according to UE status when EMS activates UE based measurements job: before UE attachment, after UE attachment, different procedures are described in 4.1.2.12.2, 4.1.2.12.3.

##### 4.1.2.12.2	Activation of UE measurements task before UE attaches to the network

As shown in figure 4.1.2.12.1, by adding configurations of UE based measurements EMS activate the Trace Session for UE measurements job.

- Figure 4.1.2.12.1: UE measurement activation procedure in EPC

When HSS activates the trace, for UE measurements job, to the MME the following configuration parameters shall be included in the message:

- jobType

- IMSI or IMEISV

- Area scope  (e.g. TA, Cell)

- Trace Reference

- List of measurements

- Reporting Trigger

- Report Interval

- Report Amount

- Event Threshold

- Logging Interval

- Logging duration

Note that at the same time not all the parameters can be present. The conditions are described in Section 5.10 of this document.

- IP address of Trace Collection Entity

The Specified geographical area field is available when IMSI/IMEI(SV) combined with geographical area are needed for UE selection.

When MME activate UE measurements activation to eNodeB, the MDT configuration parameters can be included in the message in the Initial Context Setup:

- Area scope (TA, Cell)

- Trace Reference

- Trace Recording Session Reference

- List of measurements

- Reporting Trigger

- Report Amount

- Report Interval

- Event Threshold

- Logging Interval

- Logging Duration

- IP address of Trace Collection Entity

Note that at the same time not all the parameters can be present. The conditions are described in Section 5.10 of this document.

##### 4.1.2.12.3	Activation of UE measurements task after UE attachment

- Figure 4.1.2.12.2: UE measurement activation in EPC after UE attachment

- The messages propagated to HSS, MME and eNodeB are the same as described in clause 4.1.2.12.2.

- When MME can send Trace Start to eNodeB, the following configuration parameters shall be included in the message:

- Area scope (TA, Cell)

- Trace Reference

- Selected UE Measurements

- IP address of Trace Collection Entity

In case of logged MDT and the UE is currently being in idle mode, the MME is not required to initiate paging of the UE in order to send the configuration.

Then eNodeB initiates RRC Connection Reconfiguration Request in case of immediate MDT or the IdleMDTConfiguration RRC message in case of logged MDT toward the UE and sends the MDT measurement configuration parameters as received from the MME.

#### Next change

#### 4.1.2.14	CS domain activation mechanism for UE based network performance measurements

In UMTS it is also possible to send the UE measurement job activation via the CS domain instead of the PS domain. The activation mechanism is shown in figure 4.1.2.14.1

- Figure 4.1.2.14.1 UE measurement activation procedure in PS domain during attach procedure

The Trace Session activation is started from the EMS, when it activates the Trace Session to the HSS. The HSS stores the trace control and configuration parameters in its database.

When a UE registers with the network by sending an ATTACH_REQUEST message to the MSC Server, it updates the location information in the HSS by sending the UPDATE_LOCATION message to the HSS. The HSS checks if the UE is being traced. If it is being traced, the HSS shall propagate the trace control and configuration parameters to the MSC Server by sending a MAP-ACTIVATE_TRACE_MODE - see 3GPP TS 29.002 [11] message to the MSC Server (This message can be embedded also in the MAP INSERT SUBSCRIBER DATA message). When an inter-VLR Location Area update occurs, HSS shall send the MAP-ACTIVATE_TRACE_MODE message to the new VLR / MSC Server. The Trace Session Activation from HSS to MSC Server shall contain the following MDT specific parameters in addition to the existing trace parameters:

- Job type

- Area Scope

- List of measurements

- Reporting Trigger

- Report Interval

- Report Amount

- Event Threshold

- Logging Interval

- Logging Duration

Note that at the same time not all of the parameters can be present. The condition which parameters shall be present is described in section 5.10 of the present document.

When MSC Server receives the MAP-ACTIVATE_TRACE_MODE message it shall store the trace control and configuration parameters and shall start a Trace Session and shall send the CN_INVOKE_TRACE message to the RNC. The MSC Server shall send the following parameters to the RNC beside the existing trace parameters:

- Job type

- Area Scope

- List of measurements

- Reporting Trigger

- Report Interval

- Report Amount

- Event Threshold

- Logging Interval

- Logging Duration

Note that at the same time not all of the parameters can be present. The condition which parameters shall be present is described in section 5.10 of the present document.

In case of Immediate MDT trace (e.g., IMSI/IMEI based selection), the Immediate MDT trace session context of the UE shall be preserved in the network when the UE enters idle mode.

The Logged MDT trace session is preserved in the UE until the duration time of the trace session expires, including also multiple idle periods interrupted by idle-connected-idle state transitions.

The Logged MDT trace session context of the UE is stored in the network as long as the trace session is active, including also the periods when the UE is in connected state.

### Next Change

## 5.7	Trace Recording Session Reference (M)

- This parameter shall be a 2 byte Octet String.

## 5.8	Void

5.9	IP Address of Trace Collection Entity (M)

This is a mandatory parameter which defines the IP address to which the Trace records shall be transferred. IPv4 and/or IPv6 address(es) may be signalled.

### 5.10.9	Logging Duration

The parameter is mandatory if the the job type parameter is configured for Logged MDT. The parameter determines the validity of MDT logged configuration for IDLE. The timer starts at time of receiving configuration by the UE, and continues independent of UE state transitions and RAT or RPLMN changes.

Detailed definition of the parameter is in 3GPP TS 37.320 [30], 3GPP TS 25.331 [31] and 3GPP TS 36.331 [32]:

The parameter can have the following values in UMTS and LTE:

- 600sec,

- 1200sec,

- 2400sec,

- 3600sec,

- 5400sec,

- 7200sec

### 5.10.10	Void