3GPP TSG-SA5 (Telecom Management)	S5-111509

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111435

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 101 |  rev | 2 |  Current version: | 10.2.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Update the E-UTRAN activation mechanism for area based MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM-PM-UE |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | When MME sends the the subscriber identities (IMSI, IMEI(SV) ) to the TCE, it should send the Trace Recording Session Reference and Trace Reference together with the IMSI/ IMEI(SV). But in TS 32.422, this is not reflected in the E-UTRAN activation mechanism for area based MDT. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Update the description of the step 7 in the E-UTRAN activation mechanism for area based MDT. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The E-UTRAN activation mechanism for area based MDT will not be clear enough. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.1.1.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

#### 4.1.1.6a	E-UTRAN activation mechanisms for area based UE measurement collections without IMSI/IMEI(SV) selection

For area based MDT data collection with no IMSI/IMEI(SV) criteria, the UE selection can  be done in the radio network at eNB/RNC based on the input information received from OAM. This mechanism works for the following input parameters:

- Area information only

- Area information and device capability information

- Device capability information only

The following figure summarizes the flow how the MDT configuration is done utilising the cell traffic trace functionality for this scenario:

- The EM sends a Trace Session activation request to the eNB. This request includes the parameters for configuring UE measurements:

  - Job type

  - Area selection condition where the UE measurements should be collected: list of E-UTRAN cells. Tracking Area should be converted to E-UTRAN cell.

- Device capability requirements (e.g. min battery status/free memory, etc.) this can also include the UE Capability (e.g. UE Category etc.). This parameter may not always be present.

  - List of measurements

  - Reporting Trigger

  - Report Interval

  - Report Amount

  - Event Threshold

  - Logging Interval

  - Logging Duration

  - Trace Reference

  - IP address of TCE

Note that at the same time not all the parameters can be present. The criteria for which parameters are present are described in section 5.10 of this document..

- When eNB receives the Trace Session activation request from its EM, it shall start a Trace Session and should save the parameters associated to the Trace Session.

- eNB should retrieve the device capabilities from all UEs active in the cell(s) if device capability requirement is given for the Trace Session and eNB didn’t retrieve the device capabilities before.

- eNB shall select the suitable UEs for MDT data collection. The selection is based on the device capability requirements received from EM and the actual UE capabilities got from the UEs and on the area received from the OAM and the area where UE is roaming.

- eNB shall activate the MDT functionality to the selected UEs. eNB shall assign Trace Recording Session Reference corresponding to the selected UE. The eNB shall send at least the following configuration information to the UE in case of Logged MDT:

  - Trace Reference

  - Trace Recording Session Reference

  - IP address of TCE

  - Logging Interval

  - Logging Duration

  - Absolut time reference

Note: For UEs currently being in idle mode and camping in the cell the logged MDT configuration cannot be sent. These UEs may be configured when they initiate some activity (e.g., Service Request or Tracking Area Update) at next time.

In case of Immediate MDT the following parameters shall be sent to the UE:

- List of measurements

- Reporting trigger

- Report Interval

- Report Amount

- Event Threshold

Note that at the same time not all the parameters can be present. Conditions of the parameters are described in section 5.10 of this document.

6.	When UE receives the MDT activation it shall start the MDT functionality based on the received configuration parameters. The MDT related measurements are reported to the eNB via RRC signalling. In case of Logged MDT the MDT reporting is done when the network requests the log  First the UE indicates the MDT log availability as a one bit indicator in the RRCConnectionSetupComplete message during connection establishment as specified in 3GPP TS 37.320 [30]. The UE should report the Trace reference, Trace Recording Session Reference and IP address of TCE together with the MDT reports to the network in case of Logged MDT.

- When the eNodeB receives the MDT trace record from UE, the eNodeB shall get the Trace Recording Session Reference, Trace Reference and TCE Id from the record, and send the Trace Recording Session Reference, Trace Reference and TCE IP address in the CELL TRAFFIC TRACE message to the MME via the S1 connection. When MME receives this S1 signalling message containing the Trace Recording Session Reference and Trace Reference, the MME shall check the user consent. If the result of the user consent checking is that user consent is available (as described in section 4.2.8.2 in this documernt), then the MME shall look up the subscriber identities (IMSI, IMEI (SV)) of the given call from its database, and send the subscriber identities (IMSI, IMEI (SV)) together with the Trace Recording Session Reference and Trace Reference to the TCE.

- For Immediate MDT when the eNB receives the MDT report from the UE in the RRC message the eNB shall capture it and put it to the trace record. A UE configured to perform Logged MDT measurements in IDLE indicates the availability of MDT measurements, by means of a one bit indicator, in RRCConnectionSetupComplete message during connection establishment as specified in [2]. The eNB can decide to retrieve the logged measurements based on this indication by sending the UEInformationRequest message to the UE. The UE can answer with the collected MDT logs in UEInformationResponse message.

- The Trace Records shall be forwarded to the Trace Collection Entity indicated in the MDT report received from the UE in case of Logged MDT.

The Immediate MDT measurement configuration is deleted in the UE together with the RRC context when entering idle mode.

The Logged MDT trace session is preserved in the UE until the duration time of the trace session expires, including also multiple idle periods interrupted by idle-connected-idle state transitions.

The Logged MDT trace session context of the UE is stored in the network as long as the trace session is active, including also the periods when the UE is in connected state.

End of changes