3GPP TSG-SA5 (Telecom Management)	S5-110181

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-11abcd

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.526 | CR | 0001 |  rev | - |  Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correcting the support qualifiers of SONControl attributes - Align with 32.522 SON NRM IRP Information Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | The support qualifiers of SONControl attributes are optional with one note indicating “*) NOTE: At least one of the attributes shall be supported.”. This is ambiguous and can make the implementation problems. Whether and which attribute shall be supported depends on the related SON function is supported or not. For example, the support qualifier of attribute lboSwitch shall be M in case the Load Balancing Optimization function is supported. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Change the support qualifiers of SONControl attributes to CM and add the related attribute constraints. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Ambiguous support qualifiers cause implementation problems. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | A.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st change

- A.2.2.2	IOC SONControl

| Attribute of IOC SONTargets in 3GPP TS 32.522 [4] | SS Attribute | SS Type | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- | --- |
| hooSwitch | hooSwitch | boolean | CM | M | M |
| lboSwitch | lboSwitch | boolean | CM | M | M |
| NOTE: For all conditional qualifiers, see attribute constraints in TS 32.522 [4]. |  |  |  |  |  |

End of changes