3GPP TSG-SA5 (Telecom Management)	S5-110411

SA5#75, 24 - 28 Jan 2011; Sorrento, Italy	revision of S5-110165

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.426 | CR | 0037 |  rev | 1 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add combined TA/LA update related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | The CS fallback (CSFB) in EPS enables the provision of voice and other CS-domain services by reuse of the CS infrastructure. When a CS fallback capable UE is EPS/IMSI attached, it initiates the combined TA/LA procedure based on the triggers specified in TS 23.401 and when a CS fallback and/or SMS over SGs capable UE is not EPS/IMSI attached, it may initiate a combined TA/LA procedure in order to use CS Fallback or SMS over SGs services. If a CSFB capable user fails to perform the combined TA/LA update procedure, the user may not be able to use the CS-domain services, and if the success rate is low the user experience would be bad. So it is necessary to define combined TA/LA update procedure related measurements to evaluate the combined TA/LA update performance of EPC network by calculating the success rate. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | 1) Add the related reference 2) Add combined TA/LA update related measurements 3) Add use case of combined TA/LA update related measurements |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 4.1.12(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[2]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[3]	3GPP TS 24.301: " Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[4]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS); Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access (Release 8)".

[6]	3GPP TS 29.274: " Tunnelling Protocol for Control plane (GTPv2-C)".

[7]	3GPP TS 29.281: "GPRS Tunnelling Protocol User Plane (GTPv1-U)".

[8]	3GPP TS 36.414: "Evolved Universal Terrestrial Access Network (E-UTRAN); S1 data transport".

[9]	3GPP TS 29.272: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[10]	3GPP TS 23.203: "Policy and charging control architecture".

[11]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[12]	3GPP TS 36.412: "Evolved Universal Terrestrial Access Network (E-UTRAN);  S1 signaling transport".

[13]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[14]	IETF RFC 5136: "Defining Network Capacity".

[15]		3GPP TS 23.246: "Multimedia Broadcast Multicast Service (MBMS); Architecture and functional description".

[16]	3GPP TS 36.445: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); M1 data transport".

[X]	3GPP TS 23.272: Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2

Next change

### 4.1.12	Combined TA/LA update procedure

The three measurement types defined in this clause are subject to the "2 out of 3 approach".

#### 4.1.12.1	Attempted Combined TA/LA update `

- This measurement provides the number of attempted Combined TA/LA update procedures.

- CC.

- Receipt of "TRACKING AREA UPDATE REQUEST" message from a UE with the Update Type indicates that this is a combined Tracking Area/Location Area Update Request or a combined Tracking Area/Location Area Update with IMSI attach Request (TS 23.272 [17]).

- A single integer value per measurement type defined in e).

- MM.CmbTauLauAtt

- MMEFunction

- Valid for packet switching.

- EPS.

#### 4.1.12.2	Successful Combined TA/LA update

- This measurement provides the number of successfully performed Combined TA/LA update procedure.

- CC.

- Transmission of a "TRACKING AREA UPDATE ACCEPT" message with the LAI information element and without EMM cause information element to the UE, in response to a "TRACKING AREA UPDATE REQUEST" message in which the Update Type information element indicated to the MME that it is a Combined TA/LA update. If the "TRACKING AREA UPDATE ACCEPT" message is caused by a retransmission, this will not cause the counter to be increased. (TS 24.301 [3]).

- A single integer value per measurement type defined in e).

- MM. CmbTauLauSucc

- MMEFunction

- Valid for packet switching.

- EPS.

#### 4.1.12.3	Failed Combined TA/LA update

- This measurement provides the number of failed Combined TA/LA update procedure. The measurement is split into subcounters per the reject cause.

- CC

- Transmission of a “TRACKING AREA UPDATE REJECT” message to the UE; or Transmission of a "TRACKING AREA UPDATE ACCEPT" message with no EMM cause and LAI information element or with EMM cause information element to the UE, in response to a "TRACKING AREA UPDATE REQUEST" message in which the Update Type information element indicated to the MME that it is a Combined TA/LA update (TS 24.301 [3]).,

- A single integer value per measurement type defined in e).

- MM. CmbTauLauFail

- MMEFunction

- Valid for packet switching

- EPS.

Next change

# A.X	Use case for combined TA/LA update procedure related measurements

The CS fallback (CSFB) in EPS enables the provision of voice and other CS-domain services by reuse of the CS infrastructure. When a CS fallback capable UE is EPS/IMSI attached, it initiates the combined TA/LA procedure based on the triggers specified in TS 23.401 and when a CS fallback and/or SMS over SGs capable UE is not EPS/IMSI attached, it may initiate a combined TA/LA procedure in order to use CS Fallback or SMS over SGs services.If a CSFB capable user fails to perform the combined TA/LA update procedure, the user may not be able to use the CS-domain services, and if the success rate is low the user experience would be bad. So it is necessary to define combined TA/LA update procedure related measurements to evaluate the combined TA/LA update performance of EPC network by calculating the success rate.

End of changes