3GPP TSG-SA5 (Telecom Management)	S5-110433

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-110322

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0088 |  rev | 2 |  Current version: | 10.2.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Clarify trace parameters for E-UTRAN activation mechanism and update EPC starting mechanism - Align with 32.441 Trace Management IRP Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | - In E-UTRAN Cell Traffic trace, it’s important that the trace control parameters should be passed to eNB from its EM within trace activation messages, especially the E-UTRAN cells list, and interfaces of eNB that need to be traced. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | - Add the clarification that describes all the parameters which EM sends to the eNodeB within trace activation message in E-UTRAN Cell Traffic trace. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | - If the trace activation parameters of E-UTRAN Cell Traffic trace are not clarified here, different vendors may have their own judgement and take different ways to pass some parameters to eNB. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.1.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | N | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | N | Test specifications |  |  |  |  |  |  |
|  |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

#### 4.1.1.6	E-UTRAN activation mechanisms

In E-UTRAN the Management Based Trace Activation can be fulfilled with the E-UTRAN Cell Traffic trace functionality. In this case the Trace Session Activation is done to one or a list E-UTRAN cells within one eNodeB, where Trace Session is activated.

The following trace control and configuration parameters of the Trace Session are received by eNodeB in the Trace Session activation message from the EM:

•	Trace Reference

•	Trace Depth

•	E-UTRAN cells list

•	List of interfaces for eNB

•	IP address of Trace Collection Entity

When eNodeB receives the Trace Session Activation message from the EM for a given or a list of E-UTRAN cell(s) the eNodeB shall start a Trace Session for the given or list of E-UTRAN cell(s).

End of changes