3GPP TSG-SA5 (Telecom Management)	 S5-110511

Meeting SA5#75, 24-28 Jan 2011, Sorrento, Italy	revised to S5-110102

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.762 | CR | 0045 |  rev | 2 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add attributes to RNFunction in E-UTRAN Network Resource Model IRP Information Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Motorola Solutions, Nokia Siemens Networks, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | For RN, after Attach for RN preconfiguration (phase 1) , it will get a list of DeNB cells from OAM, and use the list to make the Attach for RN operation (phase 2) This DeNB cell list is missing in current NRMs. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add the candidateDeNBCells attributes into RNFunction |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The configuration of candiated DeNBCells for the RN cannot be supported on Itf-N; |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.18.2, 6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  | x |  | O&M Specifications |  |  | 32.766 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 6.3.18.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| rnId | M | M | - |
| candidateDeNBCells | M | M | M |

Editor’s note: the need of attribute candidateDeNBCells is for FFS.

Next Modified Section

### 6.5.1	Definition and legal values

Table 6.5.1.1 defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

- Table 6.5.1.1: Attributes definitions and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| adjacentCell | This attribute contains the DN of a EUtranGenericCell or ExternalEUtranGenericCell. |  |
| adjacentSector | This attribute contains the DN of an ExternalSector. |  |
| relatedAntennaList | This attribute contains the DNs of one or more AntennaFunction |  |
| relatedTmaList | This attribute contains the DNs of one or more TmaFunction |  |
| relatedEUtranCellList | This attribute contains the DNs of one or more EUtranGenericCell |  |
| candidateDeNBCells | A list of ECGIs of the candidates DeNB cells for the subject Relay Node in Attach for RN operation (phase 2), see 36.300[4]. | See 3GPP TS 36.413[27], 36.300[4] |
| cellLocalId | Unambiguously identify a cell within an eNodeB | 0 - 255. |
| cellIdList | This holds a list of DN of EUtranGenericCell. These cells all belong to one MBSFN Area. |  |
| cellSize | See cell-Size in TS 36.423 [24]. | See cell-Size in TS 36.423 [24]. |
| cellResvInfo | This attribute represents whether the cell is MBSFN Area Reserved Cell or not. See TS 36.300[11] for MBSFN Area Reserved Cell. | See 3GPP TS 36.443 [28] for Cell Reservation Info. |
| confOutputPower | It defines the allowed total power to use for all cells together in this sector. It may be set by the operator and/or limited by HW limitation or licensed power, e.g.: 20, 40, 60, 80,120 watts |  |
| relatedSector | This attribute contains the DN of an SectorEquipmentFunction |  |
| referenceSignalPower | This defines the cell specific downlink reference signal transmit power, which is described in 3GPP TS 36.213[25] | See 3GPP TS 36.331[10] |
| earfcn | It is the frequency number for the central frequency. See 3GPP TS 36.104[14]. | See 3GPP TS 36.104[14]. |
| earfcnul | Specifies the channel number for the central UL frequency. The mapping from channel number to physical frequency is described in 3GPP specification TS 36.101 [13] subclause 5.7.3. | See EARFCN in TS 36.101 [13] subclause 5.7.3. |
| earfcnDl | Specifies the channel number for the central DL frequency. The mapping from channel number to physical frequency is described in 3GPP specification TS 36.101 [13] subclause 5.7.3. | See EARFCN in TS 36.101 [13] subclause 5.7.3 |
| energySavingControl | This attribute allows to IRPManager to initiate energy saving activation or deactivation. Its value can not be changed by the IRPAgent. | Enumerated {energySaving, notEnergySaving}. |
| energySavingState | Specifies the status regarding the energy saving in the cell. If the value of energySavingControl is energySaving, then it shall be tried to achieve the value isEnergySaving for the energySavingState. If the value of energySavingControl is notEnergySaving, then it shall be tried to achieve the value isNotEnergySaving for the energySavingState. | Enumerated {isNotEnergySaving, isEnergySaving}. |
| eNBId | Unambiguously identifies an eNodeB within a PLMN | See 3GPP TS 36.413[27], 36.300[4] |
| fqBand | This is the frequency band supported by the hardware associated with the SectorEquipmentFunction. The earfcnDl and earfcnUl of cells associated with the SectorEquipmentFunction must be assigned with value within this fqBand value. | See section 5 Table 5.2-1 "E-UTRA frequency band" of TS 36.104 [14]. Other legal values would be applicable for other technologies such as for UTRA. |
| id | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| isESCoveredBy | The value of the attribute is configured by the IRPmanager and is not changed by the IRPAgent. It indicates whether the adjacentCell according to this planning provides no, partial or full coverage for the cell which name-contains the EUtranRelation instance. Adjacent cells with this attribute equal to “yes” are recommended to be considered as candidate cells to take over the coverage when the original cell is about to be transferred to energySaving state. The entirety of adjacent cells with this property equal to “partial” are recommended to be considered as entirety of candidate cells to take over the coverage when the original cell is about to be transferred to energySaving state. The value “partial” is not allowed in an eNB overlaid scenario. | No, partial, yes |
| isRemoveAllowed (see note 1) | This indicates if the subject EUtranRelation can be removed (deleted) or not. If ‘yes’, the subject EUtranRelation instance can be removed (deleted). If ‘no’, the subject EUtranRelation instance shall not be removed (deleted) by any entity but an IRPManager. | yes, no |
| isHOAllowed (see note 1) | This indicates if HO is allowed or prohibited. If ‘yes’, handover is allowed from source cell to target cell. The source cell is identified by the name-containing EUtranGenericCell of the EUtranRelation that has the isHOAllowed. The target cell is referenced by the EUtranRelation that has this isHOAllowed. If ‘no’, handover shall not be allowed. | yes, no |
| isICICInformationSendAllowed | This indicates if ICIC (Inter Cell Interference Coordination) load information message (see TS 36.423 [24] Section 9.1.2.1 LOAD INFORMATION) sending is allowed or prohibited. If ‘yes’, ICIC load information message sending is allowed from source cell to target cell. The source cell is identified by the name-containing EUtranGenericCell of the EUtranRelation that has the isICICInformationSendAllowed. The target cell is referenced by the EUtranRelation that has this isICICInformationSendAllowed. If ‘no’, ICIC load information message sending shall not be allowed. | yes, no |
| isLBAllowed | This indicates if load balancing is allowed or prohibited from source cell to target cell. If ‘yes’, load balancing is allowed from source cell to target cell. The source cell is identified by the name-containing EUtranGenericCell of the EUtranRelation that has the isLBAllowed. The target cell is referenced by the EUtranRelation that has this isLBAllowed. If ‘no’, load balancing shall be prohibited from source cell to target cell. | yes, no |
| maximumTransmissionPower | This is the maximum possible for all downlink channels, used simultaneously in a cell, added together. |  |
| mbsfnAreaId | This is the identifier of MBSFN Area. See TS 36.300[11] for MBSFN Area. | See 3GPP TS 36.443 [28] for mbsfnAreaId |
| partOfSectorPower | This is the requested part (i.e. %) of the total radio power available to the SectorEquipmentFunction. The requested % power should be allocated to the cell. | 0 : 100 |
| pb | PB, which is described in Section 5.2 of TS 36.213 [25] | See 3GPP TS 36.213[25] |
| pci | This holds the Physical Cell Identity (PCI) of the cell (for NM-Centralized, EM-Centralized and Distributed PCI assignment cases). In the case of NM-Centralized PCI assignment, see TS 36.300, [11] subclause 22.3.5, IRPManager signals a specific value by writing this attribute. | See TS 36.211 [12] subclause 6.11 for legal values of pci. |
| pciList | This holds a list of physical cell identities that can be assigned to the pci attribute by eNB. The assignment algorithm is not specified. This attribute shall be supported if and only if the EM-Centralized or Distributed PCI Assignment is supported. See TS 32.500, ref [15] subclause 6.1.6. | See TS 36.211 [12] subclause 6.11 for legal values of pci. The number of pci in the list is 1 to 504. |
| plmnIdList | List of unique identities for PLMN. Note: A cell can broadcast up to 6 PLMN-id's. This is to support the case that one cell can be used by up to 6 operators’ core networks. One member of plmnIdList is the primary PLMN Id. See TS 36.331 [10] section 6.2.2: SystemInformationBlockType1/cellAccessRelatedInformation/plmn-IdentityList is a SEQUENCE (SIZE (1..6)) | A list of at most six entries of PLMN Identifiers. The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC). See TS 23.003 [3] subclause 2.2 and 12.1. |
| sfAssignment | This is the uplink-downlink subframe configuration number of a TDD E-UTRAN cell. | See 3GPP TS 36.211[12]. |
| specialSfPatterns | This is the special subframe configuration number of a TDD E-UTRAN cell. | See 3GPP TS 36.211[12]. |
| tac | Common Tracking Area Code for the PLMNs. The identity used to identify tracking areas. | - It is the Tracking Area Code (TAC). - A cell can only broadcast one TAC. See TS 36.300 [11], section 10.1.7 (PLMNID and TAC relation). - TAC is defined in TS 23.003 [3], section 19.4.2.3. |
| tCI | This is the Target Cell Identifier. It consists of E-UTRAN Cell Global Identifier (ECGI) and Physical Cell Identifier (PCI) of the target cell. The EUtranRelation.tCI identifies the target cell from the perspective of the EUtranGenericCell, the name-containing instance of the subject EUtranRelation instance. | The Target Cell Identifier is defined in TS 36.300 [11]. See TS 36.211 [12] subclause 6.11 for legal values of the PCI. |
| x2BlackList | This is a list of DNs of ENBFunction and ExternalENBFunction. If the target node DN is a member of the source node’s ENBFunction.x2BlackList, the source node is: - Prohibited from sending X2 connection request to target node; - Forced to tear down established X2 connection to target node - Not allowed to accept incoming X2 connection request from target node. The same DN may appear here and in ENBFunction.x2WhiteList. In such case, the DN in x2WhiteList shall be treated as if it is absent. |  |
| x2IpAddressList | Represents one or more IP addresses used by ENBFunction for this ENBFunction’s X2 Interface | One or more IPv4 or IPv6 addresses |
| x2WhiteList | This is a list of DNs of ENBFunction and ExternalENBFunction. If the target node DN is a member of the source node’s ENBFunction.x2WhiteList, the source node : - Is allowed to request the establishment of X2 connection with the target node; - Is not allowed to initiate the tear down of established X2 connection to target node The same DN may appear here and in ENBFunction.x2BlackList. In such case, the DN here shall be treated as if it is absent. |  |
| x2HOBlackList | This is a list of DNs of ENBFunction. The ENBFunction.x2HOBlackList identifies a list of neighbour ENBFunction with whom the subject ENBFunction is prohibited to use X2 interface for HOs even if the X2 interface exists between them. |  |
| rnId | This is defined as RN Indicator. Based on that, the deNB selects an MME supporting relay functionality | See 3GPP TS 36.300 [11] for RN Indicator. |
| deNBId | This is defined as deNB Indicator. It indicates that specific cells is allowed to access by RNs. |  |
| allowedAccessClasses | This holds information for access classes (10-15) – [3GPP TS 22.011] that are allowed for the eUTRANCell . The access classes are: Class 10 – emergency call Class 11 - For PLMN Use. Class 12 - Security Services; Class 13 - Public Utilities (e.g. water/gas suppliers); Class 14 - Emergency Services; Class 15 - PLMN Staff; | The default value is all access classes are allowed See TS 22.011 [29] and 36.331 [10] for more details on the definition and SIB2 broadcast message definition |

NOTE: Attributes isRemoveAllowed and isHOAllowed each has 2 legal values, allow (A) and prohibited (P).  The two attributes are semantically equivalent to one attribute with 4 legal values such as:

hOAllow; hOProhibited; hOWhiteListed; hOBlackListed;

where

-	hOAllow == isRemoveAllowed is A and isHOAllowed is A;

-	hOProhibited == isRemoveAllowed is A and isHOAllowed is P;

-	hOWhiteListed == isRemoveAllowed is P and isHOAllowed is A;

-	hOBlackListed == isRemoveAllowed is P and isHOAllowed is P.

Therefore, the choice of an option is FFS.

End of modifications