3GPP TSG-SA5 (Telecom Management)	 S5-110512

Meeting SA5#75, 24-28 Jan 2011, Sorrento, Italy 	revised to S5-110111

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.425 | CR | 0032 |  rev | 2 |  Current version: | 10.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Apply the RRC connection reestablishment related measurements to RN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Motorola Solutions, Nokia Siemens Networks, China Mobile, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8)\ Rel-9 (Release 9)\ Rel-10 (Release 10)\ |  |
| Reason for change:  |  | The use case A.1also applies to the RN, but the corresponding RRC connection reestablishment related measurements for RN are missing. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Apply RRC connection reestablishment related measurements to RN |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The performance of RRC connection reestablishment in RN cannot be known |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 4.1.2	RRC connection re-establishment

The three measurement types defined in the subclause 4.1.2.n are subject to the "2 out of 3 approach".

#### 4.1.2.1	Attempted RRC connection re-establishments

- This measurement provides the number of RRC connection re-establishment attempts for each re-establishment cause.

- CC.

- Receipt of an RRCConnectionReestablishmentRequest message by the eNodeB/RN from the UE. Each RRCConnectionReestablishmentRequest received is added to the relevant per reestablishment cause measurement. The possible causes are included in TS 36.331 [8]. The sum of all supported per cause measurements shall equal the total number of RRC connection re-stablishment attempts. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRC.ConnReEstabAtt.Cause
where Cause identifies the reestablishment cause.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switching.

- EPS

#### 4.1.2.2	Successful RRC connection re-establishments

- This measurement provides the number of successful RRC connection re-establishments for each re-establishment cause.

- CC.

- Receipt by the eNodeB/RN of an RRCConnectionReestablishmentComplete message following a RRC connection reestablishment request. Each RRCConnectionReestablishmentComplete message received is added to the relevant per reestablishment cause measurement. The possible causes are included in TS 36.331 [8]. The sum of all supported per cause measurements shall equal the total number of successful RRC connection re-establishments. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRC.ConnReEstabSucc.Cause
where Cause identifies the establishment cause.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switching.

- EPS

#### 4.1.2.3	Failed RRC connection re-establishments

- This measurement provides the number of RRC re-establishment failures for each re-establishment cause.

- CC.

- Transmission of an RRCConnectionReestablishmentReject message by the eNodeB/RN to the UE or an expected RRCConnectionReestablishmentComplete message not received by the eNodeB/RN. 
Each failed RRC connection re-establishment is added to the relevant per re-establishment.cause measurement. The possible causes are included in TS 36.331 [8].
The sum of all supported per cause measurements shall equal the total number of RRC connection re-establishment failures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRC.ConnReEstabFail.Cause
where Cause identifies the re-establishment.cause.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switching.

- EPS

End of modifications