3GPP TSG-SA5 (Telecom Management)	S5-110514

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-110389

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.300 | CR | 0012 |  rev | 1 |  Current version: | 9.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Clarify name space and Distinguished Name usage examples and general updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Incorrect examples. Ambiguous text. Terms and ideas that are defined that are appropriate for Rel 99 to Rel-9 inclusive are no longer appropriate for modern releases |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Correct examples. Update Reference lists. Modify terms and ideas for use in modern releases. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The specification (written originally in R99) that specifies the IRP framework and architecture is no longer up-to-date to reflect the latest IS/SS developments. This would introduce, to specification readers, confusion and wrong expectations and can lead to systems’ implementations having interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 7.2, 8, Appendix A. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | N | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | N | Test specifications |  |  |  |  |  |  |
|  |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void.

[2]	ITU-T Recommendation X.500 (1993): "Information technology - Open Systems Interconnection - The Directory: Overview of concepts, models and services".

[3]	T. Howes, ISBN 1-57870-070-1: "Understanding and Deploying LDAP Directory Services".

[4]	IETF RFC 1737 (1994): "Functional Requirements for Uniform Resource Names".

[5]	IETF RFC 2247 (1998): "Using Domains in LDAP/X.500 Distinguished Names".

[6]	IETF RFC 1035 (1987): "Domain names - implementation and specification".

[7]	IETF RFC 2253 (1997): "Lightweight Directory Access Protocol (v3): UTF-8 String Representation of Distinguished Names".

[8]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[9]	3GPP TS 32.622: " Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[10]	Void.

[11]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[12]	3GPP TS 32.102: "Telecommunication management; Architecture".

[13]	ISO/IEC 14977: "Information technology – Syntactic metalanguage – Extended BNF".

[14]	ISO/IEC 646: "Information technology – ISO 7-bit coded character set for information interchange".

[15]	ISO/IEC 10646: "Information technology – Universal multiple-octet Coded Character Set (UCS)".

[16]	3GPP TS 32.150: "Integration Reference Point (IRP) Concept and definitions".

[17]	3GPP2 S.S0028-E “OAM&P for cdma2000 (Overview, 3GPP R7 Delta Specification,  3GPP2 Network Resource Model IRP)”

[18]	MEF Technical Specification MEF 7.1, Phase 2 EMS-NMS Information Model, October 2009

[19]	ATM Forum, Technical Committee, Network Management, M4 Network View CMIP MIB Specification

2nd Modified Section

## 7.2	Character syntax

This subclause specifies the character syntax for AttributeType and AttributeValue.

They are:

1.	Any character except:

-	comma character (',', U+002C),

-	equals sign character ('=', U+003D),

-	carriage return character (U+000D),

-	line feed character (U+000A),

-	plus sign character ('+', U+002B),

-	less-than sign character ('<', U+003C),

-	greater-than sign character ('>', U+003E),

-	number sign character ('#', U+0023),

-	semicolon character (';', U+003B),

-	reverse solidus character ('\', U+005C),

-	quotation mark character ('"', U+0022).

2.	The full stop character ('.', U+002E). This character may be used in the AttributeValue whose AttributeType is "DC". An example is "DC=marketing.CompanyXYZ.com". This full stop character shall not be used in AttributeType.

3.	The asterisk character ('*', U+002A) is reserved to denote wildcard. Wildcard character(s) can appear in AttributeType and AttributeValue. The wildcard character can be used to represent one or more characters which are legal in that field (i.e., attributeType, attributeValue).

3rd Modified Section

# 8	Examples of DN in string representation

This subclause gives a few examples of DN written in the string representation specified in the present document.

EXAMPLE 1:	"DC=com,DC=CompanyXYZ,DC=marketing,IRPAgent=ATMPVCBilling, Log=19990101131000,AccountingRecord=100098". In this example, the name space aligns with DNS. The AttributeType of the top three RDN are "DC". Concatenation of the corresponding AttributeValues produces the DNS registered name, i.e. "marketing.CompanyXYZ.com". The top RDN is the Global Root because DNS defines "DC=com" as the root of its name space. That top RDN is the Local Root as well.

EXAMPLE 2:	"DC=marketing.CompanyXYZ.com,IRPAgent=ATMPVCBilling, Log=19990101131000,AccountingRecord=100098". In this example, the name space aligns with DNS as well. Instead of using three RDNs to represent the DNS registered name, this example chooses to use one RDN. The top RDN is the Global Root (and Local Root as well).

EXAMPLE 3:		"SubNetwork=ABCNetwork,SubNetwork=TN2,MSCFunction=B5C0100". In this example, the name space designer chooses not to name its objects under the DNS nor X.500 scheme. The name space designer chooses to use "SubNetwork=ABCNetwork" as the Local Root of its name space (by looking at the DN string, it is not possible to say if the Local Root is the Global Root). DNs in this name space will start with that string as their Local Root.

EXAMPLE 4:	The following example illustrates the use of the comma character as separator for RDNs. It also illustrates the use of space and full stop characters as part of the legal character syntax for RDNs: "CN=John T. Mills, O=Cyber System Consulting"

4th Modified Section

Annex A (normative):
Mapping of RDN AttributeType to Strings

NOTE:	This annex is normative for users of string representation.

AttributeType of RDN is mapped into string for use in the DN string representation. This annex specifies the mapping.

The AttributeType shall include all MO classes defined in the 3GPP Network Resource Model (NRM) IRP specifications such as the 3GPP TS 32.622 [9].

It can also include classes defined by other domain specific NRMs such as [17], [18] and [19].  Such inclusion is necessary where 3GPP-defined mobile access network is part of a Fixed Mobile Convergence network management environment.

There is one AttributeType that is not defined in NRM of 3GPP TS 32.622 [9] or other domain specific NRMs as listed in the Introduction clause. This special AttributeType is used to denote the domain component of the DNS. The following partial DN string representations are examples to illustrate the valid use of "DC" strings for the three DNS domain components of "marketing.CompanyXYZ.com":

-	"DC=com.CompanyXYZ.marketing,…"

-	"DC=com,DC=CompanyXYZ,DC=marketing,…"

-	"DC=com,DC=CompanyXYZ.marketing,…"

-	"DC=com.CompanyXYZ,DC=marketing,…"

Table A.1: Example of RDN AttributeType Strings

| String | AttributeType |
| --- | --- |
| DC | Domain component of DNS |
| SubNetwork | MO class name SubNetwork defined in NRM of 3GPP TS 32.622 [9]. |
| etc. | See note. |
| NOTE: For each MO class name found in 3GPP set of specifications, its corresponding AttributeType String shall be identical to the class name with the leading character capitalised. |  |

1st Modified Section

End of Modified Section