3GPP TSG-SA5 (Telecom Management)	S5-111171

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-11abcd

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.646 | CR | 0009 |  rev | - |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Alignment with IS 32.642; cleanup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | To support the changes in the IS 32.642. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The link to the IS is updated. Attributes that have changed Support Qualifier in the IS has been changed. Notes are inserted to refer to the IS for Support Qualifier CM Attribute names that starts with uppercase letter have been change to start with lower case letter. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Stage 3 would not be aligned with Stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 1, A.2.2.2, A.2.2.5 and A.2.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

# 1	Scope

- The present document specifies the Solution Sets for the UTRAN NRM IRP.

- This Solution Set specification is related to 3GPP TS 32.642 V10.2.X [4].

Next change

### A.2.2.2	IOC UtranGenericCell

Mapping from NRM IOC UtranGenericCell attributes and associations to SS equivalent MOC UtranGenericCell attributes

| NRM Associations/Attributes of IOC UtranGenericCell in TS 32.642 [4] | SS Attributes | SS Type | Support Qualifier | Read | Write |
| --- | --- | --- | --- | --- | --- |
| id | id | string | M | M | _ |
| cId | cId | long | M | M | M |
| localCellId | localCellId | long | M | M | M |
| retAntennaFunctionList | retAntennaFunctionList | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet | O | M | M |
| maximumTransmissionPower | maximumTransmissionPower | short | M | M | M |
| lac | lac | long | M | M | M |
| pichPower | pichPower | float | CM | M | O |
| pchPower | pchPower | float | CM | M | O |
| fachPower | fachPower | float | CMO | M | O |
| rac | rac | long | CM | M | M |
| sac | sac | long | M | M | M |
| uraList | uraList | GenericNetworkResourcesIRPSystem::AttributeTypes::LongSet | CM | M | M |
| AssociatedWith/ utranCell-IubLink | utranCellIubLink | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | M | M | - |
| cellMode | cellMode | GenericNRMAttributeTypes:: CellModeEnumType | M | M | - |
| operationalState | operationalState | StateManagementIRPOptConstDefs::OperationalStateTypeOpt | O | M | - |
| hsFlag | hsFlag | short | CM | M | - |
| hsEnable | hsEnable | short | CM | M | - |
| numOfHspdschs | numOfHspdschs | short | CM | M | - |
| numOfHsscchs | numOfHsscchs | short | CM | M | - |
| frameOffset | frameOffset | short | CO | M | - |
| cellIndividualOffset | cellIndividualOffset | float | CO | M | - |
| hcsPrio | hcsPrio | short | CO | M | - |
| maximumAllowedUlTxPower | maximumAllowedUlTxPower | short | CO | M | - |
| snaInformation | snaInformation | GenericNetworkResourceMAttributeTypes:: snaInformationType | CO | M | - |
| qrxlevMin | qrxlevMin | short | CO | M | - |
| deltaQrxlevmin | deltaQrxlevmin | short | CO | M | - |
| qhcs | qhcs | short | CO | M | - |
| penaltyTime | penaltyTime | short | CO | M | - |
| referenceTimeDifferenceToCell | referenceTimeDifferenceToCell | short | CO | M | - |
| readSFNIndicator | readSFNIndicator | boolean | CO | M | - |
| restrictionStateIndicator | restrictionStateIndicator | GenericNetworkResourceMAttributeTypes:: restrictionStateEnumType | CO | M | - |
| dpcModeChangeSupportIndicator | dpcModeChangeSupportIndicator | GenericNetworkResourceMAttributeTypes:: dpcModeChangeSupportEnumType | CO | M | - |
| tmaFunctionList | tmaFunctionList | GenericNetworkResourceIRPSystem::AttributeTypes::MOReferenceSet | O | M | - |
| NOTE 1: For all support qualifiers with the value “O”, see attribute constraints in TS 32.642 [4]. NOTE 2: For all support qualifiers with the value “CO” see attribute constraints in TS 32.642 [4]. NOTE 3: For all support qualifiers with the value “CM” see attribute constraints in TS 32.642 [4]. |  |  |  |  |  |

Next change

### A.2.2.5	IOC ExternalUtranGenericCell

Mapping from NRM IOC ExternalUtranGenericCell attributes and associations to SS equivalent MOC ExternalUtranGenericCell attributes

| NRM Attributes of IOC ExternalUtranGenericCell in TS 32.642 [4] | SS Attributes | SS Type | Support Qualifier | Read | Write |
| --- | --- | --- | --- | --- | --- |
| id | id | string | M | M | - |
| cId | cId | long | M | M | M |
| mcc | mcc | short | M | M | M |
| mnc | mnc | short | M | M | M |
| rncId | rncId | long | M | M | M |
| cellMode | cellMode | GenericNRMAttributeTypes::CellModeEnumType | M | M | - |
| lac | lac | long | M | M | M |
| rac | rac | long | CM | M | M |
| controllingRnc | controllingRnc | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | O | M | - |
| hsFlag | hsFlag | short | CM | M | - |
| frameOffset | frameOffset | short | CO | M | - |
| cellIndividualOffset | cellIndividualOffset | long | CO | M | - |
| hcsPrio | hcsPrio | short | CO | M | - |
| maximumAllowedUlTxPower | maximumAllowedUlTxPower | short | CO | M | - |
| qrxlevMin | qrxlevMin | short | CO | M | - |
| deltaQrxlevmin | deltaQrxlevmin | short | CO | M | - |
| qhcs | qhcs | short | CO | M | - |
| penaltyTime | penaltyTime | short | CO | M | - |
| referenceTimeDifferenceToCell | referenceTimeDifferenceToCell | short | CO | M | - |
| readSFNIndicator | readSFNIndicator | boolean | CO | M | - |
| restrictionStateIndicator | restrictionStateIndicator | GenericNetworkResourceMAttributeTypes:: restrictionStateEnumType | CO | M | - |
| dpcModeChangeSupportIndicator | dpcModeChangeSupportIndicator | GenericNetworkResourceMAttributeTypes:: dpcModeChangeSupportEnumType | CO | M | - |
| snaInformation | snaInformation | GenericNetworkResourceMAttributeTypes:: snaInformationType | CO | M | - |
| NOTE 1: For all support qualifiers with the value “O”, see attribute constraints in TS 32.642 [4]. NOTE 2: For all support qualifiers with the value “CO” see attribute constraints in TS 32.642 [4]. NOTE 3: For all support qualifiers with the value “CM” see attribute constraints in TS 32.642 [4]. |  |  |  |  |  |

Next change

### A.2.2.16	IOC TmaFunction

Mapping from NRM IOC TmaFunction attributes and associations to SS equivalent MOC TmaFunction attributes

| NRM Attributes of IOC tmaFunction in TS 32.642 [4] | SS Attributes | SS Type | Support Qualifier | Read | Write |
| --- | --- | --- | --- | --- | --- |
| id | tmaFunctionId | string | M | M | - |
| tmaSubunitNumber | tmaSubunitNumber | unsigned short | M | M | M |
| tmaStateFlag | tmaStateFlag | unsigned short | M | M | O |
| tmaFunctionFlag | tmaFunctionFlag | unsigned short | M | M | M |
| tmaMinGain | tmaMinGain | unsigned short | M | M | - |
| tmaMaxGain | tmaMaxGain | unsigned short | M | M | - |
| tmaResolution | tmaResolution | unsigned short | M | M | - |
| tmaGainFigure | tmaGainFigure | unsigned short | M | M | O |
| tmaNumberOfSubunits | tmaNumber OfSubunits | unsigned short | M | M | - |
| tmaNumberOfNonLinearGainValues | tmaNumberOfNonLinearGainValues | unsigned short | M | M | - |
| tmaNonLinearGainValue | tmaNonLinearGainValue | sequence of unsigned short | M | M | M |
| tmaUtranCellList | tmaUtranCellList | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet | M | M | M |
| tmaAdditionalDataFieldNumber | tmaAdditionalDataFieldNumber | unsigned short | CO | M | CO |
| tmaAntennaModelNumber | tmaAntennaModelNumber | string | CO | M | CO |
| tmaAntennaSerialNumber | tmaAntennaSerialNumber | string | CO | M | CO |
| tmaAntennaOperatingBands | tmaAntennaOperatingBands | unsigned short | CO | M | CO |
| tmaBeamwidthForEachOpBandInBandOrder | tmaBeamwidthForEachOpBandInBandOrder | sequence of unsigned short | CO | M | CO |
| tmaGainForEachOpBandInBandOrder | tmaGainForEachOpBandInBandOrder | sequence of unsigned short | CO | M | CO |
| tmaInstallationDate | tmaInstallationDate | string | CO | M | CO |
| tmaInstallersId | tmaInstallersId | string | CO | M | CO |
| tmaBaseStationId | tmaBaseStationId | string | CO | M | CO |
| tmaSectorId | tmaSectorId | string | CO | M | CO |
| tmaAntennaBearing | tmaAntennaBearing | unsigned short | CO | M | CO |
| tmaInstalledMechanicalTilt | tmaInstalledMechanicalTilt | short | CO | M | CO |
| tmaSubunitType | tmaSubunitType | unsigned short | CO | M | CO |
| tmaSubunitRxFrequencyBand | tmaSubunitRxFrequencyBand | sequence of unsigned short | CO | M | CO |
| tmaSubunitTxFrequencyBand | tmaSubunitTxFrequencyBand | sequence of unsigned short | CO | M | CO |
| tmaMaxSupportedGain | tmaMaxSupportedGain | unsigned short | CO | M | CO |
| tmaMinSupportedGain | tmaMinSupportedGain | unsigned short | CO | M | CO |
| tmaGainResolution | tmaGainResolution | unsigned short | CO | M | CO |

End of changes