3GPP TSG-SA5 (Telecom Management)	 S5-111367

Meeting SA5#76, 28 Feb -04 Mar 2011, San Diego, CA, USA 	revised to S5-111133

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.425 | CR | 0069 |  rev | 1 |  Current version: | 10.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Exclude HOs from RN in inter-eNB HO measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8)\ Rel-9 (Release 9)\ Rel-10 (Release 10)\ |  |
| Reason for change:  |  | The inter-eNB HO related measurements in section 4.3.1.2 are also applicable to DeNB, however, these measurements also imply to include the forwarded HOs for RN and those HOs were controlled and measured in RN and not directly related to DeNB performance. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Exclude HOs from RN in inter-eNB HO measurements |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The inter-eNB measurements include the forwarded HOs for RN which are not direclty related to the DeNB performance, so that the DeNB HO performance can not be explicitly known. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.3.1.2, Annex A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 4.3.1.2	Inter-eNB Handover related measurements

##### 4.3.1.2.1	Attempted outgoing inter-eNB handover preparations

- This measurement provides the number of attempted outgoing inter-eNB handover preparations, the forwarded handovers for RN in DeNB are exclusive.

- CC.

- Transmission of the X2AP message HANDOVER REQUEST from the source eNB to the target eNB (see TS 36.423[10]), indicating the attempt of an outgoing inter-eNB handover preparation or on transmission of S1AP message HANDOVER REQUIRED to the MME (see TS 36.413 [9]), the forwarded X2AP message HANDOVER REQUEST and S1AP message HANDOVER REQUIRED for RN in DeNB are exclusive.

- A single integer value.

- HO.InterEnbOutPrepAtt

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

##### 4.3.1.2.2	Attempted outgoing inter-eNB handover executions per handover cause

- This measurement provides the number of attempted outgoing inter-eNB handovers per handover cause.

- CC.

- Transmission of the RRC ConnectionReconfiguration message to UE triggering the handover from the source eNB to the target eNB, indicatingthe attempt of an outgoing inter-eNB handover (see TS 36.331 [8]).
The sum of all supported per cause measurements shall equal the total number of outgoing inter-eNB handover event Each RRCConnectionReconfiguration message transimtted is added to the relevant per handover cause measurement, the possible causes are included in TS 36.413 [9]. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- HO.InterEnbOutAtt.Cause
where Cause identifies the cause for handover

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

##### 4.3.1.2.3	Successful outgoing inter-eNB handover executions per handover cause

- This measurement provides the number of successful outgoing inter-eNB handovers per handover cause, the forwarded handovers for RN in DeNB are exclusive.

- CC.

- Receipt at the source eNB of UE CONTEXT RELEASE [10] over the X2 from the target eNB following a successful handover, or if handover is performed via S1, receipt of UE CONTEXT RELEASE COMMAND [9] at the source eNB following a successful handover, the forwarded X2AP UE CONTEXT RELEASE message and S1AP UE CONTEXT RELEASE COMMAND message for RN in DeNB are exclusive. Each X2AP UE CONTEXT RELEASE message or S1AP UE CONTEXT RELEASE COMMAND message received and counted is added to the relevant per handover cause measurement, the possible causes are included in TS 36.413 [9].
The sum of all supported per cause measurements shall equal the total number of outgoing inter-eNB handover events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- HO.InterEnbOutSucc.Cause
where Cause identifies the cause for handover.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

Next Modified Section

# A.4	Overview handover related Use Cases

| Use Case | PM KPI / elementary object |
| --- | --- |
| Continuous Network Supervision: Supervision of overall handover performance. It is essential in network operations to follow the success rate of various handover. Low handover success rate will impact user experience, therefore it is important to define measurements to follow handover success rate. | - outgoing Intra RAT HO Success Rate (cell) *1 - outgoing Inter RAT HO Success Rate (cell) *1 - outgoing Inter System HO Success Rate (cell) *1 *3 - outgoing Intra Frequency HO Success Rate (cell) *1 - outgoing Inter Frequency HO Success Rate (cell) *1 - outgoing Intra eNB HO Success Rate (cell) *2 - outgoing Inter eNB HO Success Rate (cell) *2 *2.2 *1: It is expected that the HO success rate may vary depending on the respective scenarios : intra-RAT, inter-RAT, inter System, intra frequency, inter frequency *2: it is expected that the HO success rate may vary depending on the used external interfaces *2.2: For the handover from the DeNB to another eNB/DeNB, the forwarded handovers for RN in the source DeNB shall not be counted as these handovers are controlled by RN so cannot directly reflect DeNB handover performance. *3: inter system : LTE- non 3GPP HO |
| Troubleshooting: Detection of bad handover relation. The first use case provides the overall performance of handover success rate on E-UTRAN cell level, but it is essential to get a knowledge between which cell pairs the handover success rate is low. Therefore it is important to know the success rate on neighbor cell relation basis. | - HO Success Rate (neighbourcell) |
| Troubleshooting: Reason for started handover To go for further analysis of handover failures, it is essential to know what causes the handovers. For this we need to know the success rate of handovers per HO reason. | - outgoing HO Success Rate per HO reason ( neighbor cell) *4 *4 different results expected e.g. emergency or normal HO |
| Troubleshooting: Reason for failed handover. To go for further detailed analysis for handover failure it is important to know what the reason for handover failure was, or whether the handover was assisted by measurement gaps or was with DRX. It is also important to know if measurement gaps and DRX are helping in handover procedure or not. (i.e. what is the handover failure rate if measurement gaps are switched on. Measurement gaps and DRX can cause more load and battery consumption to the UE, therefore if these are not causing any changes in handover failure rate, operators may not use them) | - outgoing HO Failure Distribution Rate (cell+neighbourcell) - HO Path Switching Failure Distribution Rate (cell or Interface) - HO Failure Rate DRX / Non DRX (cell) *5 - Inter frequency HO Failure Rate Meas gap assisted / not assisted (cell) *5 *5: measurement only on cell basis and not per neighbourcell due to amount of counters as mentioned above. |
| Network Planning: Traffic flow analysis or Network Planning: Handover traffic optimization | - outgoing Intra RAT HO Success Rate (neighbour cell) - outgoing Inter RAT HO Success Rate (cell) - outgoing Inter System HO Success Rate (cell) |

End of modifications