3GPP TSG-SA5 (Telecom Management)	S5-111377

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111169

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.425 | CR | 0058 |  rev | 1 |  Current version: | 10.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add RRC connection setup and UE context release related measurements of RN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | The use case A.1 also applies to the RN, but the corresponding RRC connection related measurements for RN are missing. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add RRC connection related measurements of RN |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The performance of RRC connection in RN cannot be known |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.1.4.1, 4.1.4.2, 4.1.5.1, 4.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 4.1.4	RRC connection setup time

#### 4.1.4.1	Mean RRC connection setup time

- This measurement provides the mean time per establishment cause it takes to establish an RRC connection.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for every successful RRC connection establishment between the receipt of a RRCConnectionRequest  and the corresponding RRCConnectionSetupComplete  message by the eNodeB/RN over the granularity period. The end value of this time will then be divided by the number of successful RRC connections observed in the granularity period to give the arithmetic mean. The accumulator shall be reinitialised at the beginning of each granularity period. The measurement is split into subcounters per establishment cause, and the possible causes are included in TS 36.331 [8].

- Each measurement is an integer value (in milliseconds).

- The measurement name has the form RRC.ConnEstabTimeMean.Cause
where Cause identifies the establishment cause

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switching.

- EPS

#### 4.1.4.2	Maximum RRC connection setup time

- This measurement provides the maximum time per establishment cause it takes to establish an RRC connection.

- GAUGE.

- This measurement is obtained by monitoring the time intervals for each successful RRC connection establishment between the receipt of a RRCConnectionRequest and the corresponding RRCConnectionSetupComplete message by the eNodeB/RN over the granularity period. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period. The measurement is split into subcounters per establishment cause, and the possible causes are included in TS 36.331 [8].

- Each measurement is an integer value (in milliseconds).

- The measurement name has the form RRC.ConnEstabTimeMax.Cause
where Cause identifies the establishment cause

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switching.

- EPS

### 4.1.5	UE CONTEXT Release

#### 4.1.5.1	Number of UE CONTEXT Release Request initiated by eNodeB/RN

- This measurement provides the number of UE CONTEXT Release initiated by eNB/RN for each release cause.

- CC.

- Transmission of an UE CONTEXT RELEASE REQUEST message initiated by eNodeB/RN. Each release request is to be added to the relevant cause measurement. The possible causes are defined in 36.413 [9]. The sum of all supported per causes measurements shall equal the total number of UE CONTEXT Release initiated by eNodeB/RN. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form UECNTX.RelReq.Cause 
where Cause identifies the release cause.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switching.

- EPS

- By differenciate the causes, this measurement is used to count “The number of abnormal RRC connection release related to load”, which can be used for LBO target calculation..

#### 4.1.5.2	Successful UE CONTEXT Release

- This measurement provides the number of successful UE Context Release.

- CC.

- Sending of UE CONTEXT RELEASE COMPLETE from eNB/RN to MME/DeNB

- A single integer value.

- UEContext.RelSuccNbr.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switching.

- EPS

- This measurement can be used to count “the total number of RRC connection release”, which can be used for LBO target calculation.

End of modifications