3GPP TSG-SA5 (Telecom Management)	S5-111499

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	rev of S5-111385

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.425 | CR | 0065 |  rev | 2 |  Current version: | 10.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Adding separate counters for Un link and Uu link L2 measurements in DeNB. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | With the introducion of Relay architecture the L2 measurements cannot be differentatied at the DeNB for Un and Uu link however some of those measurements should be differentiated. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The L2 measurements are splitted to have separate counters for Un and Uu link. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.4.2.1, 4.4.2.2, 4.4.3.1, .4.4.4.1, 4.4.4.2, 4.4.6.1, 4.4.6.2, 4.5.1, 4.5.2, 4.5.3, 4.5.4, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

### 4.4.2	Active UEs

#### 4.4.2.1	Average number of active UEs on the DL

- This measurement provides the average number of UEs that have DTCH data queued on the downlink. The measurement is split into subcounters per E-RAB QoS level (QCI). In case of DeNB the measurement shows the number of active UEs connected directly to the DeNB.

- SI

- This measurement is obtained according to the definition in 3GPP TS 36.314 [11]. Separate counters are maintained for each QCI.

- Each measurement is an integer value. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form 
DRB.UEActiveDl.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

#### 4.4.2.2	Average number of active UEs on the UL

- This measurement provides the average number of UEs that have DTCH data queued on the uplink. The measurement is split into subcounters per E-RAB QoS level (QCI). ). In case of DeNB the measurement shows the number of active UEs connected directly to the DeNB.

- SI

- This measurement is obtained according to the definition in 3GPP TS 36.314 [11]. Separate counters are maintained for each QCI.

- Each measurement is an integer value. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form 
DRB.UEActiveUl.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

### 4.4.3	Packet Delay and Drop Rate

#### 4.4.3.1	Average DL PDCP SDU delay

- This measurement provides the average (arithmetic mean) PDCP SDU delay on the downlink.  The measurement is split into subcounters per E-RAB QoS level (QCI). In case of DeNB the measurement provides separate counter for the Un link and Uu link.

- DER (n=1)

- This measurement is obtained according to the definition in 3GPP TS 36.314 [11].

- Each measurement is an integer value representing the mean delay in ms. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form 
DRB.PdcpSduDelayDl.QCI
DRB.PdcpSduDelayDlUn.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

### 4.4.4	Packet loss rate

#### 4.4.4.1	DL PDCP SDU air interface loss rate

- This measurement provides the fraction of IP packets (PDCP SDUs) which are lost (not successfully transmitted) on the downlink air interface.  Only user-plane traffic (DTCH) is considered.  A lost packet is one whose context is removed from the eNodeB after an attempt has been made to transmit part or all of the packet on the air interface but the whole packet has not been successfully transmitted.  The measurement is split into subcounters per E-RAB QoS level (QCI). In case of DeNB the measurement provides the packet loss rate for Uu link only.

- SI

- This measurement is obtained according to the definition in 3GPP TS 36.314 [11]. Separate counters are maintained for each QCI.  In case only a subset of per QCI measurements is supported, a loss rate subcounter calculated across all QCIs will be provided first.

- Each measurement is an integer value representing the air interface loss rate multiplied by 1E6. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form 
DRB. PdcpSduAirLossRateDl.QCI
where QCI identifies the target E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

#### 4.4.4.2	UL PDCP SDU loss rate

- This measurement provides the fraction of IP packets (PDCP SDUs) which are lost (not successfully received) on the uplink.  Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered.  The measurement is split into subcounters per E-RAB QoS level (QCI). In case of DeNB the measurement provides the packet loss rate for Uu link only (i.e. Un link is excluded). The measurement is also applicable to RN.

- SI

- This measurement is obtained according to the definition in 3GPP TS 36.314 [11]. Separate counters are maintained for each QCI.  In case only a subset of per QCI measurements is supported, a loss rate subcounter calculated across all QCIs will be provided first.

- Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form 
DRB.PdcpSduLossRateUl.QCI
where QCI identifies the target E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

- NEXT MODIFICATION

### 4.4.6	IP Throughput measurements

#### 4.4.6.1	IP Throughput in DL

- This measurement provides IP throughput in downlink. In case of DeNB the measurement provides the throughput of Uu link only. The measurement is also applicable to RN.

- DER(N=1)

- This measurement is obtained according to the definition “Scheduled IP Throughput” in 3GPP TS 36.314 [11]. Separate counters are maintained for each QCI.

- Each measurement is a real value representing the throughput in kbits/s. The number of measurements is equal to the number of QCIs.

- The measurement name has the form 
DRB.IPThpDl.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

- This measurement is to support the Integrity KPI “E-UTRAN IP Throughput” defined in [13].

#### 4.4.6.2	IP Throughput in UL

- This measurement provides IP throughput in uplink. In case of DeNB the measurement provides the throughput of Uu link only (i.e. Un link throughput is excluded). The measurement is also applicable to RN.

- DER(N=1)

- This measurement is obtained according to the definition “Scheduled IP Throughput” in 3GPP TS 36.314 [11]. Separate counters are maintained for each QCI.

- Each measurement is a real value representing the volume in kbits. The number of measurements is equal to the number of QCIs.

- The measurement name has the form 
DRB.IPThpUl.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

- This measurement is to support the Integrity KPI “E-UTRAN IP Throughput” defined in [13].

- NEXT MODIFICATION

## 4.5	Radio resource utilization related measurements

### 4.5.1	DL PRB Usage for traffic

- This measurement provides the usage (in percentage) of physical resource blocks (PRBs) on the downlink for DTCH traffic.  The measurement is split into subcounters per E-RAB QoS level (QCI). The measurement provides separate counters for Un and non-Un subframes in case of DeNB.

- SI

- This measurement is obtained according to the definition in 3GPP TS 36.314 [11]. Separate counters are maintained for each QCI.  The sum of all supported per QCI measurements shall equal the total PRB usage for DTCH.  In case only a subset of per QCI measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value from 0 to 100. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form 
RRU.PrbDl.QCI
RRU.PrbDlUn.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

### 4.5.2	UL PRB Usage for traffic

- This measurement provides the usage (in percentage) of physical resource blocks (PRBs) on the uplink for DTCH traffic.  The measurement is split into subcounters per E-RAB QoS level (QCI). The measurement provides separate counters for Un and non-Un subframes in case of DeNB.

- SI

- This measurement is obtained according to the definition in 3GPP TS 36.314 [11]. Separate counters are maintained for each QCI.  The sum of all supported per QCI measurements shall equal the total PRB usage for DTCH.  In case only a subset of per QCI measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value from 0 to 100. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form 
RRU.PrbUl.QCI
RRU.PrbUlUn.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

### 4.5.3	DL Total PRB Usage

- This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink for any purpose. The measurement provides separate counters for Un and non-Un subframes in case of DeNB.

- SI

- This measurement is obtained according to the definition in 3GPP TS 36.314 [11].

- A single integer value from 0 to 100.

- RRU.PrbTotDl
RRU.PrbTotDlUn

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

### 4.5.4	UL Total PRB Usage

- This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink for any purpose. The measurement provides separate counters for Un and non-Un subframes in case of DeNB.

- SI

- This measurement is obtained according to the definition in 3GPP TS 36.314 [11].

- A single integer value from 0 to 100.

- RRU.PrbTotUl
RRU.PrbTotUlUn

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

- NEXT MODIFICATION

# A.5	Monitor of cell level QoS and radio resource utilisation

In an E-UTRAN cell the quality of service achieved is directly influenced by a number of factors, including:

- Loading of users on the cell

- Traffic loading and characteristics

- UE locations and mobility

- RRM policies

  - Scheduling

  - congestion control

  - admission control

  - layer 2 protocol configuration

- Mapping of traffic to QCI

- Setting of QoS parameters other than the QCI.

It is very important to be able to monitor the QoS to determine whether the combined effect of these policies, algorithms and external factors is satisfactory.  Unsatisfactory QoS may rectified by adjusting policies and RRM settings, for instance.

Cell bit-rate

A fundamental measure of QoS is the throughput (data rate) of the cell.  The total cell throughput measured across all radio bearers gives an indication of the loading and activity in the cell.  Adding a per QCI counter allows the loading on the different QCIs to be measured.  For example, if QCI 1 is used exclusively for VoIP then the loading of conversational speech can be directly determined.  Finally, the maximum throughput can indicate to the operator whether there is enough capacity in the network; for example, is the backhaul sufficient.  Separate counters should be configured on the downlink and uplink. Complexity may be reduced by performing the counters at layer 3, giving the ingress bit-rate to the eNB on the downlink and the egress bit-rate from the eNB on the uplink.

Cell throughput includes both User Plane data and Control Plane data. To support the User Plane data, necessary Control Plane data also need to be transmitted. This Control Plane data although required, will not be perceived (felt) by the User. The total cell throughput helps to evaluate the usage of bandwidth and radio resource.

Operators ideally want to see the Control Plane data as small as possible when compared to the User Plane data without compromising on the service.

Hence it is important to monitor the total cell throughput as well as how much is occupied by Control Plane Data.

Number of actives UEs

It is also of interest to determine how many users are enjoying the throughput numbers determined for each QCI.  Therefore, we may count the number of users that are active for each QCI – here active users have data queued pending transmission.  A simple division of the throughput (data rate) of a QCI by the number of active users on the QCI indicates the throughput per user on the QCI.  For example, taking QCI 1 this metric could indicate the typical codec rate being employed in the cell.  Alternatively, for QCI 9 supporting low priority TCP-based traffic it can indicate the typical bandwidth pipe size for a user when he has data to send / receive.

In case of DeNB the users can be connected to the eNB directly via Uu interface or connected via RN. Without having separate measurement for active UEs on Uu and users connected to RN would not provide information on the throughput per user at DeNB as on Un interface there can be many users connected via RN. Therefore it is beneficial to provide separatly the number of active users on Uu interfaces and have the same measurement also in RN. Also it is required to have separate measurements for throughput on Uu interface in DeNB and also in the RN.

DL packet delay

Latency is of prime concern for some services, particularly conversational services like speech and instant messaging.  A counter is added to measure the mean delay for IP packets incurred within the eNodeB.  Separate counters are provided per QCI which are particularly useful when the QCI is used by very few services and the packet sizes vary little.  It is only practical to measure packet delays on the downlink.

In case of DeNB the packet delay includes both the internal processing delay at DeNB and UE/RN as well as the packet transferring delay on the radio link. As RN UE’s packets need to be transferred via Un and Uu while packets for UEs directly connected to DeNB need to pass through only Un interface, packet delay optimization mechanism may be different for RN UEs and DeNB UEs.  Therefore it is beneficial to have separate measurements on packet delay for Uu and Un interface.

DL packet drop rate

When a cell is heavily loaded congestion can take place.  When congestion is not severe the impact is typically the incurrence of additional delay for non-GBR radio bearers.  However, when congestion is severe the eNodeB may be forced to discard packets.  It is important for the operator to have visibility of packet discard so that corrective action can be instigated (for example, by adjusting admission control settings in the network).  It is only practical to measure packet discards on the downlink.  Packet discards on handover should not be included in the count.

PRB Usage

The resource utilisation, measured in terms of physical resource blocks (PRBs), is a useful measure of whether a cell is lightly loaded or not.  Loading is a key input to network capacity planning and load balancing.  Furthermore, when resource utilisation per QCI is reported the distribution of resources between different services can be estimated.

For Type 1 relay, the Un subframe configuration is necessary for RN operation. The RN can only be scheduled by the DeNB within subframes configured for RN backhaul communication (i.e, Un subframes), while, in principle, the UEs connected directly to DeNB can be scheduled in any subframe. Therefore, in certain scenarios, it may be possible that the PRB usage in the Un subframes and in the non-Un subframes is quite different. For example, in case there are many RNs in the network and only few UEs connected directly to the DeNB, it may happen that the PRB usage in the Un subframes is quite high while the PRB usage in non-Un subframes is low.Therefore it is seen useful to have separate measurements on PRB usage for Un subframe and non-Un subframes.

Downlink Air interface packet loss rate

The downlink air interface packet loss can be directly compared with the PELR value of a QCI to see if the packet loss (over the air interface) aspect of quality of service is being met within the cell (see [12] for more details on PELR).  On the downlink this measurement can be added to the congestion losses (see DL packet drop rate) to determine the total packet loss rate at the eNodeB.  Consequently, the downlink useful bit-rate can be estimated by scaling the measurement of the downlink PDCP ingress bit-rate by (1 – DL packet drop rate) (1 – air interface packet loss rate).

Uplink packet loss rate

The uplink air interface packet loss rate (per QCI) can be compared directly with the PELR defined for that QCI.  An estimate of the uplink air interface packet loss may be provided by the “Uplink PDCP SDU loss rate”.  This uplink measurement is based on PDCP sequence numbers and cannot precisely measure the air interface losses.  Any packets discarded by the UE within the protocol stack (i.e. at layer 2) are also counted since they will have been given a PDCP sequence number.  Discards at layer 3 are not counted.

RACH Usage

The RACH plays a vital role in the following procedures:

-	Initial access from RRC_IDLE;

-	Initial access after radio link failure;

-	Handover requiring random access procedure;

-	DL data arrival during RRC_CONNECTED requiring random access procedure;

-	UL data arrival during RRC_CONNECTED requiring random access procedure;

Furthermore, the random access procedure takes two distinct forms:

-	Contention based using a randomly selected preamble (applicable to all five events);

-	Non-contention based using a dedicated preamble (applicable to only handover and DL data arrival).

In the use-case of RACH configuration optimization, received Random Access Preambles are signaled across an OAM interface.

Monitoring of the preamble usage in a cell allows the operator to determine if the resources allocated to the RACH by the eNodeB are appropriate for the number of random access attempts.  If the resources are underutilised then the operator may reconfigure the eNodeB (via CM) to allocate less resource to RACH thereby freeing up resource for other uplink transmissions.  Alternatively, if the resources are heavily utilised then this is indicative of RACH congestion leading to increased latency for the procedures listed above.

The eNodeB can partition the RACH resource between dedicated preambles, randomly selected preambles in group A and randomly selected preambles in group B.  This partitioning can be evaluated when usage measurements are made on each set separately.