3GPP TSG-SA5 (Telecom Management)	S5-111502

SA5#76, 28 Feb - 4 Mar 2011; San Diego, USA	revision of S5-111373

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.762 | CR | 0053 |  rev | 2 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correct Relay and Donor eNodeB model in E-UTRAN Network Resource Model IRP Information Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | China Mobile, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | The Donor eNodeB (DeNB) is a functionality of the ordinary eNodeB which could be activated and later also deactivated (either from first installation/startup of the node or at a later stage). In addition, when this functionality is activated, the eNodeB needs to keep the cells (instances) that it had before the activation occurred. The current model would create a problem in such a situation - forcing the manager to delete the ENBFunction instance and all its contained cells, and then recreate all cells under a new DENBFunction instance, thereby losing all earlier cell data (which would impact ongoing traffic) and changing all cell DNs. Moreover, the associations between served Relay nodes and supporting DeNB cell are missing, both for ordinary IOCs within the same IRPAgent as well as for external IOCs. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Model the DeNB functionality as a new IOC “DeNBCapability” contained by ENBFunction. Add missing associations and constraints, and add maxNbrRNAllowed attribute to DeNBCapability. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The activation of the DeNB capability within an operating eNodeB would not work properly, in particular, all existing cells would have to be deleted. Necessary relations between the DeNB and RN for configuration and fault management would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 3.2, 6.2.1, 6.2.2, 6.3.18.2, 6.3.19.1, 6.3.19.2, 6.3.20, 6.3.20.1, 6.3.20.2, 6.3.21, , 6.4.X (new), 6.4.Y (new), 6.4.Z (new), 6.4.W (new), 6.5.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.766 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First Modified Section

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TS 32.150 [16], TS 32.101 [1], TS 32.102 [2] and TS 21.905 [17] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TS 32.150 [16], TS 32.101 [1], TS 32.102 [2] and TS 21.905 [17], in that order.

DeNB	Donor eNodeB

DN	Distinguished Name (see 3GPP TS 32.300 [4])

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

ME	Managed Element

MO	Managed Object

MBSFN		Multimedia Broadcast multicast service Single Frequency Network

NR	Neighbour cell Relation

PM	Performance Management

RDN	Relative Distinguished Name (see 3GPP TS 32.300 [4])

RN	Relay Node

Next Modified Section

### 6.2.1	Attributes and relationships

…

Note 1:

If an instance of the ServesRN association is present, then a corresponding instance of ServedByEGC must be present.  In this case, the ENBFunction and RNFunction instances are under the management scope of the same IRPAgent.

If an instance of the ServesExtRN association is present, then a corresponding instance of ServedByExtEGC must be present.  In this case, the ENBFunction and RNFunction instances are under the management scope of two different IRPAgents.

Note 2: The modelling of the DeNB capability as a separate IOC or as attributes of ENBFunction is FFS

Figure 6.2.1.2: E-UTRAN relaying view of E-UTRAN NRM

Next Modified Section

### 6.2.2	Inheritance

…

Figure 6.2.2.1: E-UTRAN NRM Inheritance Hierarchy

Next Modified Section

### 6.3.18	RNFunction

#### 6.3.18.1	Definition

This IOC represents Relay Node (RN) functionality. For more information about RN, see 3GPP TS 36.300 [11].

#### Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| servingCell | M | M | M |

#### 6.3.18.3			Attribute constraints

None.

#### 6.3.18.4	Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.19	ExternalRNFunction

#### 6.3.19.1	Definition

This IOC represents the properties of a Relay Node (RN) controlled by another IRPAgent.. For more information about RN, see 3GPP TS 36.300 [11].

#### Attributes

None.

#### 6.3.19.3	Attribute constraints

None.

#### 6.3.19.4	Notifications

The common notifications defined in subclause 6.6.2 are valid for this IOC, without exceptions or additions.

### 6.3.20	DeNBCapability

#### 6.3.20.1	Definition

This IOC represents the capability for an eNodeB to act as a Donor eNodeB (DeNB). For more information about the DeNB, see 3GPP TS 36.300 [11].

#### 6.3.20.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| servedRN | M | M | M |
| maxNbrRNAllowed | M | M | M |

#### 6.3.20.3			Attribute constraints

None

#### 6.3.20.4	Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.21	Void

Next Modified Section

### 6.4.X	ServesRN (O)

#### 6.4.X.1	Definition

This unidirectional association represents the relation between a DeNB (represented by an ENBFunction containing a DeNBCapability) and one or more served RNFunction instances.

#### 6.4.X.2	Roles

| Name | Definition |
| --- | --- |
| servedRN | This role represents the RN instance served by a DeNB instance . |

#### 6.4.X.3	Constraints

None.

### 6.4.Y	ServesExtRN (O)

#### 6.4.Y.1	Definition

This unidirectional association represents the relation between a DeNB (represented by an ENBFunction containing a DeNBCapability) and one or more served ExternalRNFunction instances.

#### 6.4.Y.2	Roles

| Name | Definition |
| --- | --- |
| servedRN | This role represents the external RN instance served by a DeNB instance . |

#### 6.4.Y.3	Constraints

None.

### 6.4.Z	ServedByEGC (O)

#### 6.4.Z.1	Definition

This unidirectional association represents the relation between one or more RNs and their serving DeNB cell.

#### 6.4.Z.2	Roles

| Name | Definition |
| --- | --- |
| servingCell | This role represents the cell serving one or more RNFunction instances . |

#### 6.4.Z.3	Constraints

Associations ServedByEGC and ServedByExtEGC are mutually exclusive.

### 6.4.W	ServedByExtEGC (O)

#### 6.4.W.1	Definition

This unidirectional association represents the relation between one or more RNs and their external serving DeNB cell (under another IRPAgent).

#### 6.4.W.2	Roles

| Name | Definition |
| --- | --- |
| servingCell | This role represents the external cell serving one or more RNFunction instances . |

#### 6.4.W.3	Constraints

Associations ServedByEGC and ServedByExtEGC are mutually exclusive.

Next Modified Section

## 6.5	Information attribute definitions

### 6.5.1	Definition and legal values

Table 6.5.1.1 defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

Table 6.5.1.1: Attributes definitions and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| adjacentCell | This attribute contains the DN of a EUtranGenericCell or ExternalEUtranGenericCell. |  |
| adjacentSector | This attribute contains the DN of an ExternalSector. |  |
| relatedAntennaList | This attribute contains the DNs of one or more AntennaFunction |  |
| relatedTmaList | This attribute contains the DNs of one or more TmaFunction |  |
| relatedEUtranCellList | This attribute contains the DNs of one or more EUtranGenericCell |  |
| cellLocalId | Unambiguously identify a cell within an eNodeB | 0 - 255. |
| cellIdList | This holds a list of DN of EUtranGenericCell. These cells all belong to one MBSFN Area. |  |
| cellSize | See cell-Size in TS 36.423 [24]. | See cell-Size in TS 36.423 [24]. |
| cellResvInfo | This attribute represents whether the cell is MBSFN Area Reserved Cell or not. See TS 36.300[11] for MBSFN Area Reserved Cell. | See 3GPP TS 36.443 [28] for Cell Reservation Info. |
| confOutputPower | It defines the allowed total power to use for all cells together in this sector. It may be set by the operator and/or limited by HW limitation or licensed power, e.g.: 20, 40, 60, 80,120 watts |  |
| relatedSector | This attribute contains the DN of an SectorEquipmentFunction |  |
| referenceSignalPower | This defines the cell specific downlink reference signal transmit power, which is described in 3GPP TS 36.213[25] | See 3GPP TS 36.331[10] |
| earfcn | It is the frequency number for the central frequency. See 3GPP TS 36.104[14]. | See 3GPP TS 36.104[14]. |
| earfcnul | Specifies the channel number for the central UL frequency. The mapping from channel number to physical frequency is described in 3GPP specification TS 36.101 [13] subclause 5.7.3. | See EARFCN in TS 36.101 [13] subclause 5.7.3. |
| earfcnDl | Specifies the channel number for the central DL frequency. The mapping from channel number to physical frequency is described in 3GPP specification TS 36.101 [13] subclause 5.7.3. | See EARFCN in TS 36.101 [13] subclause 5.7.3 |
| energySavingControl | This attribute allows to IRPManager to initiate energy saving activation or deactivation. Its value can not be changed by the IRPAgent. | Enumerated {energySaving, notEnergySaving}. |
| energySavingState | Specifies the status regarding the energy saving in the cell. If the value of energySavingControl is energySaving, then it shall be tried to achieve the value isEnergySaving for the energySavingState. If the value of energySavingControl is notEnergySaving, then it shall be tried to achieve the value isNotEnergySaving for the energySavingState. | Enumerated {isNotEnergySaving, isEnergySaving}. |
| eNBId | Unambiguously identifies an eNodeB within a PLMN | See 3GPP TS 36.413[27], 36.300[4] |
| fqBand | This is the frequency band supported by the hardware associated with the SectorEquipmentFunction. The earfcnDl and earfcnUl of cells associated with the SectorEquipmentFunction must be assigned with value within this fqBand value. | See section 5 Table 5.2-1 "E-UTRA frequency band" of TS 36.104 [14]. Other legal values would be applicable for other technologies such as for UTRA. |
| id | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| isESCoveredBy | The value of the attribute is configured by the IRPmanager and is not changed by the IRPAgent. It indicates whether the adjacentCell according to this planning provides no, partial or full coverage for the cell which name-contains the EUtranRelation instance. Adjacent cells with this attribute equal to “yes” are recommended to be considered as candidate cells to take over the coverage when the original cell is about to be transferred to energySaving state. The entirety of adjacent cells with this property equal to “partial” are recommended to be considered as entirety of candidate cells to take over the coverage when the original cell is about to be transferred to energySaving state. The value “partial” is not allowed in an eNB overlaid scenario. | No, partial, yes |
| isRemoveAllowed (see note 1) | This indicates if the subject EUtranRelation can be removed (deleted) or not. If ‘yes’, the subject EUtranRelation instance can be removed (deleted). If ‘no’, the subject EUtranRelation instance shall not be removed (deleted) by any entity but an IRPManager. | yes, no |
| isHOAllowed (see note 1) | This indicates if HO is allowed or prohibited. If ‘yes’, handover is allowed from source cell to target cell. The source cell is identified by the name-containing EUtranGenericCell of the EUtranRelation that has the isHOAllowed. The target cell is referenced by the EUtranRelation that has this isHOAllowed. If ‘no’, handover shall not be allowed. | yes, no |
| isICICInformationSendAllowed | This indicates if ICIC (Inter Cell Interference Coordination) load information message (see TS 36.423 [24] Section 9.1.2.1 LOAD INFORMATION) sending is allowed or prohibited. If ‘yes’, ICIC load information message sending is allowed from source cell to target cell. The source cell is identified by the name-containing EUtranGenericCell of the EUtranRelation that has the isICICInformationSendAllowed. The target cell is referenced by the EUtranRelation that has this isICICInformationSendAllowed. If ‘no’, ICIC load information message sending shall not be allowed. | yes, no |
| isLBAllowed | This indicates if load balancing is allowed or prohibited from source cell to target cell. If ‘yes’, load balancing is allowed from source cell to target cell. The source cell is identified by the name-containing EUtranGenericCell of the EUtranRelation that has the isLBAllowed. The target cell is referenced by the EUtranRelation that has this isLBAllowed. If ‘no’, load balancing shall be prohibited from source cell to target cell. | yes, no |
| maximumTransmissionPower | This is the maximum possible for all downlink channels, used simultaneously in a cell, added together. |  |
| mbsfnAreaId | This is the identifier of MBSFN Area. See TS 36.300[11] for MBSFN Area. | See 3GPP TS 36.443 [28] for mbsfnAreaId |
| partOfSectorPower | This is the requested part (i.e. %) of the total radio power available to the SectorEquipmentFunction. The requested % power should be allocated to the cell. | 0 : 100 |
| pb | PB, which is described in Section 5.2 of TS 36.213 [25] | See 3GPP TS 36.213[25] |
| pci | This holds the Physical Cell Identity (PCI) of the cell (for NM-Centralized, EM-Centralized and Distributed PCI assignment cases). In the case of NM-Centralized PCI assignment, see TS 36.300, [11] subclause 22.3.5, IRPManager signals a specific value by writing this attribute. | See TS 36.211 [12] subclause 6.11 for legal values of pci. |
| pciList | This holds a list of physical cell identities that can be assigned to the pci attribute by eNB. The assignment algorithm is not specified. This attribute shall be supported if and only if the EM-Centralized or Distributed PCI Assignment is supported. See TS 32.500, ref [15] subclause 6.1.6. | See TS 36.211 [12] subclause 6.11 for legal values of pci. The number of pci in the list is 1 to 504. |
| plmnIdList | List of unique identities for PLMN. Note: A cell can broadcast up to 6 PLMN-id's. This is to support the case that one cell can be used by up to 6 operators’ core networks. One member of plmnIdList is the primary PLMN Id. See TS 36.331 [10] section 6.2.2: SystemInformationBlockType1/cellAccessRelatedInformation/plmn-IdentityList is a SEQUENCE (SIZE (1..6)) | A list of at most six entries of PLMN Identifiers. The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC). See TS 23.003 [3] subclause 2.2 and 12.1. |
| servedRN | This attribute contains the DNs of one or more associated instances of RNFunction and ExternalRNFunction. |  |
| servingCell | This attribute contains the DN of one associated instance of EutranGenericCell or ExternalEutranGenericCell. |  |
| maxNbrRNAllowed | This is an integer indicating the maximum number of RNs allowed to be connected. It is a number which can be configured by the operator to control the node/network load. |  |
| sfAssignment | This is the uplink-downlink subframe configuration number of a TDD E-UTRAN cell. | See 3GPP TS 36.211[12]. |
| specialSfPatterns | This is the special subframe configuration number of a TDD E-UTRAN cell. | See 3GPP TS 36.211[12]. |
| tac | Common Tracking Area Code for the PLMNs. The identity used to identify tracking areas. | - It is the Tracking Area Code (TAC). - A cell can only broadcast one TAC. See TS 36.300 [11], section 10.1.7 (PLMNID and TAC relation). - TAC is defined in TS 23.003 [3], section 19.4.2.3. |
| tCI | This is the Target Cell Identifier. It consists of E-UTRAN Cell Global Identifier (ECGI) and Physical Cell Identifier (PCI) of the target cell. The EUtranRelation.tCI identifies the target cell from the perspective of the EUtranGenericCell, the name-containing instance of the subject EUtranRelation instance. | The Target Cell Identifier is defined in TS 36.300 [11]. See TS 36.211 [12] subclause 6.11 for legal values of the PCI. |
| x2BlackList | This is a list of DNs of ENBFunction and ExternalENBFunction. If the target node DN is a member of the source node’s ENBFunction.x2BlackList, the source node is: - Prohibited from sending X2 connection request to target node; - Forced to tear down established X2 connection to target node - Not allowed to accept incoming X2 connection request from target node. The same DN may appear here and in ENBFunction.x2WhiteList. In such case, the DN in x2WhiteList shall be treated as if it is absent. |  |
| x2IpAddressList | Represents one or more IP addresses used by ENBFunction for this ENBFunction’s X2 Interface | One or more IPv4 or IPv6 addresses |
| x2WhiteList | This is a list of DNs of ENBFunction and ExternalENBFunction. If the target node DN is a member of the source node’s ENBFunction.x2WhiteList, the source node : - Is allowed to request the establishment of X2 connection with the target node; - Is not allowed to initiate the tear down of established X2 connection to target node The same DN may appear here and in ENBFunction.x2BlackList. In such case, the DN here shall be treated as if it is absent. |  |
| x2HOBlackList | This is a list of DNs of ENBFunction. The ENBFunction.x2HOBlackList identifies a list of neighbour ENBFunction with whom the subject ENBFunction is prohibited to use X2 interface for HOs even if the X2 interface exists between them. |  |
| allowedAccessClasses | This holds information for access classes (10-15) – [3GPP TS 22.011] that are allowed for the eUTRANCell . The access classes are: Class 10 – emergency call Class 11 - For PLMN Use. Class 12 - Security Services; Class 13 - Public Utilities (e.g. water/gas suppliers); Class 14 - Emergency Services; Class 15 - PLMN Staff; | The default value is all access classes are allowed See TS 22.011 [29] and 36.331 [10] for more details on the definition and SIB2 broadcast message definition |

End of document