3GPP TSG-SA5 (Telecom Management)	S5-111506

SA5#76, 28 Feb- 4 Mar 2011; San Diego, USA	revision of S5-111398

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.582 | CR | 0008 |  rev | 1 |  Current version: | 10.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add new Home NodeB configuration parameters related to neighbor cell list, coverage edge quality, and noise rise |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 04/03/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | This CR introduces the parameters to Home NodeB data model in 32.582. This reflects the changes to the HNB data model submitted to the Boradband Forum. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Addition of new neighbour cell, coverage edge quality and noise rise parameters. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The changes to the HNB data model in the BBF specifications would not be reflected in TS 32.582. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 6	Information Model Definition

## 6.1	Configuration Management

NOTE:	State Management (Operational Status, Administrative status of the device) is covered as part of configuration management

### 6.1.1	HNB Access Network

Table 6.1.1: HNB Access Network Related Parameter Types

| Parameter Type | Description | Reference |
| --- | --- | --- |
| HNB-GW Gateway Identity | Identity of the HNB-GW the HNB connects to. The type of identity may be either “name” or IP address. | 25.467 |
| Security Gateway Identity | Identity of the Security GW the HNB connects to. The type of identity may be either “name” or IP address. | 25.467 |
| Access Mode | Indicates the type of access mode the HNB is to operate; it can be either “Open Access,” “Closed Access,” or “Hybrid Access.” | 22.011 |
| ACL list | Defines the ACL, which consists of one or more IMSI of the UE that the access to the HNB is allowed. It is applicable to either “Closed Access” and “Hybrid Access.” | 22.011 |
| CSG Identity | Defines the CSG ID to which the HNB is a member of. It consists of one or more than one CSG ID. | 22.011 |
| CSG specific info | Defines the CSG specific split information including Primary Scrambling Code (PSC) and UARFCN dedicated to the CSG. This information is intended in the BCCH SIB information for the UEs under the HNB. | 25.331, 10.3.2.8,10.3.2.9, 10.2.48.8.14a |
| HNB ID | Home NodeB Identifier broadcasted when it operates as CSG cell and is to be displayed on the UE. | 22.011 |
| Home Zone Name | String with a length 48 Bytes encoded in UTF-8 that defines the “home zone name” that is to be displayed to the UE that camps on to the HNB. | 36.331 section 6.3.1 |
| URA list | List of one or more URA that the HNB belongs to. | 25.331, sec.10.3.2.6 |
| RNC ID | RNC-ID of the HNB. | 25.413, sec.9.2.1.39, 9.2.1.39a |
| RANAP Timers | RANAP message related timers, typically named as “T-xxx” | 25.413, sec.9.5 |
| Cell ID | Cell Identity, bit string (28). | 25.331, sec.10.3.2.2 |
| HSDPA related parameter | HSDPA specific parameters including: 1) HSDPA is used in the cell or not, 2) number of codes at the defined spreading factor within the code tree, 3) number of HS-SCCHs used in the HNB. | 32.642, sec.6.3.9 |
| Measurement related parameter | Measurement related parameters including: 1) measurement quantity, 2) minimum Rx signal level, 3) offset to the minimum Rx level, 4) cell re-selection hysteresis, 5) timers for re-selection, 6) threshold for re-selection. Note that separate sets of parameters exist for intra-frequency, inter-frequency, and inter-RAT frequency measurements. | 25.331, sec.10.3.2.3, 10.3.2.4 |
| HCS related parameter | Measurement related parameters including: 1) timers, threshold, hysteresis for HCS to determine when the UE enters or exists the high-mobility state, 2) quality threshold level for HCS, 3) flag to indicate HCS is used or not in the HNB, 4) HCS priority value for the HNB if the HCS is used. | 25.331, sec.10.3.2.3, 10.3.7.12, 10.3.7.47, 10.3.7.54a |
| RACH related parameter for UE | RACH related parameters for UE when accessing the PRACH, including: 1) Maximum Tx power level a UE may use, 2) initial Tx power, 3) ramp-up step, 4) maximum preamble retransmission value, 5) maximum RACH preamble cycles, 6) back-off period lowerbound/upperbound. | 25.331, sec.10.3.2.3, 10.3.6.11, 10.3.6.54, 10.3.6.67 25.304, sec.5.2.3 |
| DRX related parameter | DRX related parameters including: DRX cycle length coefficient. | 25.331, sec.10.3.3.6, 10.3.3.49 |
| Cell Barred related parameter | Cell barred related parameters including: 1) Indicates whether the HNB is barred from service or not, 2) indicates whether the intra-frequency cell re-selection is allowed or not, 3) time period which the UE is barred from accessing the HNB, 4) list of Access Class barred, 5) indicates whether the HNB is reserved for operator use or not. | 25.331, sec.10.3.2 |
| Intra-frequency measurement related parameter | Intra-frequency measurement related parameters including: 1) measurement quantity, 2) weighting factor, 3) hysteresis, 4) time-to-trigger, 5) threshold. | 25.331, sec.10.3.7.38, 10.3.7.39, 10.3.7.64 |
| Inter-frequency measurement related parameter | Inter-frequency measurement related parameters including: 1) filter coefficient, 2) measurement quantity, 3) weighting factor, 4) hysteresis, 5) time-to-trigger, 6) threshold. | 25.331, sec.10.3.7.9, 10.3.7.18, 10.3.7.19, 10.3.7.64, |
| Inter-RAT measurement related parameter | Inter-RAT measurement related parameters including: 1) filter coefficient, 2) BSIC verification required, 3) weighting factor, 4) hysteresis, 5) time-to-trigger, 6) threshold. | 25.331, sec.10.3.7.9, 10.3.7.29, 10.3.7.30, 10.3.7.64, |
| RRC timers and constants | Timers and constants used in RRC. Timers and constants have names designated as “T3xx,” and “N3xx,” respectively. | 25.331, sec.13.1, 10.3.3.43, 10.3.3.44 |
| UE Internal Measurement related parameters | UE-internal measurement related parameters including: 1) filter coefficient, 2) UE Tx power threshold and time-to-trigger. | 25.331, 10.3.7.9, 10.3.7.79, 10.3.7.80, |
| Radio physical layer related parameters | Radio physical layer related parameters including: 1) UARFCN, 2) Primary Scrambling Code, 3) Max HNB Tx power, 4) Max UL Tx power, 5) P-CPICH power, 6) PDPDCH power, 7) P-SCH power, 8) S-SCH power, 9) PICH power, 10) PCH power, 11) FACH power, 12) BCH power, 13) AICH power, 14) Noise Rise Limit 15) Coverage edge Ec/Io and pathloss. | 25.433, sec. 9.2.2.33, 9.2.2.34, 9.1.3.1, 9.1.6.1, 9.1.24, 9.2.1.21, 9.2.1.40, 9.2.1.49A, 9.2.1.65, 9.2.2.D. 25.331, sec. 10.3.6.3, 10.3.6.39, 10.3.6.50. 25.104. 32.642, sec.6.3.9, 6.3.11, 9.3.11 |
| Intra-frequency neighbour list | Intra-frequency neighbour list parameters including: 1) PLMN-ID, 2) RNC ID, 3) LAC, 4) RAC, 5) URA, 6) P-CPICH Scrambling Code, 7) P-CPICH Tx power 8) Collocation indicator, 9) Maximum Tx Power, 10) Maximum DPCH Tx Power. | 25.413, 9.2.1.39, 9.2.1.39a, 9.2.3.6, 9.2.3.7 23.003, sec.4.1, 4.2 24.008 25.331, sec.10.3.2.6 25.433, sec.9.2.2.33 32.642, sec.6.3.11 |
| Inter-frequency neighbour list | Inter-frequency neighbour list parameters including: 1) PLMN-ID, 2) RNC ID, 3) LAC, 4) RAC, 5) URA, 6) UARFCN, 7) P-CPICH Scrambling Code, 8) P-CPICH Tx power. | 25.413, sec.9.2.3.7, 9.2.1.39, 9.2.1.39a, 9.2.3.6, 9.2.3.7 23.003, sec.4.1, 4.2 24.008, 25.331, sec.10.3.2.6 25.433, sec.9.2.2.33 32.642, sec.6.3.11 |
| Inter-RAT neighbour list | Inter-RAT neighbour list (GSM only) parameters including: 1) PLMN-ID, 2) LAC, 3) BSIC, 4) Band Indicator, BCCH ARFCN. | 25.413, sec.9.2.3.33, 23.003 sec.4.1,51.011, sec10.3.42, 24.008 |
| State management related parameters | State management related parameters such as: 1) enable/disable (operational state), 2) lock/unlock (administrative state) | 32.671 |

End of modifications