3GPP TSG-SA5 (Telecom Management)	S5-110124

SA5#75, 24 - 28 Jan 2011; Sorrento, Italy	revision of S5-11abcd

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.425 | CR | 0042 |  rev | - |  Current version: | 9.5.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add missing S1 reset to E-RAB release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM8 |  |  |  |  |  |  | Date:  |  | 28/01/2011 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | S1 reset is missing in E-RAB release measurements (in R10, this error does not exist, i.e. S1 reset is already there) |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add missing S1 reset |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Wrong implementation of E-RAB release measurements |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 4.2.2	E-RAB release

#### 4.2.2.1	Number of E-RABs requested to release initiated by eNodeB per QCI

- This measurement provides the number of E-RABs requested to release initiated by eNodeB. The measurement is split into subcounters per E-RAB QoS level (QCI).

- CC

- On transmission by the eNodeB of an E-RAB RELEASE INDICATION or a UE CONTEXT RELEASE REQUEST or a RESET message to MME, each corresponding E-RAB requested to release is added to the relevant measurement per QCI, the possible QCIs are included in TS 36.413 [9]. The sum of all supported per QCI measurements shall equal the total number of E-RABs requested to release initiated by eNodeB. In case only a subset of per QCI measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form ERAB.RelEnbNbr.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

- 4.2.2.2	Number of E-RABs requested to release initiated by eNodeB per cause

- This measurement provides the number of E-RABs requested to release initiated by eNodeB. The measurement is split into subcounters per cause.

- CC

- On transmission by the eNodeB of an E-RAB RELEASE INDICATION or a UE CONTEXT RELEASE REQUEST or a RESET message to MME, each corresponding E-RAB requested to release is added to the relevant measurement per cause.  Possible causes are included in TS 36.413 [9].

- Each measurement is an integer value. The number of measurements is equal to the number of supported causes.

- The measurement names have the form ERAB.RelEnbNbr.cause
where cause identifies the reason for the E-RABs release request initiated by eNodeB.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

#### 4.2.2.3	Number of E-RABs attempted to release

- This measurement provides the number of E-RABs attempted to release. The measurement is split into subcounters per E-RAB QoS level (QCI).

- CC

- On receipt by the eNodeB of an E-RAB RELEASE COMMAND or UE CONTEXT RELEASE COMMAND or a UE CONTEXT RELEASE or a RESET message from MME, or transmission by the eNodeB of a RESET message to MME, each corresponding E-RAB to release is added to the relevant measurement per QCI, the possible QCIs are included in TS 36.413 [9]. The sum of all supported per QCI measurements shall equal the total number of E-RABs attempted to release. In case only a subset of per QCI measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form ERAB.RelAttNbr.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

#### 4.2.2.4	Number of E-RABs successfully released

- This measurement provides the number of E-RABs successfully released. The measurement is split into subcounters per E-RAB QoS level (QCI).

- CC

- On transmission by the eNodeB of an E-RAB RELEASE RESPONSE or UE CONTEXT RELEASE COMPLETE message, or the E-RAB released successfully by source eNB after receiving UE CONTEXT RELEASE, or a RESET ACKNOWLEDGE message to MME, or receipt of a RESET ACKNOWLEDGE message from MME, each corresponding E-RAB successfully released is added to the relevant measurement per QCI, the possible QCIs are included in TS 36.413 [9]. The sum of all supported per QCI measurements shall equal the total number of E-RABs successfully released. In case only a subset of per QCI measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form ERAB.RelSuccNbr.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

#### 4.2.2.5	Number of E-RABs failed to release

- This measurement provides the number of E-RABs failed to release. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the eNodeB of an E-RAB RELEASE RESPONSE message, each E-RAB failed to release is added to the relevant measurement per cause, the possible causes are included in TS 36.413 [9]. The sum of all supported per cause measurements shall equal the total number of E-RABs failed to release. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form ERAB.RelFailNbr.Cause
where Cause identifies the cause resulting in the E-RAB release failure.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

#### 4.2.2.6	Number of released active E-RABs

- This measurement provides the number of released E-RABs that were active at the time of release (i.e.when there was user data in the queue in any of the directions). The measurement is split into subcounters per E-RAB QoS level (QCI).

- CC

- On transmission by the eNodeB of an E-RAB RELEASE RESPONSE or UE CONTEXT RELEASE COMPLETE message with the exception of those succeeding the message UE CONTEXT RELEASE COMMAND with “Cause” indicating that the release was performed due to a successful Mobility activity, or a RESET ACKNOWLEDGE to MME, or receipt of a RESET ACKNOWLEDGE message from MME, if any of the UL or DL are considered active (according to the definition used for "Number of active UEs in TS 36.314 when there is still data in the DL or UL buffer, each corresponding E-RAB to release is added to the relevant measurement per QCI. The possible QCIs are described in TS 36.413 [9]. The sum of all supported per QCI measurements shall equal the total number of E-RABs attempted to release with data in the DL or UL buffer. In case only a subset of per QCI measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of QCIs plus a possible sum value identified by the .sum suffix.

- The measurement name has the form ERAB.RelActNbr.QCI
where QCI identifies the E-RAB level quality of service class.

- EUtranCellFDD
EUtranCellTDD

- Valid for packet switched traffic

- EPS

- This measurement is to support the Retainability KPI "E-RAB Retainability" defined in [13]

End of modifications