3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240368

10th – 18th July 2024	(revision of 0277_Rev2 was 0045_Rev1)

Source:	Apple

Title:	Pseudo-CR on Solution evaluation nested API invocation

Spec:	3GPP TR 23.700-22 V0.3.0

Agenda item:	8.7

Document for:	Approval

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

This contribution proposes evaluation for the user consent for nested API invocation solution.

2. Reason for Change

Solution#2 on user consent for nested API invocation was proposed to address KI#1 on managing resource owner consent. Specifcally on how consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function. This pCR provides an evaluation for the solution and addresses the outstanding editor’s notes.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 V0.3.0.

* * * First Change * * * *

## 6.3	Solution #2: User consent for nested API invocation

### 6.3.1	Solution description

#### 6.3.1.1	General

This solution maps to KI#1 on managing resource owner consent, specifically on how consent of the resource owner can be managed through communication between the resource owner (through the resource owner function) and authorization function in the CAPIF core function. This solution proposes to extend the existing text in clause 8.32 of 3GPP TS 23.222 [2] on reducing authorization information inquiry in a nested API invocation, specifically the procedure described in clause 8.32.3.

The existing procedure focuses on ensuring the (primary) API invoker and secondary API invoker (API exposure function 1 (AEF-1)) have the necessary authorization to invoke the service APIs of their respective API exposure functions, AEF-1 & AEF-2 respectively. Such authorization may include a specific scope to provide course-grained access to resources exposed by the service API, for example limiting access to only certain attributes.

In this solution it is proposed that the secondary API invoker (AEF-1) performs an additional check to ensure suitable user consent has been provided before making service API invocation requests towards a secondary API exposing function (AEF-2). This enables the AEF-1 to check before inadvertently exposing user sensitive information provided in the request from the (primary) API invoker for which appropriate user consent has not been obtained.

The solution is described in relation to nested API invocation, but it is considered to have applicability for the general case of a single API invoker performing service invocation towards an API exposure function.

#### 6.3.1.2	Enhancement to clause 8.32.3 of 3GPP TS 23.222

With this solution the procedure to reducing authorization information inquiry in a nested API invocation in clause 8.32.3 is enhanced with the following:

New pre-condition added:

Optionally, AEF-1 is configured with policy for user consent checking before making nested API service invocation requests.

New descriptive text is added after the existing text of Step 3 (which is “Based on the service API invocation request, API exposing function 1 decides to invoke another service API exposed by API exposing function 2”):

Before making the request, if the AEF is configured with policy regarding user consent checking, the AEF will check that the service API invocation request contents is not in breach of those requirements.

An example for the user consent check could involve checking associated attributes in the request itself (e.g., a signed token relating to user consent). Another example may require the AEF to further interact with the resource owner function to gain the necessary permissions to expose the attributes provided in the request from the API invoker (possibly via the CCF / authorization function), which could require enhancement to the procedures related to step 4 (which is “API exposing function 1, acting as an API invoker, obtains from the CCF the authorization information to access the service API exposed by API exposing function 2”).

If the AEF doesn’t obtain the necessary user consent permissions, the (primary) API invoker’s service invocation request will be rejected with an appropriate failure result.

The additions are highlighted (in bold) in Figure 6.3.1.2-1.

Figure 6.3.1.2-1: Enhancements to Figure 8.32.3-1 of 3GPP TS 23.222.

Editor's note: The mechanism to ascertain appropriate user consent at the AEF is in scope of SA3.

### 6.3.2	Architecture Impacts

None.

### 6.3.3	Corresponding APIs

Editor's note:	Whether new or enhanced APIs are required in support of this solution is in scope of SA3.

### 6.3.4	Solution evaluation

This solution addresses KI#1 on managing resource owner consent by proposing that the AEF supports the ability use a configured policy to enforce user consent checking during service API invocation. Then when such policy is applied to the AEF, it will check that received service API invocation requests are not in breach of the requirements described by the policy. API invoker requests will be rejected if the necessary user consent (according to the user consent checking policy) cannot be ascertained. The need for ROF to be involved for checking user consent during service API invocation requires further investigation.

Editor's note: Prior to inclusion of this solution in the normative phase, the feasibility of providing a security mechanism to enforce user consent checking at an AEF, which is also aligned with CAPIF RNAA, needs coordination with SA3.

* * * End of changes * * * *