3GPP TSG-SA WG6 Meeting #13	S6-160963

Sophia Antipolis, France, 10th – 14th October 2016	(revision of S6-160xxx)

Source:	Motorola Solutions

Title:	pCR TR 23.781 – Solution for system configuration and authorization

Specification:	TR 23.781 v0.2.0

Agenda Item:	7.2  FS_MCSMI

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

Abstract: Provides a solution to system configuration and authorization for key issues 1.1 and 2.1

This pCR proposes adding a solution that adds configuration parameters to Service Configuration Data (on-network) to provide authorization for interconnection and migration calls with partner systems and to provide the relevant addressing information for those partner systems.

***************************************** Change 1 **************************************************

## 6.x	Solution #: System authorization and configuration for interconnection and migration

### 6.x.x	Description

This solution applies to key issues 1.1, MC system authorization to support interconnection calls, and 2.1, MC authorization for migration.

Service configuration (on-network) can contain lists of partner MC systems with which interconnection is permitted, and lists of partner MC systems with which migration is permitted.

Editor's note: it is FFS which of TS 23.280, TS 23.281, TS 23.283 and TS 23.379 will contain the relevant service configuration information.

Each entry of a partner MC system will contain a list of configuration information related to that partner MC system.  This information will include:

-	Application identity of that MC system, such as a URI

-	Addressing information for relevant entities in that MC system, such as IP addresses or URIs and identification of PDN within which the MC system can be reached.  The addressing information will be listed for at least the MC service servers and configuration servers which may be involved in interconnection and migration calls.

-	For interconnection:

-	Whether outgoing MCPTT private calls are permitted to users in that partner MC system.

-	Whether incoming MCPTT private calls are permitted from users in that partner MC system.

-	Whether MCPTT users in this primary MC system are permitted to affiliate and make MCPTT group calls to MCPTT groups defined in that partner MC system.

- 	Whether MCPTT groups defined in this primary MC system will permit affiliation and MCPTT group calls to be made from users in that partner MC system.

- 	Whether outgoing MCVideo calls can be made to users in that partner MC system.

-	Whether incoming MCVideo calls are permitted from users in that partner MC system.

-	Whether MCVideo users in this primary MC system are permitted to affiliate to and make MCVideo calls to MCVideo groups defined in that partner MC system.

- 	Whether MCVideo groups defined in this primary MC system will permit affiliation and MCVideo group calls to be made from users in that partner MC system.

- 	Whether outgoing MCData transactions can be sent to users in that partner MC system.

-	Whether incoming MCData transactions are permitted from users in that partner MC system.

-	For migration:

-	Whether users from that partner MC system are permitted to migrate to this primary MC system

-	Whether users from that partner MC system are permitted to make MCPTT private calls within this primary MC system.

-	Whether users from that partner MC system are permitted to affiliate to MCPTT groups and make MCPTT group calls within this primary MC system.

### 6.x.x	Impacts on existing nodes and functionality

Service configuration data will need to be extended to incorporate the information listed in the previous subclause.

### 6.x.x	Solution Evaluation

Editor's Note: Use this section for evaluation at solution level.

***************************************** End change 1 ************************************************