3GPP TSG-SA WG6 Meeting #28	S6-190261

Kochi, India, 21st – 25th January 2019	(revision of S6-190085/ S6-190175)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0045 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API sharing for CCF interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF |  |  |  |  |  |  | Date: |  | 2019-01-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At SA6#26 meeting, the solutions on publishing API from one CAPIF core function to another CAPIF core function in CAPIF interconnection scanerio is agreed. However, how to determine which service API to be published to another CAPIF core function is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) add an information element in the API publish request to indicate whether this API can be shared or can not be to another CAPIF 2) update the Service API publish for CAPIF interconnection procedure 3) editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Service API(s) which are required to be published in CAPIF interconnection scenario is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.1, 8.3.3, 8.25.2.1, 8.25.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***************************** 1st change ****************************

#### 8.3.2.1	Service API publish request

Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function.

Table 8.3.2.1-1: Service API publish request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M | The service API information includes the service API name, service API type, communication type, description, interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format. |
| Shareable indication information | O (NOTE) | Indicates whether the service API or the service API category can be published to other CCFs, and if sharing, shared within API provider domain, or beyond API provider domain, or both. |
| NOTE: If the shareable indication information is not present, the service API is not allowed to be shared. |  |  |

***************************** 2nd change ****************************

### 8.3.3	Procedure

Figure 8.3.3-1 illustrates the procedure for publishing the service APIs. The service API publish mechanism is supported by the CAPIF core function.

Pre-conditions:

1.	The CAPIF core function is configured with the authorization details of the API publishing function.

2.	API invokers may have subscribed with the CAPIF core function to obtain new service API information.

Figure 8.3.3-1: Publish service APIs

1.	The API publishing function sends a service API publish request to the CAPIF core function, with the details of the service API. If the service API is to be shared to other CAPIF core functions, the shareable indication information is included.

2.	Upon receiving the service API publish request, the CAPIF core function checks whether the API publishing function is authorized to publish service APIs. If the check is successful, the service API information provided by the API publishing function is stored at the CAPIF core function (API registry).

3.	The CAPIF core function provides a service API publish response to the API publishing function indicating success or failure result and triggers notifications to subscribed API invokers as described in subclause 8.8.4.

***************************** 3rd change ****************************

#### 8.25.2.1	Interconnection API publish request

Table 8.25.2.1-1 describes the information flow interconnection API publish request from CAPIF core function to CAPIF core function.

Table 8.25.2.1-1: Interconnection API publish request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CAPIF core function which publishes APIs, may include identity, authentication and authorization information |
| Service API information | O (NOTE 1) | The service API information includes the service API name, service API type, communication type, description, interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format. |
| Service API category | O (NOTE 1) | The category of the service APIs to be published, (e.g.,V2X, IoT) |
| Shareable indication information | O (NOTE 2) | Indicates whether the service API or the service API category can be published to other CCFs, and if sharing, shared within API provider domain, or beyond API provider domain, or both. |
| NOTE 1: At least one of the Service API information and Service API category shall be present. NOTE 2: If the shareable indication information is not present, the service API is not allowed to be shared. |  |  |

***************************** 4th change ****************************

#### 8.25.3.1	Service API publish for CAPIF interconnection

This subclause describes the procedure for service API publish for CAPIF interoperation.

Pre-condition:

1.	CCF-A and CCF-B connect to each other, and either belong to the same CAPIF domain or different CAPIF domains.

2.	When CCF-A and CCF-B belongs to different CAPIF domains, the two CAPIF domains have business agreement for service API sharing.

Figure 8.25.3.1-1: Interconnection API publish

1.	CCF-A gets the service APIs to be shared to CCF-B from API publish function in its CAPIF domain.

2.	Based on the shareable indication information for the service API or the service API category information, the CCF-A determines to publish the service API or the service API category information to the CCF-B. The CCF-A sends the interconnection API publish request to CCF-B with the details of service APIs or the category information of the service APIs, the identity of CCF-A, and shareable indication information if required. The API topology hiding may be applied.

3.	CCF-B store the API related information provided by the CCF-A.

4.	CCF-B provides an interconnection API publish response to the CCF-A indicating success or failure result and triggers notifications to subscribed API invokers as described in subclause 8.8.4.