3GPP TSG-SA WG6 Meeting #28	S6-190277

Kochi, India, 21st – 25th January 2019	(revision of S6-190086/0176)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0046 | rev | 2 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API invocation request routing with topology hiding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF |  |  |  |  |  |  | Date: |  | 2019-01-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In a single API provider domain, multiple AEFs may provide the same API for its serving area. For example, some kinds of individual UE specific APIs such as positioning API may be provided by multiple AEFs that each AEF is responsible for a certain area or a set of UEs. When the API is discovered with topology hiding by the API invoker, the API invoker will contact the AEF exposed to the API invoker as the entry point for API invocation. In this context, how this AEF acts as the entry point to route the API invocation is an issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) new requirements about dynamic routing is added. 2) new subclause for dynamic API invocation routing is added. 3) Step 2 is removed from subclause 8.13.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service API provided by multiple AEFs will not be correctly addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x (new), 4.x.1 (new), 4.x.2 (new), 8.13.3, 8.x (new), 8.x.1 (new), 8.x.2.1 (new), 8.x.2.2 (new), 8.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***************************** First change ****************************

## 4.x	Dynamic routing of service API invocation

### 4.x.1	Introduction

This subclause specifies the dynamic routing of service API invocation related requirements.

### 4.x.2	Requirements

[AR-4.x.2-a] The CAPIF shall provide a mechanism to support the dynamic routing of service API invocation.

***************************** Next change ****************************

### 8.13.3	Procedure

Figure 8.13.3-1 illustrates the procedure for CAPIF topology hiding.

Pre-conditions:

1.	The API invoker has performed the service discovery and received the details of the service API which includes the information about the service communication entry point of the AEF-1 in the CAPIF.

2.	The API invoker is authenticated and authorized to use the service API.

3.	The AEF-1 in the CAPIF is configured with a policy for topology hiding including the entry point address of the service API (provided via AEF-2).

Figure 8.13.3-1: Procedure for CAPIF topology hiding

1.	The API invoker performs service API invocation according to the interface of the service API by sending a service API invocation request towards the AEF-1 which exposes the service API towards the API invoker, and acts as topology hiding entity.

NOTE:	Steps 2 and 3 are not necessary when the AEF-1 is capable to serve the service API invocation request.

2.	The AEF-1 further resolves the actual destination service API address information according to the topology hiding policy and forwards the incoming service API invocation request to the service API of the related AEF-2.

3.	The AEF-1 receives a response request for service API invocation from service API provided by AEF-2.

4.	The AEF-1 resolves the destination API invoker address and also modifies the source address information of the AEF-2 within the response request as per topology hiding policy and forwards the response request to the API invoker.

***************************** Next change ****************************

## 8.x	Dynamically routing service API invocation

### 8.x.1	General

The procedure in this subclause corresponds to the architectural requirements for dynamic routing of service API invocation. The CAPIF enables dynamically routing the service API invocation request based on the detailed information of the invocation.

### 8.x.2	Information flows

#### 8.x.2.1	Obtain routing information request

Table 8.x.2.1-1 describes the information flow dynamic routing information request from the API exposing function to the CAPIF core function.

Table 8.x.2.1-1: Obtain routing information request

| Information element | Status | Description |
| --- | --- | --- |
| Service API identification information | M | The identification information of the service API for which invocation is requested. The service API identification is part of the specific service API invocation request. |
| AEF identity information | M | Identity information of the entity requesting the routing information |

#### 8.x.2.2	Obtain routing information response

Table 8.x.2.2-1 describes the information flow dynamic routing information response from the CAPIF core function to the API exposing function.

Table 8.x.2.2-1: Obtain routing information response

| Information element | Status | Description |
| --- | --- | --- |
| Service API identification information | M | The identification information of the service API for which invocation is requested. |
| API invocation routing rule | M | Indicates the routing rule for service API invocation |

### 8.x.3	Procedure

Figure 8.x.3-1 illustrates the procedure for dynamically routing the service API invocation from the AEF acting as service communication entry point to the destination AEF for handling service API.

Pre-conditions:

1.	The API invoker has performed the service discovery and received the details of the service API which includes the information about the service communication entry point of the AEF-1 in the CAPIF.

2.	The API invoker is authenticated and authorized to use the service API.

3.	The AEF-1 is the AEF acting as service communication entry point for the service API, and AEF-2 is the destination AEF for handling the service API.

Figure 8.x.3-1: Procedure for dynamic routing of service API invocation

1.	The API invoker performs service API invocation according to the interface of the service API by sending a service API invocation request towards the AEF-1 which exposes the service API towards the API invoker, and acts as topology hiding entity.

2.	If the routing rule information is not available, the AEF-1 sends obtain routing information request to the CAPIF core function.

3.	The CAPIF core function creates routing rule information for the service API and sends obtain routing information response with the routing rule information.

NOTE:	Steps 2 and 3 can be performed before step 1and after receiving the API topology hiding notify as described in subclause 8.24.3.

4.	The AEF-1 further resolves the actual destination of the service API address information (AEF-2) according to the routing rule information and the invocation parameters in service API invocation request.

5.	The AEF-1 forwards the incoming service API invocation request to AEF-2.

6.	The AEF-2 returns the service API invocation response to AEF-1.

7.	The AEF-1 sends the service API invocation response to the API invoker.