3GPP TSG-SA WG6 Meeting #28	S6-190241

Kochi, India, 21st – 25th January 2019	(revision of S6-190040)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 113 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to MCVideo Group join information flows and procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus DS SLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCVideo |  |  |  |  |  |  | Date: |  | 2019-01-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Information flow names are misaligned between 23.379 (“Group join request”) and 23.281 (“Group call join request”). Information element “Implicit transmit media request” is missing from the Group join request information flow. Note that this correction is needed also for stage3 TS 24.281. Preconditions of 7.1.2.3.1.2.2 Chat group call setup need some improvement. Current precondition 1 is not a real precondition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Throughout the TS, “Group call join request” is changed to “Group join request” and “Group call join response” is changed to “Group join response”. “Implicit transmit media request” IE is added to Group join request information flow (7.1.2.2.10). Preconditions of 7.1.2.3.1.2.2 are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Same procedure (group join) has different information flow names in MCPTT and MCVideo specifications. The “Implicit transmit media request” functionality shall be missing from the chat group join procedure  different behaviour for MCPTT and MCVideo chat groups. (Note: same error in stage3 TSs). Ambiguous precondition in 7.1.2.3.1.2.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2.10, 7.1.2.2.11, 7.1.2.3.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * 1st change * * * *

##### 7.1.2.2.10	Group join request (MCVideo client – MCVideo server)

Table 7.1.2.2.10-1 describes the information flow group join request from the MCVideo client to the MCVideo server.

Table 7.1.2.2.10-1: Group join request (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member joining the group communications for the group |
| MCVideo group ID | M | The MCVideo group ID of the group to which the group communication is requested |
| SDP offer | M | Media parameters of MCVideo client |
| Implicit transmit media request | O | An indication that the user is also requesting the permission to transmit video |

* * * 2nd change * * * *

##### 7.1.2.2.11	Group join response (MCVideo server – MCVideo client)

Table 7.1.2.2.11-1 describes the information flow group join response from the MCVideo server to the MCVideo client.

Table 7.1.2.2.11-1: Group join response (MCVideo server – MCVideo client)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member joining the group communications for the group |
| MCVideo group ID | M | The MCVideo group ID of the group to which the group communication is requested |
| SDP answer | M | Media parameters selected |

* * * 3rd change * * * *

7.1.2.3.1.2.2	Chat group call setup

MCVideo client 1, client 2, and client 3 are served by the home MCVideo service provider in figure 7.1.2.3.1.2.2-1.

Pre-conditions:

1.	MCVideo user 2 and MCVideo user 3 have previously joined (affiliated) to the group call. MCVideo client 1, client 2, and client 3 are registered and all users (MCVideo user 1, user 2, and user 3) have been authenticated and authorized to use the MCVideo service.

2.	No call is currently in progress for the group.

Figure 7.1.2.3.1.2.2-1: MCVideo chat group call

1.	MCVideo user 1 indicates to join the group communication for the group. This may include a transmit media request.

1a.	MCVideo client 1 sends a group join request with the MCVideo group ID of the desired group. It contains the MCVideo user's MCVideo ID and the MCVideo client media parameters. If there is a request for media transmission, then the group join request contains an indication of an implicit transmit media request.

1b.	The MCVideo server receives the group join request. MCVideo server generates an implicit affiliation (if the MCVideo user is not already affiliated to the group) and verifies that MCVideo user 1 is authorized to affiliate to the group by following the affiliation procedure for MCVideo.

1c.	The MCVideo server replies with a group join response indicating the acceptance of the group join request and also returns the MCVideo server selected media parameters for the group call in the group join response.

2.	If MCVideo user 1 requests to transmit media by sending transmit media request to MCVideo server, the MCVideo server establishes the media plane (if not already established) for the call.

3.	Transmission control will continue to be used by the transmission control participants associated with MCVideo client 1, MCVideo client 2 and MCVideo client 3 for the duration of the call.

* * * End of changes * * * *