3GPP TSG-SA WG6 Meeting #28	S6-190254

Kochi, India, 21st – 25th January 2019	(revision of S6-190046, S6-190162)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0127 | rev | 2 | Current version: | 16.1.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Generic SDS procedure with MCData message store |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData2 |  |  |  |  |  |  | Date: |  | 2019-01--3 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With MCData message store added to R16 architecture it is needed to show how it impacting the MCData procedures in general. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new generic SDS procedure that shows how the MCData message store is involved in a MCData SDS service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no guidance for stage 3 development on how MCData message store involved in a MCData procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.13.4 (new), 7.13.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 7.13.4 	Generic outgoing SDS procedure with MCData message store

#### 7.13.4.1	General

When a MCData user is supported with MCData message store all his outgoing communications shall be stored in his account in the MCData message store when he has requested. This generic SDS procedure applies to all prcedures in subclause 7.4.2 when the MCData user requests to store the MCData communication.

#### 7.13.4.2 	Procedure

The procedure in figure 7.13.4.2-1 describes the generic SDS service where MCData message store is supported.

Pre-condictions:

- MCData user has an account created with MCData message store.

- MCData user has requested to store his MCData communication.

Figure 7.13.4.2-1 Generic outgoing SDS procedure with MCData message store

- MCData client initiates an MCData SDS service request; this service request can be a private or group communication.

- MCData server stores the communication as an object to the MCData user account in the MCData message store.

- MCData server checks and authorizes the service request and continue the service request toward the targeted recipient(s) as described in subclause 7.4.2.

### 7.13.5 	Generic incoming SDS procedure with MCData message store

#### 7.13.5.1	General

When a MCData user is supported with MCData message store all his incoming communications shall be stored in his account in the MCData message store when he has requested. This generic SDS procedure applies to all prcedures in subclause 7.4.2 when the MCData user requests to store the MCData communication.

#### 7.13.5.2 	Procedure

The procedure in figure 7.13.5.2-1 describes the generic SDS service where MCData message store is supported.

Pre-condictions:

- MCData user has an account created with MCData message store.

- MCData user has requested to store his MCData communication.

Figure 7.13.5.2-1 Generic incoming SDS procedure with MCData message store

- The MCData server receives an incoming MCData SDS service request for the MCData user. This service request can be a response to an earlier service request sent by the MCData user or a new service request coming from any sender.

- MCData server stores the communication as an object to the MCData user account in the MCData message store.

- MCData server delivers the service request to MCData user as described in subclause 7.4.2.