3GPP TSG-SA WG6 Meeting #28	S6-190293

Kochi, India, 21st – 25th January 2019	(revision of S6-190047, S6-190163, S6-190255)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0128 | rev | 3 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing data for a user entering an ongoing MCData group conversation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData2 |  |  |  |  |  |  | Date: |  | 2019-01-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To support the following new requirement for R16: [R-6.1.1.2-011] If configured by the MCX Administrator the MCData Service shall be able to provide a MCX User the data that had previously been distributed in an ongoing MCData group conversation before this MCX User had affiliated or joined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new configuration parameter and procedure to support this requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 will have no guidance to develop the correct procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.3.5 (new), A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.4.3.5	Providing data for a user entering an ongoing MCData group conversation

##### 7.4.3.5.1	General

The MCData service shall support mechanisms that allow a MCData user be presented with the whole content of a group conversation in a group that he is a member of. This includes the content (messages) exchanged before the MCData user joins the group conversation.

##### 7.4.3.5.2	Procedure

Figure 7.4.3.5.2-1 describes procedures for a MCData user joins late to a group conversation.

Pre-conditions:

- The MCData group is provisioned for lossless communication.

- All members of the MCData group have an account created in the MCData message store.

- MCData client 1, MCData client 2 and MCData client 3 are members of the same MCData group,

- MCData client 1 and 2 are served by MCData server 1 and have registered and affiliated to the MCData group.

- MCData client 3 is served by MCData server 2 and has not affiliated to the MCData group yet.

NOTE 1: 	The interactions of MCData client 1 and MCData client 2 to MCData message store are not shown in the figure.

Figure 7.4.3.5.2-1: Providing data for a user entering an ongoing MCData group conversation

- A group conversation is initiated according to procedures in subclause 7.4.2.6, and all members of the group are invited into the communication whether affiliated or not. As MCData user 3 is not affiliated at this time, MCData server 2 accepts the invitation to the group conversation on behalf of MCData user 3.

Editor's Note:	It is FFS if a new procedure for lossless conversation needs to be added to subclause 7.4.2 showing key difference from regular group conversation such as no check for affiliation to start and MCData server answers on behalf of a non-affiliated member.

2.	The media plane is established for the group conversation. MCData server 2 is in the media plane to receive the conversation on behalf of MCData user 3.

3.	MCData server 2 stores the received conversation to MCData user 3 account in the MCData message store.

NOTE: If the received conversation requests delivery notification the MCData server 2 will send message delivered to the message sender. If the received conversation requests read notification the MCData client 3 will send message read to the message sender once it has presented the message to the user.

4.	MCData user 3 is online and using MCData client 3 to affiliate to the MCData group.

5.	MCData client 3, through the message store client, synchronizes with the MCData user 3 account in the MCData message store.

6.	MCData server 2 invites MCData client 3 to the MCData group conversation.

7.	MCData user 3 joins the MCData group conversation.

* * * Second Change * * * *

# A.4	MCData related Group configuration data

The general aspects of group configuration are specified in 3GPP TS 23.280 [5].

Parameters specified in table A.4-1 are child parameters of the "MCData configuration" parameter specified in table A.4-1 in 3GPP TS 23.280 [5]. Parameters specified in table A.4-2 are child parameters of the "MCData configuration" parameter specified in table A.4-2 in 3GPP TS 23.280 [5]. Parameters specified in table A.4-3 are child parameters of the "MCData configuration" parameter specified in table A.4-3 in 3GPP TS 23.280 [5].

Editor's note: Further contents of this configuration data are FFS.

Table A.4-1: Group configuration data (on and off network)

| Reference | Parameter description | MCData UE | MCData Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-5.12-001] of 3GPP TS 22.280 [2] | >> Media confidentiality and integrity protection (see NOTE) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | >> Transmission control confidentiality and integrity protection (see NOTE) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | >> Group media protection security material (see NOTE) | Y | N | Y |
| Subclause 5 | >> MCData sub-services and features enabled for the group |  |  |  |
|  | >>> Short data service enabled | Y | Y | Y |
|  | >>> File distribution enabled | Y | Y | Y |
|  | >>> Conversation management enabled | Y | Y | Y |
|  | >>> Transmission control enabled | Y | Y | Y |
|  | >>> Reception control enabled | Y | Y | Y |
|  | >>> Enhanced status enabled | Y | Y | Y |
|  | >> Transmission and reception control |  |  |  |
| [R-6.2.2.1-001] of 3GPP TS 22.282 [3] | >>> Whether MCData user is permitted to transmit data in the group | Y | Y | Y |
| [R-6.2.3-006] of 3GPP TS 22.282 [3] | >>> Maximum amount of data that the MCData user can transmit in a single request during group communication | Y | Y | Y |
| [R-6.2.3-006] of 3GPP TS 22.282 [3] | >>> Maximum amount of time that the MCData user can transmit in a single request during group communication | Y | Y | Y |
|  | >> Enhanced status |  |  |  |
| [R-6.1.3.2-002] of 3GPP TS 22.282 [3] | >>> List of operational status values | Y | N | Y |
| NOTE: Security mechanisms are specified in 3GPP TS 33.180 [13]. |  |  |  |  |

Table A.4-2: Group configuration data (on network)

| Reference | Parameter description | MCData UE | MCData Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-6.4.5-001], [R-6.4.5-003] of 3GPP TS 22.280 [2] | >> Authorisation of a user to request a list of affiliated members of a group | Y | Y | Y |
| [R-5.1.7-002], [R-6.2.2-001], [R-6.6.2.2-006], [R-6.8.7.2-003] of 3GPP TS 22.280 [2] | >> Priority of the group | N | Y | Y |
| [R-6.1.1.2-011] of 3GPP TS 22.282 [2] | >> Lossless communication | N | Y | Y |
| Subclause 6.2.2 of 3GPP TS 22.282 [3] | >> Transmission and reception control |  |  |  |
|  | >>> Maximum data size for SDS | Y | Y | Y |
|  | >>> Maximum data size for FD | Y | Y | Y |
|  | >>> Maximum data size for auto-receive | N | Y | Y |
| 3GPP TS 23.283 [18] | >> Indication whether use of LMR E2EE is permitted on the MCData group | Y | N | Y |
| 3GPP TS 23.283 [18] | >> LMR specific identity for MCData group (see NOTE) | Y | N | Y |
| 3GPP TS 23.283 [18] | >> Group to key binding (see NOTE) | Y | N | Y |
| NOTE: This is an LMR specific parameter with no meaning within MC services. |  |  |  |  |

Table A.4-3: Group configuration data (off network)

| Reference | Parameter description | MCData UE | MCData Server | Group management server |
| --- | --- | --- | --- | --- |
| Subclause 10.10 of 3GPP TS 23.280 [5] | >> Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | >>> MCData group call signalling | Y | N | Y |
|  | >>> MCData group call media | Y | N | Y |