3GPP TSG-SA WG6 Meeting #28	S6-190170

Kochi, India, 21st – 25th January 2019	(revision of S6-190131)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0028 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IWF alignment for current talker location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Harris Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCCI |  |  |  |  |  |  | Date: |  | 2019-01-10 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Align with TS 23.379 changes |  |  |  |  |  |  |  |  |
| Summary of change: |  | All changes are aimed at improving alignment with TS 23.379 by adding optional location to the following procedures: First change: Location of current talker (MCPTT server to IWF). This change also corrects a typographical error. Second change: Location of current talker (IWF to MCPTT server). This change also corrects a typographical error. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with TS 23.379, resulting in needless divergence between interworking and interconnect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.11.2, 10.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

### 10.11.2	Location of current talker (MCPTT server to IWF)

Figure 10.11.2-1 shows the high-level procedure to for MCPTT service to provide the location information about the current talking user to all the receiving MCPTT users and the IWF.

Pre-conditions:

1.	There is on-going group call involving MCPTT client 1 and MCPTT client 2 and the IWF.

2.	MCPTT client 1 is the current talking user.

3.	MCPTT server has obtained the location information of MCPTT client 1.

Figure 10.11.2-1: Providing location information of the current talker

1.	MCPTT client 1 gets the floor to transmit voice media.

2.	The MCPTT server checks the privacy policy (authorisation to provide location information to other MCPTT users on a call when talking, as defined in 3GPP TS 23.379 [7] Annex A.3) of the current talking MCPTT user to decide if the location information of MCPTT client 1 can be provided to other MCPTT users on the call.

3.	If the privacy policy permits, the MCPTT server provides the location information of MCPTT client 1 to MCPTT client 2 and the IWF. The procedures for this are described in 3GPP TS 23.280 [5] subclause 10.9.3.6. Optionally, the location information may be provided in the floor taken message sent to MCPTT client 2 and the IWF according to 3GPP TS 23.379 [7] subclause 10.9.1.3.1, if the privacy policy permits.

* * * Second change * * *

### 10.11.3	Location of current talker (IWF to MCPTT server)

Figure 10.11.3-1 shows the high-level procedure to for the IWF to provide the location information about the current LMR talking user to all the receiving MCPTT users.

Pre-conditions:

1.	There is on-going group call involving MCPTT client 1 and MCPTT client 2 and the IWF.

2.	An LMR user is the current talking user through the IWF.

NOTE:	How the MCPTT server acquires the location of the LMR user is outside the scope of the present document.

Figure 10.11.3-1: Providing location information of the current talker

1.	The IWF gets the floor to transmit voice media.

2.	The MCPTT server checks the privacy policy (authorisation to provide location information to other MCPTT users on a call when talking, as defined in 3GPP TS 23.379 [7] Annex A.3) of the current talking IWF user to decide if the location information of the user on the IWF can be provided to other MCPTT users on the call.

3.	The MCPTT server provides the location information of the IWF user to MCPTT client 1 and MCPTT client 2. The procedures for this are described in 3GPP TS 23.280 [5] subclause 10.9.3.6.  Optionally, the location information may be provided in the floor taken message sent to MCPTT client 2 and the IWF according to 3GPP TS 23.379 [7] subclause 10.9.1.3.1.

* * * End of changes * * *