3GPP TSG-SA WG6 Meeting #36BIS-e	S6-200506

E-meeting, 31st Mar 2020 – 8th Apr 2020	(revision of S6-20xxxx)

Source:	Huawei, Hisilicon

Title:	User plane management events API

Spec:	3GPP TS 23.558 v0.1.2

Agenda item:	7.4

Document for:	Approval

Contact:	Niranth (namogh@huawei.com)

1. Introduction

The contribution provides a proposal for user plane path management event API

2. Reason for Change

The EES provides the capabilities via API(s) towards the EAS to obtain the user plane path management events from the 3GPP core network. Two API models are supported viz., request/response and subscribe/notify.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v0.1.2.

* * * First Change * * * *

### 8.6.x	User plane path management events

#### 8.6.x.1	General

The Edge Enabler Server exposes user plane path management event notifications of an UE to an Edge Application Server (e.g. in order to trigger the application context relocation). User plane path management event notifications API exposed by the Edge Enabler Server may rely on the NEF northbound API for monitoring event of user plane path management event.

#### 8.6.x.2	User plane path management event API

Figure 8.6.x.2-1 illustrates the subscribe/unsubscribe operation between the Edge Application Server and the Edge Enabler Server for user plane path management event notifications.

Figure 8.6.x.2-1: User plane path management event API: Subscribe/Unsubscribe Operation

1.	The Edge Application Server requests user plane path management event subscribe operation for tracking the UE's user plane path change continuously. The Edge Application Server shall include UE Identifier. For cancelling the subscription, the Edge Application Server uses the unsubscribe operation.

2.	If the request is subscribe, the Edge Enabler Server checks if the Edge Application Server is authorized for this operation. If authorized, Edge Enabler Server checks if there exists a subscription with the 3GPP core network for the user plane path management event notifications corresponding to the UE information obtained in step 1 as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], which may be triggered by other Edge Application Server for the same UE.

a.	if a subscription with 3GPP core network does not exist, then the Edge Enabler Server subscribes with the 3GPP core network for the user plane path management event notifications of the UE as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3];

b.	if a subscription with 3GPP core network exists, then the Edge Enabler Server uses the locally cached user plane path management event notification information of the UE to respond to the Edge Application Server.

The Edge Enabler Server stores the subscription related to the Edge Application Server.

If the request is unsubscribe, the Edge Enabler Server checks if the Edge Application Server is authorized for this operation and if authorized removes the subscription related to the Edge Application Server.

Editor's note: It is FFS how EES determines the Core Network type related to a UE.

3.	If Edge Application Server is authorized, the Edge Enabler Server responds with a subscription information for the subscribe request. If Edge Application Server is not authorized, the Edge Enabler Server provides a rejection response with cause information. If the request is for unsubscribe, the Edge Enabler server provides an ACK response for the unsubscribe request.

Figure 8.6.x.2-2 illustrates the notify operation between the Edge Enabler Server and the Edge Application Server for continuous User plane path management event notifications.

Figure 8.6.x.2-2: User plane path management event API: Notify Operation

1.	The Edge Enabler Server detects the user plane path management event of the UE (e.g. receiving User plane path management event notification for the UE from the 3GPP core network). The Edge Enabler Server may cache the detected User plane path management event notification locally with timestamp as the latest information of the UE. The Edge Enabler Server determines to notify the user plane path management event notification information (e.g., DNAI) to the Edge Application Servers which has subscribed for the user plane path management event.

2.	The Edge Enabler Server sends user plane path management event notify operation to the Edge Application Server. The Edge Enabler Server includes the user plane path management event notification information of the UE and optionally the timestamp of the location. The timestamp can be included to indicate the age of the location information. The Edge Enabler Server may only provide part of information included in the user plane path management event notification from 3GPP network (e.g. DNAI).

#### 8.6.x.3	Information flows

##### 8.6.x.3.1	User plane path management event API subscribe request

Table 8.6.x.3.1-1 describes the information elements for a User Plane Path Management Event API Subscribe Request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.x.3.1-1: User plane path management event API subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the UE (i.e. GPSI or identity token) |
| Event ID(s) | M | Event ID(s) as specified in 3GPP TS 23.501 [2] |
| Event Reporting Information | M | Event Reporting Information as specified in 3GPP TS 23.502 [3] |
| Notification Target Address | M | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Event Filter | O | Event filter as specified in 3GPP TS 23.501 [2] |

##### 8.6.x.3.2	User plane path management event API subscribe response

Table 8.6.x.3.2-1 describes the information elements for a User Plane Path Management Event API Subscribe Response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.x.3.2-1: User plane path management event API subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates whether the subscription is successful or failure |
| Cause information (see NOTE 1) | O | Indicates the cause information for subscription failiure |
| Subscription ID (see NOTE 2) | O | Subscription identifier corresponding to the subscription stored in the EES for the request |
| Event report (see NOTE 2) | O | Event report as specified in clause 4.15.1 of 3GPP TS 23.502 [3] |
| Expiry time (see NOTE 2, NOTE 3) | O | The expiry time of the subscription |
| NOTE 1: The Cause information IE is included when the Result indicates failure. NOTE 2: The Subscription ID IE, Event report IE and Expiry time IE are included when the Result indicates success. NOTE 3: The Expiry time IE if not included indicates that the subscription never expires. |  |  |

##### 8.6.x.3.3	User plane path management event API unsubscribe request

Table 8.6.x.3.3-1 describes the information elements for a User Plane Path Management Event API Unsubscribe Request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.x.3.3-1: User plane path management event API unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |

##### 8.6.x.3.4	User plane path management event API unsubscribe response

Table 8.6.x.3.4-1 describes the information elements for a User Plane Path Management Event API Unsubscribe Response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.x.3.4-1: User plane path management event API unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates whether the unsubscription is successful or failure |
| Cause information (see NOTE 1) | O | Indicates the cause information for subscription failiure |

##### 8.6.x.3.5	User plane path management event API notification request

Table 8.6.x.3.5-1 describes the information elements for a User Plane Path Management Event API Notification Request from the Edge Enabler Server to the Edge Application Server.

Table 8.6.x.3.5-1: User plane path management event API notification request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |
| Event report | O | Event report as specified in clause 4.15.1 of 3GPP TS 23.501 [2] |

#### 8.6.x.4	APIs

##### 8.6.x.4.1	General

Table 8.6.x.4.1-1 illustrates the API for user plane path management event.

Table 8.6.x.4.1-1: User plane path management API

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| EDGE3_User_plane_management_event API | Subscribe | Subscribe/Notify | Edge Application Server |
|  | Notify | Subscribe/Notify | Edge Application Server |
|  | Unsubscribe | Subscribe/Notify | Edge Application Server |

##### 8.6.x.4.2	EDGE3_User_plane_management_event_subscribe operation

Service operation name: EDGE3_User_plane_management_event_subscribe

Description: The consumer subscribes to receive an event, or if the event is already defined in Edge Enabler Server, then the subscription is updated.

Inputs: See clause 8.6.x.3.1.

Outputs: See clause 8.6.x.3.2.

See clause 8.6.x.2 for details of usage of this operation.

##### 8.6.x.4.3	EDGE3_User_plane_management_event_notify operation

Service operation name: EDGE3_User_plane_management_event_notify

Description: The consumer is notified of an event by the Edge Enabler Server.

Inputs: See clause 8.6.x.3.5.

Outputs: None.

See clause 8.6.x.2 for details of usage of this operation.

##### 8.6.x.4.4	EDGE3_User_plane_management_event_unsubscribe operation

Service operation name: EDGE3_User_plane_management_event_unsubscribe

Description: The consumer unsubscribes for the previously subscribed events.

Inputs: See clause 8.6.x.3.3.

Outputs: See clause 8.6.x.3.4.

See clause 8.6.x.2 for details of usage of this operation.