3GPP TSG-SA WG6 Meeting #37-e	S6-200761

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Title:	[Draft] LS on IP address to GPSI translation

Release:	Rel-17

Work Item:	EDGEAPP

Source:	3GPP TSG SA WG6

To:	3GPP TSG SA WG2

Cc:	3GPP TSG SA WG3

Contact Person:

#### Name:	Nishant Gupta

E-mail Address:	<nishant.gup@samsung.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

1. Overall Description:

In SA6 architecture for enabling Edge Applications, the Edge Enabler Server (EES) facilitates use of 3GPP core network capability exposure by the Edge Application Servers (EAS). Some EAS may not have the GPSI of the UE(s) for e.g. due to undesired GPSI configuration in static way. Such EAS can use EES's service to obtain the GPSI of a UE dynamically using available information about the UE, such as UE's IP address. The EAS can then use this GPSI to utilize core network capabilities.

For this purpose, the Edge Enabler Server acting as an AF (trusted or untrusted) requires to translate UE's IP address provided by the Edge Application Server, to the GPSI of the UE.

This IP address provided by the Edge Application Server could be the UE's private (i.e. non-NAT’d) IP address or the public (i.e. NAT’d) IP address, in case NAT systems are employed by the 3GPP network.

Question to SA2:

Does the core network provide an API to translate UE's IP address (private or public) to its GPSI, and if not, would it be feasible to provide such functionality in Rel-17 in order to address SA6 requirement for UE IP address translation?

Note to SA3:

Since the GPSI may be used by an Edge Application Server to obtain sensitive information about a UE, such as UE's location, SA6 kindly asks SA3 to consider the role of user and user's consent in providing the GPSI or any sensitive information about the UE to an Edge Application Server. Also, there may be a need to ensure that the EAS can not share the GPSI with unauthorized entities.

2. Actions:

To SA WG2 group.

ACTION: 	SA6 requests SA2 to kindly consider the above request and provide feedback accordingly.

3. Date of Next SA6 Meetings:

SA6#38 20th – 30th July 2020	e-meeting