3GPP TSG-SA WG6 Meeting #37-e	S6-200677

Online, 14th - 26th May 2020	(revision of S6-20xxxx)

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on EES to EES interface

Spec:	3GPP TS 23.558 v0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Eddy Hall

edhall@qti.qualcomm.com

1. Introduction

In order to enable application context transfer between two EAS supported by different Edge Enabler Servers, it has been agreed in SA6 that there must be data transfer between the supporting EESes. As this data transfer is being standardised in TS 23.558, we need a formal interface definition and for this to be added to the application architecture.

Summary of changes:

1. Addition of EDGE-9 to Application Architecture and removal of Editor’s Note (clause 6.2);

2. Addition of application context transfer to the functional entity definition of the EES (clause 6.3.2);

3. Addition of EDGE-9 reference point description (new clause 6.4.10); and

4. Addition of EDGE-9 reference point cardinality (new clause 6.5.3.6).

2. Reason for Change

Procedures have been intruduced in TS 23.558 which require EES to EES communication in order to enable application context transfer. More procedures are likely for the same feature. This paper formalises the EES to EES interface in the Application Architecture.

3. Proposal

It is proposed to agree the following changes to 3GPP TS23.558.

* * * First Change * * * *

## 6.2	Architecture

The Figure 6.2-1 shows the architecture for enabling edge applications.

Figure 6.2-1: Architecture for enabling edge applications

The Edge Data Network is a local Data Network. Edge Application Server(s) and the Edge Enabler Server are contained within the EDN. The Edge Configuration Server provides configurations related to the EES, including details of the Edge Data Network hosting the EES. The UE contains Application Client(s) and the Edge Enabler Client. The Edge Application Server(s), the Edge Enabler Server and the Edge Configuration Server may interact with the 3GPP Core Network.

Editor's Note:	Need to split Edge Enabler Client into Edge Enabler Client and Edge Configuration Client is for further study.

Editor's Note:	Representation of the Edge Hosting Environment is for further study.

* * * End First Change * * * *

* * * Second Change * * * *

### 6.3.2	Edge Enabler Server

Edge Enabler Server (EES) provides supporting functions needed for Edge Application Servers and Edge Enabler Client.

Functionalities of Edge Enabler Server are:

a)	provisioning of configuration information to Edge Enabler Client, enabling exchange of application data traffic with the Edge Application Server;

b)	support the functionalities of API invoker and API exposing function as specified in 3GPP TS 23.222 [6]; and

c)	support the functionalities of application context transfer.

* * * End Second Change * * * *

* * * Third Change * * * *

### 6.4.10	EDGE-9

EDGE-9 reference point enables interactions between two Edge Enabler Servers. EDGE-9 reference point may be provided between EES within different EDN (Figure 6.4.10-1) and within the same EDN (Figure 6.4.10-2).

Editor’s Note: The use of EDGE-9 between EES belonging to different ECSPs is FFS.

Figure 6.4.10-1: Inter-EDN EDGE-9

Figure 6.4.10-2: Intra-EDN EDGE-9

EDGE-9 supports:

a)	discovery of target Edge Application Server information to support application context transfer

Editor's Note:	Need to extend this list with further functionalities which require EES to EES interface when agreed.

* * * End Third Change * * * *

* * * Fourth Change * * * *

#### 6.5.3.6	EDGE-9 (Between EES and EES)

The following cardinality rules apply for EDGE-9:

a)	One EES may communicate with one or more EES(s) concurrently.

* * * End Fourth Change * * * *