3GPP TSG-SA WG6 Meeting #37-e	S6-200690

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	Capabilities to support service continuity

Spec:	3GPP TS 23.558

Agenda item:	7.5

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

2. Reason for Change

This contribution provides capabilities of the Edge Enabler Client and the Edge Enabler Server for supporting service continuity at application layer.

A UE is aware of both service area information (e.g. EAS service area or location availability) and UE location so that the Edge Enabler Client can detect if application context relocation is required.

If an Edge Enabler Server acting as AF has a pre-configured Spatial Validity Condition for the AF request as described in clause 5.6.7 of TS 23.501, the Edge Enabler Server may consider this spatial condition as its service area and make the Edge Enabler Client be aware of this condition and be able to invoke application context relocation. In Table 5.6.7-1 of TS 23.501, the Spatial Validity Condition indicates that the AF request applies only to the traffic of UE(s) located in the specified location, represented by areas of validity.

3. Conclusions

* * * First Change * * * *

## 8.8	Preserving service continuity

Editor's note:	The description text in clause 8.8.2.1 and IEs for Security Credential and UE identifier in Table 8.8.3.1-1 should be corrected.

### 8.8.1	General

This clause describes features that support service continuity for Application Client in the UE to continue its service while replacing the serving Edge Application Server with a target Edge Application Server. When a UE moves to a new location, different Edge Application Servers may be more suitable for serving the Application Clients in the UE. Such transitions may not only result from a mobility event but also other non-mobility events.

The following clauses provide procedures for relocating application context from the source Edge Application Server to the target Edge Application Server for the service continuity. The Edge Enabler Client, source Edge Enabler Server, and source Edge Application Server can determine that the application context needs to be relocated and initiate the relocation procedure.

The application context relocation is based on either source Edge Enabler Server or target Edge Enabler Server according to the context relocation request direction, i.e. either source-to-target Edge Enabler Server or target-to-source Edge Enabler Server. The source Edge Enabler Server sends this context relocation request when receiving context relocation request from the Edge Enabler Client or the source Edge Application Server, or when deciding the necessity of the context relocation without receiving the requests from the Edge Enabler Client or the source Edge Application Server. On the other hand, the target Edge Enabler Server sends the context relocation request as a consequence of receiving context relocation request from the Edge Enabler Client.

This clause provides procedures for service continuity that are within the scope of the application architecture defined in clause 5 for enabling Edge Applications.

#### 8.8.1.X	Capabilities to support service continuity

Generally, a source Edge Application Server which is in communication with an Application Client is associated with an application context. To support service continuity, the application context from a source Edge Application Server is transferred to a target Edge Application Server. The capabilities for supporting service continuity provided at the Edge Enabler Layer may consider various application layer scenarios in which there may be involvement of Application Client and one or more Edge Application Server(s).

The Edge Application Server may utilize the following capabilities provided by the Edge Enabler Server for supporting service continuity at the application layer:

-	Subscribe for UP path management events notification, e.g., DNAI change.

-	Fetch the target Edge Application Server;

-	Application context relocation from a source Edge Application Server to a target Edge Application Server;

-	Notification for the result of Application Context Relocation to the Edge Enabler Client, e.g., information for target Edge Application Server and target Edge Enabler Server.

-	Issue the AF request to influence on traffic routing

Editor's note:	The capability of Edge Enabler Client for supporting service continuity at the application layer is FFS.

Each of the above capabilities to the Edge Application Server may be provided as individual API, or combined to be offered as a new service API with higher level of abstraction. Example, application context relocation service API which fulfils all the above capabilities to hide the implementation complexity from the application layer.

The Edge Configuration Server provides the following capabilities to the Edge Enabler Server for supporting service continuity at the application layer:

-	Fetch the target Edge Enabler Server.