3GPP TSG-SA WG6 Meeting #37-e	S6-200693

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	Service API Availability reporting API

Spec:	3GPP TS 23.558

Agenda item:	7.5

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

This pCR provides a method to make Edge Application Server be aware of the availability of the Service API(s).

2. Reason for Change

The followings are described in TS 23.501. “A service related with common north-bound API may become unavailable due to UE being served by a CN node not supporting the service. If the availability or expected level of support of a service API associated with a UE changes, for example due to a mobility between 5GC and EPC, the AF shall be made aware of the change.” It is thus required for EES to monitor the availability of a service API exposed by 3GPP network and make its EAS(s) be aware of the availablity of the Service API(s).

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558.

* * * First Change * * * *

### 8.6.3	User plane path management events

#### 8.6.3.1	General

The Edge Enabler Server exposes user plane path management event notifications of an UE to an Edge Application Server (e.g. in order to trigger the application context relocation). User plane path management event notifications API exposed by the Edge Enabler Server may rely on the NEF northbound API for monitoring event of user plane path management event.

The availability of the user plane path management event notifications may change due to UE mobility between 5GC and EPC. If an Edge Enabler Server re-exposes the northbound API for user plane path management notifications of 3GPP Core Network to Edge Application Server(s) for a UE, the Edge Enabler Server monitors the availability of the northbound API for UE(s) served by the Edge Application Server (e.g. by utilizing Nnef_APISupportCapability as in TS 23.502) and provides the availability information to the Edge Application Server. If CAPIF is supported, the EES determines if the user plane path management event API is available and able to be exposed to the Edge Application Server for a UE via the Availability of service APIs event notifications provided by the CAPIF core function as in TS 23.222.

* * * End of  Change * * * *