3GPP TSG-SA WG6 Meeting #37-e	S6-200695

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	Information elements in Service Provisioning request

Spec:	3GPP TS 23.558

Agenda item:	7.5

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

This pCR proposes changes on information elements for Application Client and UE in Service Provisioning Request.

2. Reason for Change

Information elements in Application Client Profile are not always available at an EEC and thus cannot be mandatorily included in the Service Provisioning request. Let us consider an initial Service Provisioning request. Application Client Profile is available at an EEC only if it is provided from the App Client. This requires that the App Client should be Edge-aware and capable of performing EDGE-5 related operations (embeded by the client developer accompanied with modification of original App code). Since the Service Provisioning operation should support both Edge-aware and Edge-unaware App Clients, the IE for App Client Profile(s) should be optional in the Service Provisioning Request.

If information for Application Client is necessary for Service Provisioning, Application Client ID can be included as it  may be available at an EEC for Edge-aware and Edge-unaware applications.

UE ID (e.g. GPSI) is required for interactions between the ECS and the 3GPP CN via EDGE-8. For example, the UE ID can be utilized to obtain UE location, which can be used for selecting EES(s).

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558.

* * * First Change * * * *

### 8.3.2	Procedure

Pre-conditions:

1.	The Edge Enabler Client has been pre-configured or discovered the address (e.g. URI) of the Edge Configuration Server.

2.	The Edge Enabler Client has been authorized to enable the communication with the Edge Configuration Server. The UE Identifier is either preconfigured or resulted from a successful authorization.

Editor's Note:	The authorization procedure required to enable communication with the Edge Configuration Server is FFS.

Figure 8.3.2-1: Service provisioning

1.	The Edge Enabler Client sends a provisioning request to the Edge Configuration Server. The request message may include the UE identifier such as GPSI or ID token and credentials received during EEC authorization procedure and Application Client Profile information.

2.	Upon receiving the request from the Edge Enabler Client, the Edge Configuration Server performs an authorization check to verify whether the Edge Enabler Client has privileges to perform the operation. The Edge Configuration Server may filter the Service Provisioning response to the UE by determining the Edge Enabler Servers matching Application Client ID or Application Client Profiles (or Application Client Profile IDs) if provided in step 1. The Edge Configuration Server may use API exposed by SCEF/NEF or LCS (Location Service) as specified in 3GPP TS 23.502 [3], 3GPP TS 23.271 [7], 3GPP TS 36.305 [8], 3GPP TS 23.273 [9] and 3GPP TS 38.305 [10] to obtain UE location and filter the Service Provisioning response. The Edge Configuration Server also determines other information that needs to be provisioned, e.g. identification of the Edge Data Network, service area information, Edge Enabler Server endpoints.

Editor's Note: It is FFS which additional information provided from 3GPP network is utilized for service provisioning.

3.	The Edge Configuration Server responds to the Edge Enabler Client by sending Edge services configuration information, e.g. identification of the Edge Data Network, service area information (for LADN), and the information for establishing a connection to the Edge Enabler Server (e.g. URI, IP address).

If the Edge Data Network configuration information includes an LADN DNN as an identifier for the Edge Data Network, the Edge Enabler Client considers the LADN as the Edge Data Network. Therefore, the service area of Edge Data Network is the LADN Service Area which can be discovered using the UE Registration Procedure.

#### 8.3.3.1	Provisioning request

Table 8.3.3.1-1 describes the information elements for a Provisioning Request from the Edge Enabler Client to the provisioning server (i.e. Edge Configuration Server).

Table 8.3.3.1-1: Provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Application Client ID | O | The identifier of Application Clients as described in clause 7.2.5. |
| Application Client Profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity Filter | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |

Editor's Note: It is FFS whether the EEC ID is mandatory.

* * * End of Change * * * *