3GPP TSG-SA WG6 Meeting #37-e	S6-200697

e-meeting, 14th – 26th May 2020	(revision of S6-20xxxx)

Source:	Samsung

Title:	EAS service continuity capability

Spec:	3GPP TS 23.558

Agenda item:	7.5

Document for:	Approval

Contact:	Hyesung Kim <hs1207.kim@samsung.com>

1. Introduction

<Introduction part >

2. Reason for Change

This pCR introduces EAS service continuity capability to make the Edge Enabler Server be aware of the EAS capability to initiate application context relocation. This enables to prevent duplicated intiation/preparation of the EES and its registered EAS for the same relocation trigger condition(s). For example, if providers of the EES and the EAS both have SLA for the use of UP path management event notification, the 3GPP network resource for monitoring/exposing/signalling the DNAI change of the same UE can be reserved in redundantly for the EES and EAS.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558.

* * * First Change * * * *

### 8.2.4	Edge Application Server Profile

Table 8.2.4-1: Edge Application Server Profile

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to Application Clients so that application clients can establish contact with the EAS. |
| Application Client ID(s) | O | Identifies the Application Client(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the EAS Provider |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Service Area | O | The geographical service area that the EAS serves |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| Service continuity support | O | Indicates if the EAS supports service continuity or not. |
| EAS service continuity capability | O | Indicates the EAS capability (e.g. availability of UP path management event notifications) to initiate application context procedure along with trigger condition. This IE is present if the EAS supports the service continuity. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heart beat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Editor's Note: The inclusion of EAS Instance ID (including optionality) is dependent on how the ID is defined and assigned, which is FFS.

Editor's Note: It is FFS whether the Application Client ID(s) should be included and how Application Client ID can be mapped with EAS(s) at an EES.

Editor's Note: The current text in the TS has the definition of a ‘service area’ IE associated with certain entities (such as the EAS Service Area in the table above). The following are FFS: (1) Which entities should have a ‘service area’ and (2) What is the relationship between the service areas of the different entities if any.

* * * Next Change * * * *

<Proposed change in revision marks>

### 8.4.3	Edge Application Server Registration

#### 8.4.3.1	General

The Edge Application Server Registration procedure allows an Edge Application Server to provide information to an Edge Enabler Server in order to request use of its edge capabilities.

#### 8.4.3.2	Procedure

Pre-conditions:

1.	The Edge Application Server has been configured with an Edge Application Server Identity;

2.	The Edge Application Server has been configured with the address (e.g. URI) of the Edge Enabler Server; and

3.	Both the Edge Application Server and Edge Enabler Server have the necessary credentials to enable communications.

Figure 8.4.3.2-1: Edge Application Server Registration procedure

1.	The Edge Application Server determines that registration to the Edge Enabler Server is needed (e.g. the Edge Application Server is instantiated and started up). The Edge Application Server may also determine that its existing registration needs to be updated (e.g. because the Edge Application Server's status or availability schedule has changed).

2.	The Edge Application Server sends a Registration Request to the Edge Enabler Server. The request indicates if the request is for a new registration or an update to an existing registration.

3.	The Edge Enabler Server performs an authorization check to verify whether the Edge Application Server has the proper privileges to register.

Editor’s note: It’s FFS whether and how the EAS instances are registered and authorized at the Edge Enabler Server.

4.	Upon successful authorization, the Edge Enabler Server stores the EAS Profile for later use (e.g. for serving Edge Application Server discovery requests received from Edge Enabler Clients, etc.) and replies to the Edge Application Server with a Registration Response. The Edge Enabler Server may provide an expiration time to indicate to the Edge Application Server when the registration will automatically expire. The Edge Application Server shall send a Registration Request message with a Registration Type Indicator of type Update prior to the expiration to maintain the current registration. The Edge Enabler Server will treat the registration expiration as an implicit de-registration.

If the EAS Service Continuity Capability is provided in step 1, the Edge Enabler Server includes Application Context Relocation trigger information that indicates relocation trigger condition(s) or capabilities of the Edge Enabler Server in the registration response.

Editor’s note: It’s FFS which additional information will be included in the Application Context Relocation trigger information.

#### 8.4.3.3	Information flows

##### 8.4.3.3.1	Edge Application Server Registration request

Table 8.4.3.3.1-1 describes information elements in the Edge Application Server Registration Request from the Edge Application Server to the Edge Enabler Server.

Table 8.4.3.3.1-1: Edge Application Server Registration request

| Information element | Status | Description |
| --- | --- | --- |
| EAS Profile | M | EAS Profile as described in Table 8.2.4-1 |
| Registration type indication | M | Indication for new or updated registration |
| Security credentials | O | Security credentials resulting from a successful authorization for the edge computing service. |
| Proposed expiration time | O | Proposed expiration time for the registration |

Editor's Note:	The use of EAS Instance ID (including optionality) is dependent on how the ID is defined and assigned, which is FFS.

Editor's Note: It is FFS whether the EAS profile ID should be included and whether the EAS instance ID is equal to the EAS profile ID.

##### 8.4.3.3.2	Edge Application Server Registration response

Table 8.4.3.3.2-1 describes information elements in the Edge Application Server Registration Response from the Edge Enabler Server to the Edge Application Server.

Table 8.4.3.3.2-1: Edge Application Server Registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the registration operation |
| Expiration time (NOTE) | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| Application context relocation trigger information | O | Indicates trigger conditions for the Edge Enabler Server to initiate application context relocation procedure |
| NOTE: If the Expiration time IE is not included, it indicates that the registration never expires. |  |  |