3GPP TSG-SA WG6 Meeting #37-e	S6-200706 rev1

e-meeting, 14th – 26th May 2020	(revision of S6-200706)

Source:	Apple

Title:	Pseudo-CR on Maximum Response Time and QoS for the EAS registration

Spec:	3GPP TS 23.558, v 0.2.0

Agenda item:	7.5

Document for:	Approval

Contact:	Robert Zaus / <rzaus> <at> <apple> <dot> <com>

1. Introduction

The present contribution addresses an editor's note in subclause 8.2.5, regarding the definition of the Maximum Response Time advertised by the Edge Application Server when it registers with the Edge Enabler Server.

For more details see also the discussion paper in S6-200705.

2. Reason for Change

Currently there is an editor's note in subclause 8.2.5, regarding the definition of the maximum response time:

Editor's note: It is FFS how the maximum response time is determined (e.g. based on measurements, estimations or a combination of both) and which round-trip time (e.g. UE to EAS, or RAN node to EAS only) is included in the maximum response time.

As discussed in S6-200705, it is proposed to leave the exact method of determination up to EAS implementation. Furthermore, it is proposed to align the definition with the definition of the response time in the AC Service KPIs information element. I.e. the maximum response time is defined to include the complete RTT between UE and EAS.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558, v 0.2.0.

* * * First Change * * * *

### 8.2.5	Edge Application Server Service KPIs

Edge Application Server Service KPIs provide information about service characteristics provided by the Edge Application Server

Table 8.2.5-1: Edge Application Server Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the Application Client supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the Application Client's service requests (NOTE). |
| Availability | O | Advertised percentage of time the server is available for the Application Client's use. |
| Available Compute | O | The maximum compute resource available for the Application Client. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the Application Client. |
| Available Memory | O | The maximum memory resource available for the Application Client. |
| Available Storage | O | The maximum storage resource available for the Application Client. |
| NOTE: The maximum response time includes the round-trip time of the request and response packet, the processing time at the server and the time required by the server to consume 3GPP Core Network capabilities, if any. |  |  |